3GPP TSG-RAN WG3 Meeting #105	R3-193699

# Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 179 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MRO(Inter-system HO too late andHO too early) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently,there is no support of transfering RLF report via NG interface which is a popular scenario of inter-system handover too late and inter-system handover too early.Also,direct RIM information transfer between NG-RAN node and eNB is not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)Introduce RLF report transfer between NG-RAN nodes via AMF to support inter-system handover too late and inter-system handover too early. 2)Introduce Mobility Information IE in Source NG RAN node TO Target NG RAN node transparent container. 3)Introduce NG-RAN Node Direct Information Transfer message and AMF Direct Information Transfer message . Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the SON function. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-system handover too late and inter-system handover too early could not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.300 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----First change----

8.X	NG-RAN Node Direct Information Transfer

8.X.1	General

The purpose of the NG-RAN Node Direct Information Transfer procedure is to transfer RAN information from the NG-RAN Node to the AMF in unacknowledged mode. The AMF does not interpret the transferred RAN information.

This procedure uses non-UE associated signalling.

8.X.2	Successful Operation

8.X.2.1	NG-RAN Node Direct Information Transfer

Figure 8.X.2.1-1: NG-RAN Node Direct Information Transfer procedure. Successful operation.

The procedure is initiated with an NG-RAN NODE DIRECT INFORMATION TRANSFER message sent from the NG-RAN Node to the AMF.

The RIM Transfer IE within the Inter-system Information Transfer Type IE shall contain the RIM Routing Address IE that identifies the final RAN destination node where the RIM information needs to be transferred to by the core network.

8.X.3	Abnormal Conditions

Not applicable.

8.Y	AMF Direct Information Transfer

8.Y.1	General

The purpose of the AMF Direct Information Transfer procedure is to transfer RAN information from the AMF to the NG-RAN Node in unacknowledged mode.

This procedure uses non-UE associated signalling.

8.Y.2	Successful Operation

8.Y.2.1	AMF Direct Information Transfer

Figure 8.Y.2-1: AMF Direct Information Transfer procedure. Successful operation.

The procedure is initiated with a AMF DIRECT INFORMATION TRANSFER message sent from the AMF to the NG-RAN Node.

The Inter-system Information Transfer Type IE indicates the nature of the transferred information. When the transferred information is of RIM nature, the RIM Information IE within the RIM Transfer IE shall contain a BSSGP RIM PDU. The RIM Routing Address IE shall not be present since the NG-RAN node is the final destination node.

8.Y.3	Abnormal Conditions

Not applicable.

-----Next change----

9.2.X	RAN DIRECT INFORAMTION TRANSFER Messages

9.2.X.1 	NG-RAN NODE DIRECT INFORMATION TRANSFER

This message is sent by the NG-RAN node in order to transfer specific information.

Direction: NG-RAN Node  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Inter-system Information Transfer Type | M |  | 9.3.1.x |  | YES | reject |

9.2.X.2 	AMF DIRECT INFORMATION TRANSFER

This message is sent by the AMF in order to transfer specific information.

Direction: AMF  NG-RAN Node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Inter-system Information Transfer Type | M |  | 9.3.1.x |  | YES | reject |

-----Next change----

9.3.1.X	Inter-system Information Transfer Type

The Inter-system Information Type IE indicates the type of information that the NG-RAN node requests to transfer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system Information Transfer Type | M |  |  |  |
| >RIM |  |  |  |  |
| >>RIM Transfer | M |  | 9.3.3.X |  |

-----Next change----

9.3.3.X	RIM Transfer

This IE contains the RIM Information and additionally in uplink transfers the RIM routing address of the destination of this RIM information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Transfer |  |  |  |  |
| >RIM Information | M |  | 9.3.3.Y |  |
| >RIM Routing Address | O |  | 9.3.3.Z |  |

9.3.3.Y	RIM Information

This IE contains the RIM Information i.e., the BSSGP RIM PDU from the RIM application part contained in the NG-RAN node, or the BSSGP RIM PDU to be forwarded to the RIM application part in the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Information |  |  |  |  |
| >RIM Information | M |  | OCTET STRING | Contains the BSSGP RIM PDU as defined in TS 48.018 [18]. |

9.3.3.Z	RIM Routing Address

This IE identifies the destination node where the RIM Information needs to be routed by the CN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE RIM Routing Address | M |  |  |  |  |  |
| >E-UTRAN-Cell-ID |  |  |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 |  | - |  |
| >>ECGI | M |  | 9.3.1.9 |  | - |  |

-----Next change----

9.3.1.29	Source NG-RAN Node to Target NG-RAN Node Transparent Container

This IE is produced by the source NG-RAN node and is transmitted to the target NG-RAN node. For inter-system handovers to 5G, the IE is transmitted from the external handover source to the target NG-RAN node.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in TS 38.331 [18] if the target is a gNB. Includes the RRC HandoverPreparationInformation message as defined in TS 36.331 [21] if the target is an ng-eNB. |
| PDU Session Resource Information List |  | 0..1 |  | For intra-system handovers in NG-RAN. |
| >PDU Session Resource Information Item |  | 1..<maxnoofPDUSessions> |  |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  |
| >>QoS Flow Information List |  | 1 |  |  |
| >>>QoS Flow Information Item |  | 1..<maxnoofQoSFlows> |  |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.51 |  |
| >>>>DL Forwarding | O |  | 9.3.1.33 |  |
| >>DRBs to QoS Flows Mapping List | O |  | 9.3.1.34 |  |
| E-RAB Information List |  | 0..1 |  | For inter-system handovers to 5G. |
| >E-RAB Information Item |  | 1..<maxnoofE-RABs> |  |  |
| >>E-RAB ID | M |  | 9.3.2.3 |  |
| >>DL Forwarding | O |  | 9.3.1.33 |  |
| Target Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  |
| UE History Information | M |  | 9.3.1.95 |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the external handover source provides it in order to enable later analysis of the conditions that led to a wrong HO. |

-----Next change----

9.3.3.7	SON Information

This IE identifies the nature of the configuration information transferred, i.e., a request, a reply or a report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information | M |  |  |  |
| >SON Information Request |  |  |  |  |
| >>SON Information Request | M |  | ENUMERATED (Xn TNL Configuration Info, …) |  |
| >SON Information Reply |  |  |  |  |
| >>SON Information Reply | M |  | 9.3.3.8 |  |
| >SON Information Report |  |  |  |  |
| >>SON Information Report | M |  | 9.3.3.x |  |

-----Next change----

9.3.3.X	SON Information Report

This IE contains the configuration information to be transferred to the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >RLF Report Information |  |  |  |  |
| >>RLF Report Information | M |  | 9.3.3.Y |  |

9.3.3.Y	RLF Report Information

This IE contains the RLF report information to be transferred to the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RLF Report Container | M |  | OCTET STRING | rlf-Report contained in UEInformationResponse message as defined in TS 38.331 [16]. |