3GPP TSG-RAN WG3 Meeting #105	R3-194129

# Ljubljana, Slovenien, 26th – 30th August 2019

Agenda Item:	08.1

Source:	Ericsson

Title: 	Temporary Identifier usage at interworking

Document for:	Discussion and Approval

# 1	Discussion

In [1], three different understandings of the ID handling for Interworking with 5GS and EPS are described:

About the SA2 status on the CN identities used in 5GS and EPS, RAN3 has different understanding: Understanding 1: SA2: One understanding is that SA2 has agreed support of combined AMF/MME in release 15 (similar as e.g. former combined MME/SGSN). RAN3: The current NGAP and S1AP stage 3 do not support the above. Understanding 2: SA2: Another understanding is that SA2 has agreed means to avoid the need to coordinate CN identities used in 5GS and EPS but not agreed on support of combined AMF/MME nodes. RAN3: The current NGAP stage 3 is aligned with the above. Understanding 3: SA2: Another understanding is that SA2 has agreed means to avoid the need to coordinate CN identities used in 5GS and EPS and the combined AMF/MME nodes is not explicitly excluded. RAN3: The current NGAP and S1AP stage 3 do not support the above.

In [2] RAN3 asks SA2 to clarify if CN needs to indicate the GUMMEI Type/GUAMI Type to RAN in order to support the avoidance of the coordination of CN identities used in 5GS and EPS

In [3], SA2 replied to RAN3 that they have discussed the issue and the agreement is captured in [4]. SA2 has specified that:

4.11.0a.X Network Configuration To avoid the need for identifier coordination between MMEs, AMFs and SGSNs, the MME provides to the eNB its served GUMMEIs by separating the values between native MME GUMMEI values, values mapped from AMF, and values mapped from SGSN. 4.11.5.X Network Configuration To avoid the need for identifier coordination between AMFs and MMEs, the AMF provides to the NG-RAN its served GUAMIs by separating the values between native AMF values and the values mapped from MME.

With the above agreement, we have the below proposals:

Proposal 1:   RAN3 to agree to include the GUMMEI Type in the S1AP Setup Response

Proposal 2:   RAN3 to agree to include the GUAMI Type in the NGAP Setup Response

# 2	Proposal

Proposal 1:   RAN3 to agree to include the GUMMEI Type in the S1AP Setup Response

Proposal 2:   RAN3 to agree to include the GUAMI Type in the NGAP Setup Response

CRs are submitted in [5] and [6].

# 3	Reference

[1]	R3-192048	Summary of offline discussion, Ericsson

[2]	R3-193265	LS on the coordination of CN identities used in 5GS and EPS, RAN3

[3]	S2-1907913	Reply LS on "LS on the coordination of CN identities used in 5GS and EPS", SA2

[4]	S2-1907912	CN identifier coordination, Ericsson

[5]	R3-194127	Temporary Identifier usage at interworking, Ericsson

[6]	R3-194128	Temporary Identifier usage at interworking, Ericsson