3GPP TSG-RAN WG3 Meeting #105	R3-194190

# Ljubljana, Slovenien, 26th – 30th August 2019

Agenda Item:	31.3.5

Source:	Ericsson

Title: 	PDU Session NAS-PDU in PDU Session Resource Release Command

Document for:	Discussion and Approval

# 1	Discussion

At the last RAN3 meeting, we discussed that in the current NGAP message, PDU SESSION RESOURCE RELEASE COMMAND, it is not possible to send the NAS message per PDU session to NG-RAN node. As there is only one NAS-PDU which is at the message level.

We discussed if we should introduce the NAS-PDU insider the PDU Session Resource to Release List”. The conclusion is that we do not want to introduce the change.

We also discussed if to make a assumption, that: when releasing a PDU session “end-to-end” (i.e. NAS part and AS part), we can only release one at a time; deactivating multiple PDU sessions at the same time is possible, but in this case there are no NAS PDUs transferred”

In NGAP, the PDU Session Resources Release procedure is described as:

#### 8.2.2.1 General The purpose of the PDU Session Resource Release procedure is to enable the release of already established PDU session resources for a given UE. The procedure uses UE-associated signalling. #### 8.2.2.2 Successful Operation Figure 8.2.2.2-1: PDU session resource release: successful operation The AMF initiates the procedure by sending a PDU SESSION RESOURCE RELEASE COMMAND message. The PDU SESSION RESOURCE RELEASE COMMAND message shall contain the information required by the NG-RAN node to release at least one PDU session resource, and include each PDU session resource to release in the PDU Session Resource to Release List IE. If a NAS-PDU IE is contained in the PDU SESSION RESOURCE RELEASE COMMAND message, the NG-RAN node shall pass it to the UE.

There is no support for the the assumption that only one PDU session resource is released in the procedural text. In fact, the procedure is meant to release at least one PDU Session.

How the NAS-PDU is constructed is out of the scope of RAN3.

From RAN point of view, NG-RAN node would release the related PDU Session resources for all the PDU Sessions listed in the “to be released” information.  As NG-RAN node would not inspect the NAS-PDU, but only passes it to the UE, NG-RAN would not mind how many PDU sessions are released “end-to-end” between 5GC and UE.

We would like to conclude that the PDU session resource release procedure is kept as it is, without any modification or further assumption made.

# 2	Conclusion

Conclusion: RAN3 to keep the PDU Session Resource Release procedure as it is without further modification or assumption made.