3GPP TSG-RAN WG3 #105							                              R3-194405

Ljubljana, Slovenia, 26-30 August 2019

Agenda item:	10.2.2

Source:	CMCC

Title:	Mobility load balancing

Document for:	Discussion and decision

Introduction

According to the progress of last RAN plenary meeting, SON/MDT support for NR WI has been officially approved as the continuation of the RAN-Centric Data Collection and Utilization SI. Within the scope of SON/MDT WI, one of the most essential use cases is the mobility load balancing.

In this contribution, we provide our considerations on mobility load balancing during the normative phase.

Discussion

Load balancing has the task to handle uneven distribution of the traffic load over multiple cells [1]. The automating of such optimization can provide high quality user experience, simultaneous improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

According to the majority opinion from the email discussion [2], LTE MLB mechanism could be used as baseline. In 36.300 [1], the basic functions which support MLB are given as below,

Support for mobility load balancing consists of one or more of following functions:

-	Load reporting;

-	Load balancing action based on handovers;

-	Adapting handover and/or reselection configuration.

In our opinion, such classification on functions can be reused for MLB in NR. And similar descriptions on handover actions (bullet 2) and configuration adapting (bullet 3) given in 36.300 can also be reused in NR.

Proposal 1: the general description including the classification on functions, as well as the detailed descriptions on handover actions and configuration adapting given by TS36.300 can be reused for the normative description of MLB in NR.

Therefore, the main leftover issues are to define the specific load information and the corresponding Stage 2&3 procedures for the supporting functions.

For the load information, TR37.816 v16.0.0 has drawn a primary conclusion which is stated as follows,

Load sharing and load balancing functions over X2, Xn, F1 and E1 interfaces should be specified in Release 16 normative phase. At least the following load related information should be specified. Whether the reporting mechanism supports both periodic and event-triggered procedures is for further decision during normative phase.

-	Composite Available Capacity per cell (DL/UL), reported via X2, Xn and F1 interfaces

-	Cell Level Load (DL/UL/SUL) reported via X2, Xn, and F1 interfaces

-	TNL Load reported via X2, Xn, F1, and E1 interfaces

Besides the load information mentioned above, according to the real deployment of MLB in our network, the number of RRC connections for users is proven to be a useful parameter for cell load information exchange. The source node can use this parameter to decide whether to initiate the HANDOVER REQUEST to a potential target node.

For example, if there are two potential target nodes with equivalent cell level load, one with less number of RRC connections while the other with more number of RRC connections, with which the source node may prefer to trigger the handover procedure to the potential target node with less number of RRC connections because less number of connected UEs indicates more potential capability for the node, as long as the connected UEs are handed over to other nodes due to mobility.

Proposal 2: Introducing parameters to reflect the number of RRC connections.

The next issue is the procedures introduced to support MLB related functions.

In LTE, the procedures related to intra-system MLB includes the Resource Status Reporting procedures (used for load reporting on X2), Handover procedures (used for performing handover actions due to MLB on X2), and Mobility Settings Change procedure (used for parameter adapting on X2). In our opinion, all of the procedures mentioned above can be reused in NR.

More specifically, the intra-system MLB in NR can be further classified as intra-system inter-RAT scenario and intra-NR scenario. The intra-system inter-RAT case is related to X2/Xn specs while intra-NR case in related to F1/Xn specs. Note that current specs for X2, Xn and F1 have covered the Handover procedures with at most some text update (indicating the handover action can be performed due to MLB) to achieve MLB handover action function.

To achieve load reporting function, for Xn interface, the LTE X2-like Resource Status Reporting procedure can be reused; and for F1 interface, similar procedure can also be adopted, and the initiation of the resource status reporting is more appropriate to be triggered by gNB-CU; once the initiation is completed, the following resource status update is triggered by gNB-DU.

To achieve configuration adapting function, for Xn interface, the LTE X2-like Mobility Settings Change procedure can be reused; and for F1 interface, similar procedure can also be adopted, and the mobility settings change is more appropriate to be triggered by gNB-CU. It is FFS on how to interact in an efficient way between gNB-CU and gNB-DU, as well as details for related messages.

Proposal 3: For intra-NR MLB, the resource status reporting initiation is triggered by gNB-CU; while the resource status update is triggered by gNB-DU.

Proposal 4: For intra-NR MLB, the mobility settings change is initiated by gNB-CU

Proposal 5: It is FFS on how to interact in an efficient way between gNB-CU and gNB-DU, as well as details for related messages.

Proposal

The paper discussed the load information for MLB use case, and came to the following proposals:

Proposal 1: Proposal 1: the general description including the classification on functions, as well as the detailed descriptions on handover action s and configuration adapting given by TS36.300 can be reused for the normative description of MLB in NR.

Proposal 2: Introducing parameters to reflect the number of RRC connections.

Proposal 3: For intra-NR MLB, the resource status reporting initiation is triggered by gNB-CU; while the resource status update is triggered by gNB-DU.

Proposal 4: For intra-NR MLB, the mobility settings change is initiated by gNB-CU

Proposal 5: It is FFS on how to interact in an efficient way between gNB-CU and gNB-DU, as well as details for related messages.

References

- TR36.300, Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2(Release 15)

- R3-190565, Report of offline coordination of uses cases for RAN-centric DCU, CMCC (Rapporteur)

- TR37.816 v16.0.0

- TS38.423 v15.4.0

- TS38.473 v15.6.0