3GPP TSG-RAN3 Meeting #106	R3-197162

Reno, Nevada, US, 18-22 November 2019

Title: 	(TP for SON BL CR for TS 38.413): Intra-System and Inter-System Connection Failure

Source: 	Huawei

Agenda item:	10.2.1.1

Document for:	pCR

# 1. Introduction

MRO was discussed during RAN3#105b and a set of BL CRs were agreed. In this paper we propose some further refinement for MRO for Intra-System and Inter-System Connection Failure.

# 2. Discussion

The RLF report is being discussed in RAN2. It seems however clear that we will have on RLF report defined in TS 36.331 and one in TS 38.331. In LTE, we include two containers: RLF-Report-r9 and RLF-Report-r9, RAN2 need to provide further clarification on whether support them in NG-RAN and inter-system.

In NGRAN we have gNB and ng-eNB using different RRC specification. The RLF report is reported from the UE in the 3rd node (see figure below) and then forwarded to the last serving cell (2nd node) and in some cases forwarded to the cell handling the UE before the last successful HO (3rd node). These three nodes can be either ng-eNB or gNB.

Fig.1 the RLF report transferred over NG

Even if the details of the RLF report is still being discussed in RAN2, it is likely that we will see two different RLF reports from LTE and NR. This is also how our BL CRs are currently written (either RLF report from TS 36.331 or from TS 38.331). Including two different reports is not a problem, but it may be suboptimal if the receiving node cannot distinguish these two before it tries to decode the container.

Therefore we propose to introduce a choice structure for the RLF report containing either the RLF report-r16 in 36.331 or the RLF report in 38.331.

Define RLF as a choice structure with one entry for the RLF report in TS 36.331 and one for the one in TS 38.331.

For the HO report, we believe that we can also make the source and target cell conditional on RLF report.

Make source and target cell conditional in HO report

# 3. Conclusion

Based on the discussion in this paper, we propose to make a choice structure for RLF report transferred over NG interface. The detailed proposals are:

Proposal 1:	Define RLF as a choice structure with one entry for the RLF report in TS 36.331 and one for the one in TS 38.331.

Proposal 2:	Make source and target cell conditional in HO report

# 4. Reference

- R2-1914087, Reply LS on RACH Optimization and Mobility Robustness Optimization

# Annex – TP

Start of the change

#### 9.3.3.Y3	Failure Indication

This IE contains the failure indication to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RLF Report | O |  | 9.3.3.Z1 |  |

#### 9.3.3.Y4	HO Report

This IE contains the HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter System ping-pong, …) |  |
| Handover Cause | M |  | Cause 9.2.3.1 | Indicates handover cause employed for handover from source cell |
| Source cell CGI | C- ifUERLFReportAbsent |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the source cell for handover procedure |
| Target cell CGI | C- ifUERLFReportAbsent |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the target cell for handover procedure. If the Handover Report Type is set to “Inter System ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node cell |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCellandnoUERLFReport |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the cell where UE attempted re-establishment |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | E-UTRA CGI 9.3.1.9 | E-UTRA CGI of the E-UTRAN target cell for handover procedure |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from the source NG-RAN node |
| UE RLF Report | O |  | 9.3.3.Z1 | The UE RLF Report Container IE received in the FAILURE INDICATION message. |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportTypeHoToWrongCellandnoUERLFReport | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" and if the UE RLF Report IE is absent |
| ifHandoverReportTypeIntersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter System ping-pong" |
| ifUERLFReportAbsent | This IE shall be present if the UE RLF Report IE is absent |

#### 9.3.3.Z1	UE RLF Report

This IE contains the RLF Report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RLF report type | M |  |  |  |
| >gNB |  |  |  |  |
| >>UE RLF Report Container | M |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [18]. [FFS] |
| >ng-eNB |  |  |  |  |
| >> UE RLF Report Container | M |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message defined in TS 36.331 [21]. [FFS] |

End of the change