3GPP TSG-RAN WG3 Meeting #106	R3-197254

# Reno, Nevada, USA, November 18 – 22, 2019

Agenda item:	26.2

Source: 	Qualcomm Incorporated

Title: 	(TP for TR 38.856) Evaluation and Conclusions for Local Location Management Functionality

Document for:	Discussion and Decision

# 1. 	Discussion

TR 38.856 has defined three alternative architectural solutions for support of local location management functionality in NG-RAN. An evaluation and comparison of these three solutions is needed which can be based on the definition and properties of each solution as described in TR 38.856. Conclusions for local location management functionality in NG-RAN can then be determined based on the evaluation. This TP contains associated proposed additions for the TR.

Proposal: Add the attached text proposal to TR 38.856.

# Text proposal for TR 38.856

[Begin]

## 5.x	Evaluation of Alternatives

Table 5.x-1 provides a side by side comparison of the three alternatives defined in subclause 5.1 based on the description for each alternative. More favourable characteristics in Table 5.x-1 are shown in green highlight.

| Criterion | Alternative 1 | Alternative 2 | Alternative 3 |
| --- | --- | --- | --- |
| Allows multi-vendor support (LMC from different vendor than gNBs) | No | Yes | Yes |
| Options for location continuity following handover | Move LMC to new serving gNB-CU; Keep LMC in previous gNB-CU | Move LMC to new serving gNB; Keep LMC in previous gNB | Change to new LMC; Stay with old LMC |
| New interface or new XnAP impacts needed between a gNB and LMC | No | No | Yes |
| New interface or new F1AP impacts needed between a gNB-CU and LMC | No | Yes | No |
| New XnAP impacts needed between a serving gNB and neighbour gNBs | Yes – to obtain UL measurements, configure DL PRS and support HO No – to send an initial location request | Yes - to obtain UL measurements, configure DL PRS, support HO and send an initial location request if the LMC is in a neighbour gNB | Yes - to obtain UL measurements, configure DL PRS, support HO and send an initial location request if the LMC is only accessible from a neighbour gNB |
| Indication to AMF, or configuration in AMF, of NG-RAN LMC support | Yes | Yes | Yes |
| LMC able to support all functions and all location services for an LMF | Yes | Yes | Yes |
| LMC can coexist with an LMF | Yes | Yes | Yes |
| Ability to support Nlmf service operations between an AMF and LMF | Yes | Yes | Yes |
| Migration and compatibility | Subset of alternative 2. Can migrate to alternative 2 with extra implementation. | Superset of alternative 1. Can support alternative 1 with reduced implementation. | Not compatible with alternative 1 or alternative 2. |

Figure 5.x-1: Comparison of the alternative architecture solutions

All alternatives are able to support the same functions and services as an LMF, can coexist with an LMF and can support location continuity following handover of a target UE.

Alternative 1 minimizes implementation but lacks support for multi-vendor operation. However, alternative 1 is a subset of alternative 2 and can evolve later into alternative 2.

[Next Change]

# 7	Conclusion

This study focused on analysis of local location management functionality and NG RAN acting as LCS client.

For local location management functionality, three alternative architectures were evaluated. Evaluation results have shown that all three are able to support the same functions and services as an LMF, coexist with an LMF and support location continuity following handover of a target UE. Alternative 1 can minimize implementation but lacks support for multi-vendor operation. However, Alternative 1 is a subset of Alternative 2 and can evolve later into Alternative 2. Based on this evaluation, it is recommended to proceed with Alternative 1 in a normative phase with these objectives:

-	Support the same functions and location services that are applicable to an LMF;

-	Support location continuity following handover; and

-	Support coexistence with an LMF.