3GPP TSG-RAN WG3 Meeting #106	R3-197443

Reno, NV, USA, 18-22 November 2019

Agenda Item:	15.3.3

Source:	Ericsson

Title:	Conditional Handover - Disaggregated gNB impact

Document for:	Discussion and Decision

# Introduction

Discussions on Conditional Handover (CHO) are ongoing in RAN2 and RAN3. In this contribution we will elaborate on the impact of recent agreements regarding CHO on F1AP and provide our proposals.

# Discussion

Support for CHO is being discussed in RAN2 and RAN3. During RAN3#105, a discussion was held on the need for the gNB-DU to be informed that a procedure is linked to a CHO or not. The discussion was focused on the presence of a timer in the gNB-DU i.e. to release the resources if the UE is not executing the HO after some time. Even if there is no timer defined in the standard, there is a good chance that the gNB-DU implementation will not keep the resources for an infinite amount of time. Therefore, it is beneficial for the gNB-DU to be informed that the UE may wait longer before accessing the reserved resources.

In the handover preparation procedure, an indication of conditional handover is already agreed upon both for TS 36.423 and for TS 38.423. In its turn this information should be propagated in the case of a disaggregated gNB. In order to cover all the HO cases (i.e. intra-DU and inter-DU), an indication of CHO needs to be added in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message. So, we propose to add the CHO Indicator (agreed already on Xn) in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

Proposal 1: we propose to add a CHO Indicator in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message

# Conclusion

### In this contribution the impact of CHO over F1AP has been discussed and the following proposals were made:

Proposal 1: we propose to add a CHO Indicator in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message

Proposal 2: RAN3 is kindly requested to agree with the changes proposed in the associated CR