3GPP TSG-RAN WG3 #107bis-e     					                                               R3-201916

20-30 April 2020

Online

Agenda item:	9.3.8.1

Source: 	ZTE, China Telecom, China Unicom

Title: 	Discussion on DL UP TNL Information to be setup list over F1

Document for:	Discussion and Approval

Introduction

TS 38.473 v15.9.0 and v16.1.0 have been published, and there is still one issue which needs to be clarified in both of the specifications. In this paper, we will discuss the presence of the IE DL UP TNL Information to be setup List in the UE Context Modification Response message.

Discussion

From the tabular of the UE Context Modification Response message in both TS 38.473 v15.9.0 and v16.1.0, the range of the  IE DL UP TNL Information to be setup List is 1, which means that this list is mandatory in the message.

------------------------------------------------------------------TS 38.473-----------------------------------------------------------------

------------------------------------------------------------------TS 38.473-----------------------------------------------------------------

However, during the UE Context Modification procedure, it is possible that the DL UP TNL information can be unchanged when the DRB is modified. In this case, it is unnecessary to sent the DL UP TNL information through the UE CONTEXT MODIFICATION RESPONSE message. Therefore, the range of DL UP TNL Information to be setup List should not be 1.

For the correction of the specification, there are two options for the UE CONTEXT MODIFICATION RESPONSE message:

- Option 1: Change the range of DL UP TNL Information to be setup list IE from “1” to “0..1” in the IE tabular.

- Option 2: Introduce a new IE Ignore DL UP TNL Information which indicates whether the DL UP TNL Information to be setup List shall be ignored or not.

- Considering that Option1 is a NBC CR, Option2 is preferred. Anyway, here we provide the CR set for both options.

Proposal: For the IE DL UP TNL Information to be setup List in UE Context Modification Response message, two options of modification need to be selected for the specifications of Release 15 and Release 16.

The corresponding CRs for Release 15 are shown in [1] and [2] respectively.

The corresponding CRs for Release 16 are shown in [3] and [4] respectively.

Conclusion

Proposal: For the IE DL UP TNL Information to be setup List in UE Context Modification Response message, two options of modification need to be selected for the specifications of Release 15 and Release 16.

References

- R3-201917 R15_Correction on DL UP TNL Information to be setup list for TS38.473 (Option 1)  ZTE

- R3-201918  R15_Correction on DL UP TNL Information to be setup list for TS38.473 (Option 2)  ZTE

- R3-201919  R16_Correction on DL UP TNL Information to be setup list for TS38.473 (Option 1)  ZTE

- R3-201920  R16_Correction on DL UP TNL Information to be setup list for TS38.473 (Option 2)  ZTE