3GPP TSG-RAN WG3 Meeting #107bis-e	R3-202143

# Online Meeting, 20th – 30th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR |  | Rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of BL CR related to the PDCP Duplication for more than copies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Baseline CR couples the existing PDCP Duplication IE and the Additional PDCP multiplication Information IE and specifies when the later is not present in the messae, what the receiver should do. It would be much clean solution to decouple the two IEs and not touching the legacy PDCP duplication behavior. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Additional PDCP multiplication Information is corrected to Additional PDCP duplication Information to align with the other specification and RAN2. Decouple the PDCP Duplication IE and the Additional PDCP duplication Information. The first IE is to indicate the second copy as in legacy. |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 9.3.1.38, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Text Proposal for TS 38.463

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT SETUP RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	For each established PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each established PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each established DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each established DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Existing Allocated S1 DL UP Transport Layer Information IE or the Existing Allocated NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may re-use the indicated resources already allocated for this bearer context. If the gNB-CU-UP decides to re-use the indicated resources, it shall include the S1 DL UP Unchanged IE or the NG DL UP Unchanged IE in the BEARER CONTEXT SETUP RESPONSE message.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [20].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT SETUP RESPONSE message.

If the DL UP Parameters IE is contained in the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall configure the corresponding information.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE of the BEARER CONTEXT SETUP RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane ciphering for the concerned PDU session.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall use this value when enforcing the maximum integrity protected data rate for the UE.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

For each requested DRB, if the PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-UP shall include two UP Transport Layer Information IEs in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of the two UP Transport Layer Information IEs is for the primary path.

For each requested DRB, if t the Additional PDCP duplication Information IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-UP shall include the same number of UP Transport Layer Information IEs indicated by the Additional PDCP duplication Information IE in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication for intra-gNB-DU CA.

If the PDCP SN Status Information IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the QoS Flow Mapping Indication IE is contained in the QoS Flows Information To Be Setup IE within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Redundant NG UL UP Transport Layer Information IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it as the uplink termination point of the redundant tunnel for the user plane data of those QoS flows in this PDU session which need redundant transmission as described in TS 23.501 [20], and it shall include the Redundant NG DL UP Transport Layer Information IE in the PDU Session Resource Setup List IE in the BEARER CONTEXT SETUP RESPONSE message.

For each PDU Session Resource, if the Redundant Common Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [20].

For each PDU session, if the Redundant QoS Flow Indicator IE is included in the QoS Flow QoS Parameters List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider it for the redundant transmission.

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take it into account as specified in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT SETUP RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT SETUP REQUEST message.

If the Trace Activation IE is included in the BEARER CONTEXT SETUP REQUEST message the gNB-CU-UP shall, if supported, initiate the requested trace function as described in TS 32.422 [24].

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the UE CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the UE CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

If the TSC Traffic Characteristics IE is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20].

Next of Text Proposal for TS 38.463

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT MODIFICATION RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	A list of PDU Session Resources which are successfully modified shall be included in the PDU Session Resource Modified List IE;

-	A list of PDU Session Resources which failed to be modified shall be included in the PDU Session Resource Failed To Modify List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Security Information IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Aggregate Maximum Bit Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information, except for the PDCP SN UL Size IE, the PDCP SN DL Size IE and the RLC mode IE which shall be ignored.

If the E-UTRAN QoS IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the PDCP SN Status Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the UL COUNT Value IE and the DL COUNT Value IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP SN Status Information IE is contained in the DRB To Setup List IE or the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the DL UP Parameters IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Cell Group To Add IE or the Cell Group To Modify IE or the Cell Group To Remove IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall add or modify or remove the corresponding cell group.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501 [20].

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the SDAP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Flow Mapping Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

For each requested DRB, if the PDCP Duplication IE or Additional PDCP duplication Information IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-CP shall include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION REQUEST message, and the gNB-CU-UP shall also include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of these UP Transport Layer Information IEs is for the primary path.

For a certain DRB which was allocated with two or more GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the Bearer Context Modification (gNB-CU-CP initiated) procedure, i.e. only one UP Transport Layer Information per Cell Group ID is present in DL UP Parameters IE for the concerned DRB, then the gNB-CU-UP shall consider that PDCP duplication is deconfigured for this DRB. If such Bearer Context Modification (gNB-CU-CP initiated) procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the New UL TNL Information Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the new UP Transport Layer Information in the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE of the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List of the BEARER CONTEXT MODIFICATION REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane ciphering for the concerned PDU session.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Redundant NG UL UP Transport Layer Information IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, include the Redundant NG DL UP Transport Layer Information IE in the PDU Session Resource Setup List IE or the PDU Session Resource Modified List IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Redundant Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [20].

For each PDU session for which the Redundant QoS Flow Indicator IE is included in QoS Flows Information To Be Setup IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if support, shall store and use it as specified in TS 23.501 [20].

For each PDU session, if the Redundant QoS Flow Indicator IE is set to false for all QoS flows, the gNB-CU-UP shall, if supported, stop the redundant transmission and release the redundant tunnel for the concerned PDU session as specified in TS 23.501 [20].

If the QoS Flow Mapping Indication IE is contained in the QoS Flow QoS Parameters List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Data Discard Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to “Required”, the gNB-CU-UP shall consider that a RAN Paging Failure occurred for that UE. The gNB-CU-UP shall discard the user plane data for that UE and consider that the bearer context is still suspended.

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store the corresponding information and replace any existing information.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take it into account for each DRB, as specified in TS 28.552 [22].

If the DRB QoS IE is contained within the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account for each DRB, as specifed in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message including Activity Notification Level IE and its value does not match the current bearer context, the gNB-CU-UP shall ignore the Activity Notification Level IE and also the requested modification of inactivity timer.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT MODIFICATION RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT MODIFICATION REQUEST message.

If the Old QoS Flow List - UL End Marker expected IE is included in the PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message for a DRB to be modified, the gNB-CU-UP shall consider that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. The gNB-CU-UP shall consider that the Old QoS Flow List - UL End Marker expected  IE only contains UL QoS flow information for QoS flows for which no SDAP end marker has been yet received on the source side.

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

If there is at least one DRB removed by the gNB-CU-UP, the gNB-CU-UP shall, if supported, include the Retainability Measurements Information IE in the BEARER CONTEXT MODIFICATION RESPONSE message, providing information on the removed DRB(s) for retainability measurements in the gNB-CU-CP, as described in TS 32.425 [26] and TS 28.552 [22].

If the TSC Traffic Characteristics IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20].

Next of Text Proposal for TS 38.463

### 8.3.3	Bearer Context Modification Required (gNB-CU-UP initiated)

#### 8.3.3.1	General

The purpose of the Bearer Context Modification Required procedure is to allow the gNB-CU-UP to modify a bearer context (e.g., due to local problems) and inform the gNB-CU-CP. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: Bearer Context Modification Required procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies with the BEARER CONTEXT MODIFICATION CONFIRM message.

If the S1 DL UP Transport Layer Information IE or the NG DL UP Transport Layer Information IE or the Redundant NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall update the corresponding information.

If the gNB-CU-UP Cell Group Related Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall try to change the cell group related configuration accordingly. If the gNB-CU-CP is not able to update the requested cell group related configuration, it shall include the Cell Group Information IE with the current cell group configuration in the DRB Modified List IE in the BEARER CONTEXT MODIFICATION CONFIRM message.

#### 8.3.3.3	Abnormal Conditions

Not applicable.

Next of Text Proposal for TS 38.463

#### 9.3.1.25	QoS Flow QoS Parameters List

This IE contains a list of QoS Flows including the QoS Flow parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow List |  | 1 |  |  | - | - |
| >QoS Flow Item |  | 1..<maxnoofflows> |  |  | - | - |
| >>QoS Flow Identifier | M |  | 9.3.1.24 |  | - | - |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.26 |  | - | - |
| >>QoS Flow Mapping Indication | O |  | 9.3.1.60 | Indicates that only the uplink or downlink QoS flow is mapped to the DRB | - | - |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.x | This IE indicates that this QoS flow is requested for the redundant transmission. | YES | ignore |
| >>TSC Traffic Characteristics | O |  | 9.3.1.x1 | Traffic pattern information associated with the QFI. Details in TS 23.501 [20]. | YES | ignore |

#### 9.3.1.27	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255, …) | This IE contains the standardized or pre-configured 5QI as specified in TS 23.501 [20]. | - | - |
| Priority Level | O |  | 9.3.1.51 | For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. | - | - |
| Averaging Window | O |  | 9.3.1.49 | This IE applies to GBR QoS Flows only. For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. | - | - |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. | - | - |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.z3 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.z3 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

#### 9.3.1.28	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Priority Level | M |  | 9.3.1.51 | For details see TS 23.501 [20]. | - | - |
| Packet Delay Budget | M |  | 9.3.1.47 | For details see TS 23.501 [20]. This IE is ignored if the Extended Packet Delay Budget IE is present. | - | - |
| Packet Error Rate | M |  | 9.3.1.48 | For details see TS 23.501 [20]. | - | - |
| 5QI | O |  | INTEGER (0..255,…) | This IE contains the dynamically assigned 5QI as specified in TS 23.501 [20]. | - | - |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (delay critical, non-delay critical) | For details see TS 23.501 [20]. | - | - |
| Averaging Window | C-ifGBRflow |  | 9.3.1.49 | For details see TS 23.501 [20]. | - | - |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. This IE shall be included if the Delay Critical IE is set to “delay critical” and is ignored otherwise. | - | - |
| Extended Packet Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.z3 | Packet Delay Budget is specified in TS 23.501 [9] | YES | ignore |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.z3 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.z3 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

Next of Text Proposal for TS 38.463

#### 9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP SN UL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN UL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. |
| PDCP SN DL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN DL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. |
| RLC mode | M |  | ENUMERATED (RLC-TM, RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, …) | Indicates the RLC mode for the DRB. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. |
| ROHC Parameters | O |  | 9.3.1.40 |  |
| T-Reordering Timer | O |  | 9.3.1.41 |  |
| Discard Timer | O |  | 9.3.1.42 |  |
| UL Data Split Threshold | O |  | 9.3.1.43 |  |
| PDCP Duplication | O |  | ENUMERATED (True, …) | Indicates whether PDCP duplication is to be configured for the DRB. |
| PDCP Re-establishment | O |  | ENUMERATED (true,…) | Indicates PDCP entity re-establishment to be triggered as defined in TS 38.323 [17] |
| PDCP Data Recovery | O |  | ENUMERATED (true,…) | Indicates PDCP data recovery to be triggered as defined in TS 38.323 [17] |
| Duplication Activation | O |  | ENUMERATED ( Active, Inactive, …) | Information on the initial state of DL PDCP duplication |
| Out Of Order Delivery | O |  | ENUMERATED (true,…) | Indicates whether or not outOfOrderDelivery specified in TS 38.323 [17] is configured. Out of order delivery is configured only when the radio bearer is established. |
| Additional PDCP duplication Information | O |  | ENUMERATED (three, four, …) | Indicates the number of PDCP duplication configured when it is more than 2 for the DRB |

Next of Text Proposal for TS 38.463

#### 9.3.1.x	Redundant QoS Flow Indicator

This IE provides the Redundant QoS Flow Indicator for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Redundant QoS Flow Indicator | M |  | ENUMERATED (true, false) | This IE indicates that this QoS flow is requested for the redundant transmission, as specified in TS 23.501 [20]. |

Next of Text Proposal for TS 38.463

#### 9.3.1.x1	TSC Traffic Characteristics

This IE provides the traffic characteristics of TSC QoS flows.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Assistance Information Downlink | O |  | TSC Assistance Information 9.3.1.y1 |  |
| TSC Assistance Information Uplink | O |  | TSC Assistance Information 9.3.1.y1 |  |

#### 9.3.1.y1	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [20]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.z1 |  |
| Burst Arrival Time | O |  | 9.3.1.z2 |  |

#### 9.3.1.z1	Periodicity

This IE indicates the Periodicity of the TSC QoS flow as defined in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..640000, …) | Periodicity expressed in units of 1 us. |

#### 9.3.1.z2	Burst Arrival Time

This IE indicates the Burst Arrival Time of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time | M |  | OCTET STRING | Encoded in the same format as the ReferenceTime IE as defined in TS 38.331 [10]. The value is truncated to 1 us granularity. |

#### 9.3.1.z3	Extended Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Packet Delay Budget | M |  | INTEGER (0..65535, …) | Upper bound value for the delay that a packet may experience expressed in unit of 0.01ms. |

Next of Text Proposal for TS 38.463

#### 9.3.3.2	PDU Session Resource To Setup List

This IE contains PDU session resource related information used at Bearer Context Setup Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when at least one Non-GBR QoS Flows is being setup. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Existing Allocated NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >Network Instance | O |  | 9.3.1.62 | Shall be ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Contains the PDCP SN Status at setup after Resume. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB. | YES | ignore |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next of Text Proposal for TS 38.463

#### 9.3.3.5	PDU Session Resource Setup List

This IE contains setup PDU session resource related information used at Bearer Context Setup Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Result | O |  | 9.3.1.52 |  | - | - |
| >NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |
| >NG DL UP Unchanged | O |  | ENUMERATED (True, …) |  | - | - |
| >DRB Setup List |  | 1 |  |  | - | - |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next of Text Proposal for TS 38.463

#### 9.3.3.10	PDU Session Resource To Setup Modification List

This IE contains PDU session resource to setup related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when Non-GBR QoS Flows are setting up. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 |  | - | - |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next of Text Proposal for TS 38.463

#### 9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Indication | O |  | 9.3.1.23 | This IE is not used in this release. | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | - | - |
| >NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >PDU Session Data Forwarding Information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 | Shall be ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flow Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>DRB Data forwarding information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | This IE is included to be used for indicating that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. It shall only contain UL QoS flow information for QoS flows for which no SDAP end marker has been yet received on the source side. | YES | reject |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >S-NSSAI | O |  | 9.3.1.9 |  | YES | reject |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next of Text Proposal for TS 38.463

#### 9.3.3.17	PDU Session Resource Setup Modification List

This IE contains setup PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Result | O |  | 9.3.1.52 |  | - | - |
| >NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >DRB Setup List |  | 1 |  |  | - | - |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |

Next of Text Proposal for TS 38.463

#### 9.3.3.19	PDU Session Resource Modified List

This IE contains modified PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Modified Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >Security Result | O |  | 9.3.1.52 |  | - | - |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 |  | - | - |
| >DRB Setup List |  | 0.. 1 |  |  | - | - |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 |  | - | - |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >DRB Modified List |  | 0.. 1 |  |  | - | - |
| >>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >DRB Failed To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next of Text Proposal for TS 38.463

#### 9.3.3.23	PDU Session Resource Required To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Required

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Required To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>gNB-CU-UP Cell Group Related Configuration | O |  | 9.3.1.34 |  | - | - |
| >>>Flow To Remove | O |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>Cause | O |  | 9.3.1.2 |  | - | - |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next of Text Proposal for TS 38.463

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM E1AP-CommonDataTypes

Reset,

ResetAcknowledge,

ErrorIndication,

GNB-CU-UP-E1SetupRequest,

GNB-CU-UP-E1SetupResponse,

GNB-CU-UP-E1SetupFailure,

GNB-CU-CP-E1SetupRequest,

GNB-CU-CP-E1SetupResponse,

GNB-CU-CP-E1SetupFailure,

GNB-CU-UP-ConfigurationUpdate,

GNB-CU-UP-ConfigurationUpdateAcknowledge,

GNB-CU-UP-ConfigurationUpdateFailure,

GNB-CU-CP-ConfigurationUpdate,

GNB-CU-CP-ConfigurationUpdateAcknowledge,

GNB-CU-CP-ConfigurationUpdateFailure,

BearerContextSetupRequest,

BearerContextSetupResponse,

BearerContextSetupFailure,

BearerContextModificationRequest,

BearerContextModificationResponse,

BearerContextModificationFailure,

BearerContextModificationRequired,

BearerContextModificationConfirm,

BearerContextReleaseCommand,

BearerContextReleaseComplete,

BearerContextReleaseRequest,

BearerContextInactivityNotification,

DLDataNotification,

ULDataNotification,

DataUsageReport,

E1ReleaseRequest,

E1ReleaseResponse,

GNB-CU-UP-CounterCheckRequest,

GNB-CU-UP-StatusIndication,

MRDC-DataUsageReport,

DeactivateTrace,

TraceStart,

PrivateMessage

FROM E1AP-PDU-Contents

id-reset,

id-errorIndication,

id-gNB-CU-UP-E1Setup,

id-gNB-CU-CP-E1Setup,

id-gNB-CU-UP-ConfigurationUpdate,

id-gNB-CU-CP-ConfigurationUpdate,

id-e1Release,

id-bearerContextSetup,

id-bearerContextModification,

id-bearerContextModificationRequired,

id-bearerContextRelease,

id-bearerContextReleaseRequest,

id-bearerContextInactivityNotification,

id-dLDataNotification,

id-uLDataNotification,

id-dataUsageReport,

id-gNB-CU-UP-CounterCheck,

id-gNB-CU-UP-StatusIndication,

id-mRDC-DataUsageReport,

id-DeactivateTrace,

id-TraceStart,

id-privateMessage

FROM E1AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

E1AP-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome		UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURES E1AP-ELEMENTARY-PROCEDURE ::= {

E1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

E1AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-1 E1AP-ELEMENTARY-PROCEDURE ::= {

reset										|

gNB-CU-UP-E1Setup							|

gNB-CU-CP-E1Setup							|

gNB-CU-UP-ConfigurationUpdate				|

gNB-CU-CP-ConfigurationUpdate				|

e1Release									|

bearerContextSetup							|

bearerContextModification					|

bearerContextModificationRequired			|

bearerContextRelease						,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-2 E1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

bearerContextReleaseRequest				|

bearerContextInactivityNotification		|

dLDataNotification						|

uLDataNotification						|

dataUsageReport							|

gNB-CU-UP-CounterCheck					|

gNB-CU-UP-StatusIndication			|

mRDC-DataUsageReport					|

deactivateTrace							|

traceStart								|

privateMessage							,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

reset E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

gNB-CU-UP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-UP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-UP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-UP-E1Setup

CRITICALITY				reject

}

gNB-CU-CP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-CP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-CP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-CP-E1Setup

CRITICALITY				reject

}

gNB-CU-UP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-UP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-UP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-UP-ConfigurationUpdate

CRITICALITY				reject

}

gNB-CU-CP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-CP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-CP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-CP-ConfigurationUpdate

CRITICALITY				reject

}

e1Release E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E1ReleaseRequest

SUCCESSFUL OUTCOME		E1ReleaseResponse

PROCEDURE CODE			id-e1Release

CRITICALITY				reject

}

bearerContextSetup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextSetupRequest

SUCCESSFUL OUTCOME		BearerContextSetupResponse

UNSUCCESSFUL OUTCOME	BearerContextSetupFailure

PROCEDURE CODE			id-bearerContextSetup

CRITICALITY				reject

}

bearerContextModification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequest

SUCCESSFUL OUTCOME		BearerContextModificationResponse

UNSUCCESSFUL OUTCOME	BearerContextModificationFailure

PROCEDURE CODE			id-bearerContextModification

CRITICALITY				reject

}

bearerContextModificationRequired E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequired

SUCCESSFUL OUTCOME		BearerContextModificationConfirm

PROCEDURE CODE			id-bearerContextModificationRequired

CRITICALITY				reject

}

bearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseCommand

SUCCESSFUL OUTCOME		BearerContextReleaseComplete

PROCEDURE CODE			id-bearerContextRelease

CRITICALITY				reject

}

bearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseRequest

PROCEDURE CODE			id-bearerContextReleaseRequest

CRITICALITY				ignore

}

bearerContextInactivityNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextInactivityNotification

PROCEDURE CODE			id-bearerContextInactivityNotification

CRITICALITY				ignore

}

dLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLDataNotification

PROCEDURE CODE			id-dLDataNotification

CRITICALITY				ignore

}

uLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULDataNotification

PROCEDURE CODE			id-uLDataNotification

CRITICALITY				ignore

}

dataUsageReport E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataUsageReport

PROCEDURE CODE			id-dataUsageReport

CRITICALITY				ignore

}

gNB-CU-UP-CounterCheck E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-CounterCheckRequest

PROCEDURE CODE			id-gNB-CU-UP-CounterCheck

CRITICALITY				ignore

}

gNB-CU-UP-StatusIndication 	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-StatusIndication

PROCEDURE CODE			id-gNB-CU-UP-StatusIndication

CRITICALITY			ignore

}

privateMessage E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

mRDC-DataUsageReport	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MRDC-DataUsageReport

PROCEDURE CODE			id-mRDC-DataUsageReport

CRITICALITY				ignore

}

deactivateTrace E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

traceStart E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID,

TraceID,

TraceActivation,

SubscriberProfileIDforRFP,

AdditionalRRMPriorityIndex,

RetainabilityMeasurementsInfo,

Transport-Layer-Address-Info

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-TraceID,

id-TraceActivation,

id-SubscriberProfileIDforRFP,

id-AdditionalRRMPriorityIndex,

id-RetainabilityMeasurementsInfo,

id-Transport-Layer-Address-Info,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset

FROM E1AP-Constants;

-- **************************************************************

--

#### -- RESET

--

-- **************************************************************

-- **************************************************************

--

-- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

e1-Interface					ResetAll,

partOfE1-Interface				UE-associatedLogicalE1-ConnectionListRes,

choice-extension				ProtocolIE-SingleContainer	{{ResetType-ExtIEs}}

}

ResetType-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalE1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualE1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalE1-ConnectionItemRes } }

UE-associatedLogicalE1-ConnectionItemRes E1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalE1-ConnectionItem	CRITICALITY reject	TYPE UE-associatedLogicalE1-ConnectionItem	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-UE-associatedLogicalE1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalE1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalE1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualE1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalE1-ConnectionItemResAck } }

UE-associatedLogicalE1-ConnectionItemResAck 	E1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalE1-ConnectionItem	 CRITICALITY ignore 	TYPE UE-associatedLogicalE1-ConnectionItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY ignore	TYPE GNB-CU-CP-UE-E1AP-ID		PRESENCE optional}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID		PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE optional}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- GNB-CU-UP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Request

--

-- **************************************************************

GNB-CU-UP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupRequestIEs} },

...

}

GNB-CU-UP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

SupportedPLMNs-List	::=	SEQUENCE (SIZE (1..maxnoofSPLMNs)) OF SupportedPLMNs-Item

SupportedPLMNs-Item ::= SEQUENCE {

pLMN-Identity					PLMN-Identity,

slice-Support-List				Slice-Support-List												OPTIONAL,

nR-CGI-Support-List				NR-CGI-Support-List												OPTIONAL,

qoS-Parameters-Support-List		QoS-Parameters-Support-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SupportedPLMNs-ExtIEs } }		OPTIONAL,

...

}

SupportedPLMNs-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Response

--

-- **************************************************************

GNB-CU-UP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupResponseIEs} },

...

}

GNB-CU-UP-E1SetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name						PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Failure

--

-- **************************************************************

GNB-CU-UP-E1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupFailureIEs} },

...

}

GNB-CU-UP-E1SetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-CP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Request

--

-- **************************************************************

GNB-CU-CP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupRequestIEs} },

...

}

GNB-CU-CP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name						PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Response

--

-- **************************************************************

GNB-CU-CP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupResponseIEs} },

...

}

GNB-CU-CP-E1SetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Failure

--

-- **************************************************************

GNB-CU-CP-E1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupFailureIEs} },

...

}

GNB-CU-CP-E1SetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-UP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP Configuration Update

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory }|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE optional	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-GNB-CU-UP-TNLA-To-Remove-List		CRITICALITY reject	TYPE GNB-CU-UP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

GNB-CU-UP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-UP-TNLA-To-Remove-Item

-- **************************************************************

--

-- GNB-CU-UP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-UP Configuration Update Failure

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateFailureIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-CP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP Configuration Update

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name						PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Add-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Add-List		PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Remove-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Update-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Update-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-To-Add-List       ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Add-Item

GNB-CU-CP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Remove-Item

GNB-CU-CP-TNLA-To-Update-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Update-Item

-- **************************************************************

--

-- GNB-CU-CP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Setup-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Setup-List					PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Failed-To-Setup-List	CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Failed-To-Setup-List		PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-Setup-List				::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Setup-Item

GNB-CU-CP-TNLA-Failed-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Failed-To-Setup-Item

-- **************************************************************

--

-- GNB-CU-CP Configuration Update Failure

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateFailureIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- E1 RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- E1 Release Request

--

-- **************************************************************

E1ReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E1ReleaseRequestIEs} },

...

}

E1ReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- E1 Release Response

--

-- **************************************************************

E1ReleaseResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E1ReleaseResponseIEs} },

...

}

E1ReleaseResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT SETUP

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Setup Request

--

-- **************************************************************

BearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupRequestIEs} },

...

}

BearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-SecurityInformation					CRITICALITY reject	TYPE SecurityInformation							PRESENCE mandatory }|

{ ID id-UEDLAggregateMaximumBitRate			CRITICALITY reject	TYPE BitRate										PRESENCE mandatory }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate	CRITICALITY reject		TYPE BitRate								PRESENCE optional	 }|

{ ID id-Serving-PLMN						CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE mandatory }|

{ ID id-ActivityNotificationLevel			CRITICALITY reject	TYPE ActivityNotificationLevel					PRESENCE mandatory }|

{ ID id-UE-Inactivity-Timer					CRITICALITY reject	TYPE Inactivity-Timer								PRESENCE optional  }|

{ ID id-BearerContextStatusChange			CRITICALITY reject	TYPE BearerContextStatusChange					PRESENCE optional  }|

{ ID id-System-BearerContextSetupRequest	CRITICALITY reject	TYPE System-BearerContextSetupRequest				PRESENCE mandatory }|

{ ID id-RANUEID								CRITICALITY ignore	TYPE RANUEID										PRESENCE optional }|

{ ID id-GNB-DU-ID							CRITICALITY ignore	TYPE GNB-DU-ID										PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional },

...

}

System-BearerContextSetupRequest	::=	CHOICE {

e-UTRAN-BearerContextSetupRequest		ProtocolIE-Container		 	{{EUTRAN-BearerContextSetupRequest}},

nG-RAN-BearerContextSetupRequest		ProtocolIE-Container		 	{{NG-RAN-BearerContextSetupRequest}},

choice-extension						ProtocolIE-SingleContainer		{{System-BearerContextSetupRequest-ExtIEs}}

}

System-BearerContextSetupRequest-ExtIEs E1AP-PROTOCOL-IES::= {

...

}

EUTRAN-BearerContextSetupRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-To-Setup-List-EUTRAN		CRITICALITY reject	 TYPE DRB-To-Setup-List-EUTRAN		PRESENCE mandatory }|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	 TYPE SubscriberProfileIDforRFP		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex		CRITICALITY ignore	 TYPE AdditionalRRMPriorityIndex	PRESENCE optional },

...

}

NG-RAN-BearerContextSetupRequest E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-To-Setup-List		CRITICALITY reject	 TYPE PDU-Session-Resource-To-Setup-List		PRESENCE mandatory },

...

}

-- **************************************************************

--

-- Bearer Context Setup Response

--

-- **************************************************************

BearerContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupResponseIEs} },

...

}

BearerContextSetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory	}|

{ ID id-System-BearerContextSetupResponse	CRITICALITY ignore	TYPE System-BearerContextSetupResponse				PRESENCE mandatory  },

...

}

System-BearerContextSetupResponse::=	CHOICE {

e-UTRAN-BearerContextSetupResponse		ProtocolIE-Container 			{{EUTRAN-BearerContextSetupResponse}},

nG-RAN-BearerContextSetupResponse		ProtocolIE-Container 			{{NG-RAN-BearerContextSetupResponse}},

choice-extension						ProtocolIE-SingleContainer		{{System-BearerContextSetupResponse-ExtIEs}}

}

System-BearerContextSetupResponse-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextSetupResponse E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Setup-List-EUTRAN 		CRITICALITY ignore	 TYPE DRB-Setup-List-EUTRAN			PRESENCE mandatory }|

{ ID id-DRB-Failed-List-EUTRAN 		CRITICALITY ignore	 TYPE DRB-Failed-List-EUTRAN		PRESENCE optional  },

...

}

NG-RAN-BearerContextSetupResponse E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Setup-List 	CRITICALITY ignore	 TYPE PDU-Session-Resource-Setup-List		PRESENCE mandatory }|

{ ID id-PDU-Session-Resource-Failed-List 	CRITICALITY ignore	 TYPE PDU-Session-Resource-Failed-List		PRESENCE optional  },

...

}

-- **************************************************************

--

-- Bearer Context Setup Failure

--

-- **************************************************************

BearerContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupFailureIEs} },

...

}

BearerContextSetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT MODIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Modification Request

--

-- **************************************************************

BearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationRequestIEs} },

...

}

BearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-SecurityInformation						CRITICALITY reject	TYPE SecurityInformation									PRESENCE optional  }|

{ ID id-UEDLAggregateMaximumBitRate				CRITICALITY reject	TYPE BitRate												PRESENCE optional  }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate		CRITICALITY reject	TYPE BitRate											PRESENCE optional	 }|

{ ID id-BearerContextStatusChange				CRITICALITY reject	TYPE BearerContextStatusChange							PRESENCE optional  }|

{ ID id-New-UL-TNL-Information-Required			CRITICALITY reject	TYPE New-UL-TNL-Information-Required					PRESENCE optional  }|

{ ID id-UE-Inactivity-Timer						CRITICALITY reject	TYPE Inactivity-Timer										PRESENCE optional  }|

{ ID id-DataDiscardRequired					CRITICALITY ignore	TYPE DataDiscardRequired											PRESENCE optional  }|

{ ID id-System-BearerContextModificationRequest	CRITICALITY reject	TYPE System-BearerContextModificationRequest		PRESENCE optional  }|

{ ID id-RANUEID										CRITICALITY ignore	TYPE RANUEID												PRESENCE optional }|

{ ID id-GNB-DU-ID									CRITICALITY ignore	TYPE GNB-DU-ID												PRESENCE optional }|

{ ID id-ActivityNotificationLevel					CRITICALITY ignore	TYPE ActivityNotificationLevel							PRESENCE optional },

...

}

System-BearerContextModificationRequest	::=	CHOICE {

e-UTRAN-BearerContextModificationRequest		ProtocolIE-Container 			{{EUTRAN-BearerContextModificationRequest}},

nG-RAN-BearerContextModificationRequest			ProtocolIE-Container 			{{NG-RAN-BearerContextModificationRequest}},

choice-extension								ProtocolIE-SingleContainer		{{System-BearerContextModificationRequest-ExtIEs}}

}

System-BearerContextModificationRequest-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-To-Setup-Mod-List-EUTRAN		CRITICALITY reject	 TYPE DRB-To-Setup-Mod-List-EUTRAN		PRESENCE optional }|

{ ID id-DRB-To-Modify-List-EUTRAN			CRITICALITY reject	 TYPE DRB-To-Modify-List-EUTRAN			PRESENCE optional }|

{ ID id-DRB-To-Remove-List-EUTRAN			CRITICALITY reject	 TYPE DRB-To-Remove-List-EUTRAN			PRESENCE optional }|

{ ID id-SubscriberProfileIDforRFP			CRITICALITY ignore	 TYPE 	SubscriberProfileIDforRFP		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	 TYPE 	AdditionalRRMPriorityIndex		PRESENCE optional },

...

}

NG-RAN-BearerContextModificationRequest E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-To-Setup-Mod-List	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Setup-Mod-List	PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Modify-List 	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Modify-List		PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Remove-List		CRITICALITY reject	 TYPE PDU-Session-Resource-To-Remove-List		PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Response

--

-- **************************************************************

BearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationResponseIEs} },

...

}

BearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-System-BearerContextModificationResponse	CRITICALITY ignore	TYPE System-BearerContextModificationResponse		PRESENCE optional  },

...

}

System-BearerContextModificationResponse	::=	CHOICE {

e-UTRAN-BearerContextModificationResponse			ProtocolIE-Container {{EUTRAN-BearerContextModificationResponse}},

nG-RAN-BearerContextModificationResponse			ProtocolIE-Container {{NG-RAN-BearerContextModificationResponse}},

choice-extension									ProtocolIE-SingleContainer {{System-BearerContextModificationResponse-ExtIEs}}

}

System-BearerContextModificationResponse-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Setup-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Setup-Mod-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Failed-Mod-List-EUTRAN				PRESENCE optional }|

{ ID id-DRB-Modified-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Modified-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-To-Modify-List-EUTRAN		CRITICALITY ignore	TYPE DRB-Failed-To-Modify-List-EUTRAN		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo			CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo				PRESENCE optional },

...

}

NG-RAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Setup-Mod-List 		CRITICALITY reject	TYPE PDU-Session-Resource-Setup-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-Mod-List		CRITICALITY reject	TYPE PDU-Session-Resource-Failed-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Modified-List			CRITICALITY reject	TYPE PDU-Session-Resource-Modified-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-To-Modify-List	CRITICALITY reject	TYPE PDU-Session-Resource-Failed-To-Modify-List		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo				CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo						PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Failure

--

-- **************************************************************

BearerContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationFailureIEs} },

...

}

BearerContextModificationFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Modification Required

--

-- **************************************************************

BearerContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationRequiredIEs} },

...

}

BearerContextModificationRequiredIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-System-BearerContextModificationRequired	CRITICALITY reject	TYPE System-BearerContextModificationRequired			PRESENCE mandatory },

...

}

System-BearerContextModificationRequired	::=	CHOICE {

e-UTRAN-BearerContextModificationRequired		ProtocolIE-Container {{EUTRAN-BearerContextModificationRequired}},

nG-RAN-BearerContextModificationRequired		ProtocolIE-Container {{NG-RAN-BearerContextModificationRequired}},

choice-extension								ProtocolIE-SingleContainer {{System-BearerContextModificationRequired-ExtIEs}}

}

System-BearerContextModificationRequired-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationRequired E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Required-To-Modify-List-EUTRAN	CRITICALITY reject	TYPE DRB-Required-To-Modify-List-EUTRAN	PRESENCE optional }|

{ ID id-DRB-Required-To-Remove-List-EUTRAN	CRITICALITY reject	TYPE DRB-Required-To-Remove-List-EUTRAN	PRESENCE optional },

...

}

NG-RAN-BearerContextModificationRequired E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Required-To-Modify-List	CRITICALITY reject	TYPE PDU-Session-Resource-Required-To-Modify-List	PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Remove-List	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Remove-List	PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Confirm

--

-- **************************************************************

BearerContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationConfirmIEs} },

...

}

BearerContextModificationConfirmIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-System-BearerContextModificationConfirm		CRITICALITY ignore	TYPE System-BearerContextModificationConfirm		PRESENCE optional  },

...

}

System-BearerContextModificationConfirm	::=	CHOICE {

e-UTRAN-BearerContextModificationConfirm		ProtocolIE-Container {{EUTRAN-BearerContextModificationConfirm}},

nG-RAN-BearerContextModificationConfirm			ProtocolIE-Container {{NG-RAN-BearerContextModificationConfirm}},

choice-extension								ProtocolIE-SingleContainer {{System-BearerContextModificationConfirm-ExtIEs}}

}

System-BearerContextModificationConfirm-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationConfirm E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Confirm-Modified-List-EUTRAN	CRITICALITY ignore	 TYPE DRB-Confirm-Modified-List-EUTRAN	PRESENCE optional },

...

}

NG-RAN-BearerContextModificationConfirm E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Confirm-Modified-List	CRITICALITY ignore	 TYPE PDU-Session-Resource-Confirm-Modified-List	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Release Command

--

-- **************************************************************

BearerContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseCommandIEs} },

...

}

BearerContextReleaseCommandIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory },

...

}

-- **************************************************************

--

-- Bearer Context Release Complete

--

-- **************************************************************

BearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseCompleteIEs} },

...

}

BearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	}|

{ ID id-RetainabilityMeasurementsInfo	CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Release Request

--

-- **************************************************************

BearerContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseRequestIEs} },

...

}

BearerContextReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-DRB-Status-List					CRITICALITY ignore	TYPE DRB-Status-List				PRESENCE optional  }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory },

...

}

DRB-Status-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Status-Item

-- **************************************************************

--

#### -- BEARER CONTEXT INACTIVITY NOTIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Inactivity Notification

--

-- **************************************************************

BearerContextInactivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextInactivityNotificationIEs } },

...

}

BearerContextInactivityNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-ActivityInformation				CRITICALITY reject	TYPE ActivityInformation			PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- DL DATA NOTIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- DL Data Notification

--

-- **************************************************************

DLDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DLDataNotificationIEs } },

...

}

DLDataNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-PPI							CRITICALITY ignore	TYPE PPI							PRESENCE optional },

...

}

-- **************************************************************

-- **************************************************************

--

-- UL Data Notification

--

-- **************************************************************

ULDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ULDataNotificationIEs } },

...

}

ULDataNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-PDU-Session-To-Notify-List		CRITICALITY reject	 TYPE PDU-Session-To-Notify-List	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- DATA USAGE REPORT

--

-- **************************************************************

-- **************************************************************

--

-- Data Usage Report

--

-- **************************************************************

DataUsageReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DataUsageReportIEs } },

...

}

DataUsageReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-Data-Usage-Report-List			CRITICALITY ignore	TYPE Data-Usage-Report-List			PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- GNB-CU-UP COUNTER CHECK

--

-- **************************************************************

-- **************************************************************

--

-- gNB-CU-UP Counter Check Request

--

-- **************************************************************

GNB-CU-UP-CounterCheckRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNB-CU-UP-CounterCheckRequestIEs } },

...

}

GNB-CU-UP-CounterCheckRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-System-GNB-CU-UP-CounterCheckRequest	CRITICALITY reject	TYPE System-GNB-CU-UP-CounterCheckRequest	PRESENCE mandatory },

...

}

System-GNB-CU-UP-CounterCheckRequest	::=	CHOICE {

e-UTRAN-GNB-CU-UP-CounterCheckRequest		ProtocolIE-Container		{{EUTRAN-GNB-CU-UP-CounterCheckRequest}},

nG-RAN-GNB-CU-UP-CounterCheckRequest		ProtocolIE-Container		{{NG-RAN-GNB-CU-UP-CounterCheckRequest}},

choice-extension							ProtocolIE-SingleContainer	{{System-GNB-CU-UP-CounterCheckRequest-ExtIEs}}

}

System-GNB-CU-UP-CounterCheckRequest-ExtIEs E1AP-PROTOCOL-IES::= {

...

}

EUTRAN-GNB-CU-UP-CounterCheckRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Subject-To-Counter-Check-List-EUTRAN	CRITICALITY ignore	 TYPE DRBs-Subject-To-Counter-Check-List-EUTRAN	PRESENCE mandatory },

...

}

NG-RAN-GNB-CU-UP-CounterCheckRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Subject-To-Counter-Check-List-NG-RAN	CRITICALITY ignore	 TYPE DRBs-Subject-To-Counter-Check-List-NG-RAN	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- gNB-CU-UP STATUS INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- gNB-CU-UP Status Indication

--

-- **************************************************************

GNB-CU-UP-StatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNB-CU-UP-StatusIndicationIEs} },

...

}

GNB-CU-UP-StatusIndicationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID				 			PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-OverloadInformation	CRITICALITY reject	TYPE GNB-CU-UP-OverloadInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MR-DC DATA USAGE REPORT

--

-- **************************************************************

MRDC-DataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MRDC-DataUsageReportIEs } },

...

}

MRDC-DataUsageReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-PDU-Session-Resource-Data-Usage-List	CRITICALITY ignore		TYPE PDU-Session-Resource-Data-Usage-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation					CRITICALITY ignore	TYPE TraceActivation					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceID							CRITICALITY ignore	TYPE TraceID							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs E1AP-PRIVATE-IES ::= {

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

ActivityInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ActivityNotificationLevel	::=	ENUMERATED	{

drb,

pdu-session,

ue,

...

}

AdditionalPDCPduplicationInformation	::=	ENUMERATED	{

three,

four,

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AveragingWindow  ::= INTEGER (0..4095, ...)

#### -- B

BearerContextStatusChange	::= 	ENUMERATED {

suspend,

resume,

...

}

BitRate ::= INTEGER (0..4000000000000,...)

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer	{{Cause-ExtIEs}}

}

Cause-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

unknown-or-already-allocated-gnb-cu-cp-ue-e1ap-id,

unknown-or-already-allocated-gnb-cu-up-ue-e1ap-id,

unknown-or-inconsistent-pair-of-ue-e1ap-id,

interaction-with-other-procedure,

pPDCP-Count-wrap-around,

not-supported-QCI-value,

not-supported-5QI-value,

encryption-algorithms-not-supported,

integrity-protection-algorithms-not-supported,

uP-integrity-protection-not-possible,

uP-confidentiality-protection-not-possible,

multiple-PDU-Session-ID-Instances,

unknown-PDU-Session-ID,

multiple-QoS-Flow-ID-Instances,

unknown-QoS-Flow-ID,

multiple-DRB-ID-Instances,

unknown-DRB-ID,

invalid-QoS-combination,

procedure-cancelled,

normal-release,

no-radio-resources-available,

action-desirable-for-radio-reasons,

resources-not-available-for-the-slice,

pDCP-configuration-not-supported,

...,

ue-dl-max-IP-data-rate-reason,

uP-integrity-protection-failure,

release-due-to-pre-emption

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...

}

Cell-Group-Information	::= SEQUENCE (SIZE(1.. maxnoofCellGroups)) OF Cell-Group-Information-Item

Cell-Group-Information-Item	::=	SEQUENCE {

cell-Group-ID							Cell-Group-ID,

uL-Configuration						UL-Configuration		OPTIONAL,

dL-TX-Stop								DL-TX-Stop				OPTIONAL,

rAT-Type								RAT-Type				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Cell-Group-Information-Item-ExtIEs } }	OPTIONAL,

...

}

Cell-Group-Information-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Cell-Group-ID	::=		INTEGER (0..3, ...)

CipheringAlgorithm	::=	ENUMERATED {

nEA0,

c-128-NEA1,

c-128-NEA2,

c-128-NEA3,

...

}

CNSupport ::= ENUMERATED {

c-epc,

c-5gc,

both,

...

}

CommonNetworkInstance ::= OCTET STRING

ConfidentialityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

ConfidentialityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

CP-TNL-Information		::= 	CHOICE {

endpoint-IP-Address		TransportLayerAddress,

choice-extension		ProtocolIE-SingleContainer	{{CP-TNL-Information-ExtIEs}}

}

CP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

{ ID id-endpoint-IP-Address-and-Port	CRITICALITY reject	TYPE Endpoint-IP-address-and-port 	PRESENCE mandatory},

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

transactionID					TransactionID					OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxnoofErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request,

qoS-Flows-Forwarded-On-Fwd-Tunnels	QoS-Flow-Mapping-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

Data-Usage-per-PDU-Session-Report ::= SEQUENCE {

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

pDU-session-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-PDU-Session-Report-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-PDU-Session-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-QoS-Flow-List	::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF Data-Usage-per-QoS-Flow-Item

Data-Usage-per-QoS-Flow-Item ::= SEQUENCE {

qoS-Flow-Identifier		QoS-Flow-Identifier,

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

qoS-Flow-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-QoS-Flow-Item-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-QoS-Flow-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-Report-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF Data-Usage-Report-Item

Data-Usage-Report-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

rAT-Type					RAT-Type,

dRB-Usage-Report-List		DRB-Usage-Report-List,

iE-Extensions	ProtocolExtensionContainer { { Data-Usage-Report-ItemExtIEs } }	OPTIONAL,

...

}

Data-Usage-Report-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DefaultDRB	::=	ENUMERATED	{

true,

false,

...

}

DiscardTimer	::=	ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity}

DL-TX-Stop	::=	ENUMERATED	{

stop,

resume,

...

}

DRB-Activity	::= ENUMERATED {

active,

not-active,

...

}

DRB-Activity-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Activity-Item

DRB-Activity-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

dRB-Activity				DRB-Activity,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-EUTRAN

DRB-Confirm-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-NG-RAN

DRB-Confirm-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-EUTRAN

DRB-Failed-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-EUTRAN

DRB-Failed-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-NG-RAN

DRB-Failed-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-NG-RAN

DRB-Failed-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-EUTRAN

DRB-Failed-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-NG-RAN

DRB-Failed-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-ID	::=	INTEGER (1..32, ...)

DRB-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-EUTRAN

DRB-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-NG-RAN

DRB-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-UP-Transport-Parameters					UP-Parameters							OPTIONAL,

pDCP-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

flow-Setup-List								QoS-Flow-List							OPTIONAL,

flow-Failed-List							QoS-Flow-Failed-List					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Removed-Item	::=	SEQUENCE {

dRB-ID									DRB-ID,

dRB-Released-In-Session					ENUMERATED {released-in-session, not-released-in-session, ...}		OPTIONAL,

dRB-Accumulated-Session-Time			OCTET STRING (SIZE(5))													OPTIONAL,

qoS-Flow-Removed-List					SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Removed-Item		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Removed-Item-ExtIEs } }			OPTIONAL,

...

}

DRB-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-EUTRAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-EUTRAN

DRB-Required-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information							OPTIONAL,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-NG-RAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-NG-RAN

DRB-Required-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

flow-To-Remove							QoS-Flow-List								OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-EUTRAN

DRB-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

s1-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-EUTRAN

DRB-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-NG-RAN

DRB-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-NG-RAN

DRB-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Status-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

pDCP-DL-Count				PDCP-Count		OPTIONAL,

pDCP-UL-Count				PDCP-Count		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Status-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Status-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-EUTRAN

DRBs-Subject-To-Counter-Check-Item-EUTRAN	::=	SEQUENCE {

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions		ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-NG-RAN

DRBs-Subject-To-Counter-Check-Item-NG-RAN	::=	SEQUENCE {

pDU-Session-ID				PDU-Session-ID,

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions			ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-EUTRAN

DRB-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration						OPTIONAL,

eUTRAN-QoS								EUTRAN-QoS								OPTIONAL,

s1-UL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request					PDCP-SN-Status-Request						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

cell-Group-To-Add						Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify					Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove					Cell-Group-Information					OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-NG-RAN

DRB-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration						OPTIONAL,

pDCP-Configuration							PDCP-Configuration						OPTIONAL,

dRB-Data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request							PDCP-SN-Status-Request						OPTIONAL,

pdcp-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

cell-Group-To-Add							Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify						Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove						Cell-Group-Information					OPTIONAL,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List				OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject	EXTENSION QoS-Flow-List	PRESENCE optional}|

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-EUTRAN

DRB-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-EUTRAN

DRB-Required-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-NG-RAN

DRB-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-NG-RAN

DRB-Required-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-EUTRAN

DRB-To-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration,

eUTRAN-QoS								EUTRAN-QoS,

s1-UL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information					Cell-Group-Information,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

existing-Allocated-S1-DL-UP-TNL-Info	UP-TNL-Information						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-EUTRAN

DRB-To-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

pDCP-Configuration							PDCP-Configuration,

eUTRAN-QoS									EUTRAN-QoS,

s1-UL-UP-TNL-Information					UP-TNL-Information,

data-Forwarding-Information-Request			Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information						Cell-Group-Information,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

qos-flow-Information-To-Be-Setup					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-SN-Status-Information								PDCP-SN-Status-Information								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-To-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-NG-RAN

DRB-To-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer					OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-Usage-Report-List ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF DRB-Usage-Report-Item

DRB-Usage-Report-Item	::= SEQUENCE {

startTimeStamp					OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions					ProtocolExtensionContainer { { DRB-Usage-Report-Item-ExtIEs} } OPTIONAL,

...

}

DRB-Usage-Report-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Duplication-Activation	::=		ENUMERATED {

active,

inactive,

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel					QoSPriorityLevel,

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

fiveQI								INTEGER (0..255, ...)								OPTIONAL,

delayCritical						ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

averagingWindow 					AveragingWindow										OPTIONAL,

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	}|

# { ID id-CNPacketDelayBudgetDownlink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	}|

# { ID id-CNPacketDelayBudgetUplink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	},

...

}

DataDiscardRequired	::= 	ENUMERATED {

required,

...

}

#### -- E

EncryptionKey	::=	OCTET STRING

Endpoint-IP-address-and-port::= SEQUENCE {

endpoint-IP-Address			TransportLayerAddress,

portNumber					PortNumber,

iE-Extensions							ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL

}

Endpoint-IP-address-and-port-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {EUTRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

ExtendedPacketDelayBudget ::= INTEGER (1..65535, ...)

EUTRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofEUTRANQOSParameters)) OF EUTRAN-QoS-Support-Item

EUTRAN-QoS-Support-Item ::= SEQUENCE {

eUTRAN-QoS	EUTRAN-QoS,

iE-Extensions			ProtocolExtensionContainer { { EUTRAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

EUTRAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS	::= SEQUENCE {

qCI										QCI,

eUTRANallocationAndRetentionPriority	EUTRANAllocationAndRetentionPriority,

gbrQosInformation						GBR-QosInformation										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { EUTRAN-QoS-ExtIEs } }	OPTIONAL,

...

}

EUTRAN-QoS-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- F

#### -- G

GNB-CU-CP-Name 				::= PrintableString(SIZE(1..150,...))

GNB-CU-CP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-UP-Capacity				::= INTEGER (0..255)

GNB-CU-UP-CellGroupRelatedConfiguration	 ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF GNB-CU-UP-CellGroupRelatedConfiguration-Item

GNB-CU-UP-CellGroupRelatedConfiguration-Item ::= SEQUENCE {

cell-Group-ID				Cell-Group-ID,

uP-TNL-Information			UP-TNL-Information,

uL-Configuration			UL-Configuration		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs } }	OPTIONAL

}

GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-ID				::= INTEGER (0..68719476735)

GNB-CU-UP-Name 				::= PrintableString(SIZE(1..150,...))

GNB-CU-UP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-CP-TNLA-Setup-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

GNB-CU-CP-TNLA-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Add-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressgNBCUUP	CRITICALITY reject	EXTENSION CP-TNL-Information	PRESENCE optional},

...

}

GNB-CU-CP-TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage 	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Update-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress			CP-TNL-Information,

tNLAssociationTransportLayerAddressgNBCUCP	CP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddresses				TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }         OPTIONAL,

...

}

GTPTLA-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTPTunnel				::= SEQUENCE {

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { GTPTunnel-ExtIEs} } OPTIONAL,

...

}

GTPTunnel-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-OverloadInformation ::= ENUMERATED {overloaded, not-overloaded}

GNB-DU-ID	::= INTEGER (0..68719476735)

#### -- H

HFN		::=		INTEGER	(0..4294967295)

#### -- I

IntegrityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

IntegrityProtectionAlgorithm	::=	ENUMERATED {

nIA0,

i-128-NIA1,

i-128-NIA2,

i-128-NIA3,

...

}

IntegrityProtectionKey	::= OCTET STRING

IntegrityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

Inactivity-Timer	::=	INTEGER (1..7200, ...)

InterfacesToTrace ::= BIT STRING (SIZE(8))

#### -- J

#### -- K

#### -- L

#### -- M

MaxDataBurstVolume  ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MaxPacketLossRate ::= INTEGER (0..1000, ...)

MRDC-Data-Usage-Report-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp				OCTET STRING (SIZE(4)),

usageCountUL				INTEGER (0..18446744073709551615),

usageCountDL				INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { { MRDC-Data-Usage-Report-Item-ExtIEs} } OPTIONAL,

...

}

MRDC-Data-Usage-Report-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRDC-Usage-Information ::= SEQUENCE {

data-Usage-per-PDU-Session-Report		Data-Usage-per-PDU-Session-Report			OPTIONAL,

data-Usage-per-QoS-Flow-List			Data-Usage-per-QoS-Flow-List				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MRDC-Usage-Information-ExtIEs} } OPTIONAL,

...

}

MRDC-Usage-Information-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

NetworkInstance ::= INTEGER (1..256, ...)

New-UL-TNL-Information-Required::= 	ENUMERATED {

required,

...

}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofNGRANQOSParameters)) OF NG-RAN-QoS-Support-Item

NG-RAN-QoS-Support-Item ::= SEQUENCE {

non-Dynamic5QIDescriptor	Non-Dynamic5QIDescriptor,

iE-Extensions				ProtocolExtensionContainer { { NG-RAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

NG-RAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

Non-Dynamic5QIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			QoSPriorityLevel				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Non-Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Non-Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

# { ID id-CNPacketDelayBudgetDownlink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	}|

# { ID id-CNPacketDelayBudgetUplink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	},

...

}

NR-Cell-Identity	::=		BIT STRING (SIZE(36))

NR-CGI ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

nR-Cell-Identity		NR-Cell-Identity,

iE-Extensions			ProtocolExtensionContainer { { NR-CGI-ExtIEs } }	OPTIONAL

}

NR-CGI-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-CGI-Support-List ::= SEQUENCE (SIZE(1.. maxnoofNRCGI)) OF NR-CGI-Support-Item

NR-CGI-Support-Item ::= SEQUENCE {

nR-CGI	NR-CGI,

iE-Extensions				ProtocolExtensionContainer { { NR-CGI-Support-Item-ExtIEs } }	OPTIONAL

}

NR-CGI-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- O

OutOfOrderDelivery	::=	ENUMERATED	{

true,

...

}

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PDCP-Configuration	::=	SEQUENCE {

pDCP-SN-Size-UL							PDCP-SN-Size,

pDCP-SN-Size-DL							PDCP-SN-Size,

rLC-Mode								RLC-Mode,

rOHC-Parameters							ROHC-Parameters			OPTIONAL,

t-ReorderingTimer						T-ReorderingTimer		OPTIONAL,

discardTimer							DiscardTimer			OPTIONAL,

uLDataSplitThreshold					ULDataSplitThreshold	OPTIONAL,

pDCP-Duplication						PDCP-Duplication		OPTIONAL,

pDCP-Reestablishment					PDCP-Reestablishment	OPTIONAL,

pDCP-DataRecovery						PDCP-DataRecovery		OPTIONAL,

duplication-Activation				Duplication-Activation		OPTIONAL,

outOfOrderDelivery						OutOfOrderDelivery		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Configuration-ExtIEs } }	OPTIONAL,

...

}

PDCP-Configuration-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPduplicationInformation		CRITICALITY ignore	EXTENSION AdditionalPDCPduplicationInformation 	PRESENCE optional	},

...

}

PDCP-Count	::=	SEQUENCE {

pDCP-SN				PDCP-SN,

hFN					HFN,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Count-ExtIEs } }	OPTIONAL,

...

}

PDCP-Count-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN-Status-Request	::=		ENUMERATED {

requested,

...

}

PDCP-DataRecovery	::=	ENUMERATED	{

true,

...

}

PDCP-Duplication	::=	ENUMERATED	{

true,

...

}

PDCP-Reestablishment	::=	ENUMERATED	{

true,

...

}

PDU-Session-Resource-Data-Usage-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Data-Usage-Item

PDU-Session-Resource-Data-Usage-Item	::= SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

mRDC-Usage-Information						MRDC-Usage-Information,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Data-Usage-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Data-Usage-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN		::=		INTEGER	(0..262143)

PDCP-SN-Size	::=	ENUMERATED	{

s-12,

s-18,

...

}

PDCP-SN-Status-Information ::= SEQUENCE {

pdcpStatusTransfer-UL	DRBBStatusTransfer,

pdcpStatusTransfer-DL	PDCP-Count,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransfer-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBBStatusTransfer ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..131072))										OPTIONAL,

countValue				PDCP-Count,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-ID	::=	INTEGER (0..255)

PDU-Session-Resource-Activity	::= ENUMERATED {

active,

not-active,

...

}

PDU-Session-Resource-Activity-List ::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Activity-Item

PDU-Session-Resource-Activity-Item	::= SEQUENCE {

pDU-Session-ID									PDU-Session-ID,

pDU-Session-Resource-Activity					PDU-Session-Resource-Activity,

iE-Extensions	ProtocolExtensionContainer 		{ { PDU-Session-Resource-Activity-ItemExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Confirm-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Confirm-Modified-Item

PDU-Session-Resource-Confirm-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

dRB-Confirm-Modified-List-NG-RAN		DRB-Confirm-Modified-List-NG-RAN	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Confirm-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Confirm-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Item

PDU-Session-Resource-Failed-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Mod-Item

PDU-Session-Resource-Failed-Mod-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-To-Modify-Item

PDU-Session-Resource-Failed-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Modified-Item

PDU-Session-Resource-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

securityResult							SecurityResult							OPTIONAL,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN					OPTIONAL,

dRB-Failed-List-NG-RAN				DRB-Failed-List-NG-RAN				OPTIONAL,

dRB-Modified-List-NG-RAN				DRB-Modified-List-NG-RAN				OPTIONAL,

dRB-Failed-To-Modify-List-NG-RAN		DRB-Failed-To-Modify-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	},

...

}

PDU-Session-Resource-Required-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Required-To-Modify-Item

PDU-Session-Resource-Required-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

dRB-Required-To-Modify-List-NG-RAN		DRB-Required-To-Modify-List-NG-RAN		OPTIONAL,

dRB-Required-To-Remove-List-NG-RAN				DRB-Required-To-Remove-List-NG-RAN				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Required-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Required-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	},

...

}

PDU-Session-Resource-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Item

PDU-Session-Resource-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

securityResult							SecurityResult				OPTIONAL,

nG-DL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

nG-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN,

dRB-Failed-List-NG-RAN					DRB-Failed-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	},

...

}

PDU-Session-Resource-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Mod-Item

PDU-Session-Resource-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID											PDU-Session-ID,

securityResult											SecurityResult								OPTIONAL,

nG-DL-UP-TNL-Information								UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-Mod-List-NG-RAN								DRB-Setup-Mod-List-NG-RAN,

dRB-Failed-Mod-List-NG-RAN								DRB-Failed-Mod-List-NG-RAN					OPTIONAL,

iE-Extensions											ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	},

...

}

PDU-Session-Resource-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Modify-Item

PDU-Session-Resource-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

securityIndication									SecurityIndication						OPTIONAL,

pDU-Session-Resource-DL-AMBR						BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information						OPTIONAL,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Data-Forwarding-Information	Data-Forwarding-Information	OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

networkInstance										NetworkInstance							OPTIONAL,

dRB-To-Setup-List-NG-RAN							DRB-To-Setup-List-NG-RAN				OPTIONAL,

dRB-To-Modify-List-NG-RAN							DRB-To-Modify-List-NG-RAN				OPTIONAL,

dRB-To-Remove-List-NG-RAN					DRB-To-Remove-List-NG-RAN		OPTIONAL,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SNSSAI		CRITICALITY reject	EXTENSION SNSSAI		PRESENCE optional}|

{ ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

# {ID id-redundant-nG-UL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information			PRESENCE optional	}|

{ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	EXTENSION 	CommonNetworkInstance		PRESENCE optional	},

...

}

PDU-Session-Resource-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Remove-Item

PDU-Session-Resource-To-Remove-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Remove-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Remove-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Cause		CRITICALITY ignore	EXTENSION Cause		PRESENCE optional},

...

}

PDU-Session-Resource-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Item

PDU-Session-Resource-To-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

pDU-Session-Type						PDU-Session-Type,

sNSSAI									SNSSAI,

securityIndication						SecurityIndication,

pDU-Session-Resource-DL-AMBR			BitRate						OPTIONAL,

nG-UL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer			Inactivity-Timer	OPTIONAL,

existing-Allocated-NG-DL-UP-TNL-Info	UP-TNL-Information		OPTIONAL,

networkInstance							NetworkInstance		OPTIONAL,

dRB-To-Setup-List-NG-RAN				DRB-To-Setup-List-NG-RAN,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

# { ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

# { ID id-redundant-nG-UL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information					PRESENCE optional	}|

{ ID id-RedundantCommonNetworkInstance			CRITICALITY ignore	EXTENSION 	CommonNetworkInstance				PRESENCE optional	},

...

}

PDU-Session-Resource-To-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Mod-Item

PDU-Session-Resource-To-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

pDU-Session-Type									PDU-Session-Type,

sNSSAI												SNSSAI,

securityIndication									SecurityIndication,

pDU-Session-Resource-AMBR							BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

dRB-To-Setup-Mod-List-NG-RAN						DRB-To-Setup-Mod-List-NG-RAN,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-NetworkInstance		CRITICALITY ignore	EXTENSION NetworkInstance		PRESENCE optional}|

# {ID id-CommonNetworkInstance	CRITICALITY ignore	EXTENSION CommonNetworkInstance	PRESENCE optional}|

# {ID id-redundant-nG-UL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information		PRESENCE optional	}|

{ID id-RedundantCommonNetworkInstance			CRITICALITY ignore	EXTENSION 	CommonNetworkInstance	PRESENCE optional	},

...

}

PDU-Session-To-Notify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-To-Notify-Item

PDU-Session-To-Notify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

qoS-Flow-List							QoS-Flow-List,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-To-Notify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-To-Notify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Type ::= ENUMERATED {

ipv4,

ipv6,

ipv4v6,

ethernet,

unstructured,

...

}

PLMN-Identity ::= OCTET STRING (SIZE(3))

PortNumber ::= BIT STRING (SIZE(16))

PPI ::= INTEGER (0..7, ...)

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

#### -- Q

QCI ::= INTEGER (0..255)

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				Non-Dynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer	{{QoS-Characteristics-ExtIEs}}

}

QoS-Characteristics-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

QoS-Flow-Identifier	::=	INTEGER (0..63)

QoS-Flow-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Item

QoS-Flow-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoS-Flow-Mapping-Indication		PRESENCE optional},

...

}

QoS-Flow-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Failed-Item

QoS-Flow-Failed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Mapping-Item

QoS-Flow-Mapping-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowMappingIndication						QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Mapping-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Mapping-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-Indication ::= ENUMERATED {ul, dl, ...}

QoS-Parameters-Support-List	::= SEQUENCE {

eUTRAN-QoS-Support-List			EUTRAN-QoS-Support-List			OPTIONAL,

nG-RAN-QoS-Support-List			NG-RAN-QoS-Support-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { QoS-Parameters-Support-List-ItemExtIEs} } OPTIONAL,

...

}

QoS-Parameters-Support-List-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSPriorityLevel ::= INTEGER (0..127, ...)

QoS-Flow-QoS-Parameter-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-QoS-Parameter-Item

QoS-Flow-QoS-Parameter-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-QoS-Parameter-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-QoS-Parameter-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-RedundantQosFlowIndicator		CRITICALITY ignore 	EXTENSION RedundantQoSFlowIndicator			PRESENCE optional}|

# {ID id-TSCTrafficCharacteristics 		CRITICALITY ignore 	EXTENSION TSCTrafficCharacteristics 			PRESENCE optional},

...

}

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSMonitoringRequest	CRITICALITY ignore	EXTENSION QosMonitoringRequest	PRESENCE optional},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QoS-Flow-Removed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoS-Flow-Released-In-Session			ENUMERATED {released-in-session, not-released-in-session, ...}			OPTIONAL,

qoS-Flow-Accumulated-Session-Time		OCTET STRING (SIZE(5))														OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flow-Removed-Item-ExtIEs } }		OPTIONAL,

...

}

QoS-Flow-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- R

RANUEID ::= OCTET STRING (SIZE (8))

RAT-Type	::=	ENUMERATED	{

e-UTRA,

nR,

...

}

RedundantQoSFlowIndicator::= ENUMERATED {true,false}

RetainabilityMeasurementsInfo	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Removed-Item

RLC-Mode	::=	ENUMERATED	{

rlc-tm,

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

ROHC-Parameters	::= CHOICE {

rOHC					ROHC,

uPlinkOnlyROHC			UplinkOnlyROHC,

choice-Extension		ProtocolIE-SingleContainer { { ROHC-Parameters-ExtIEs} }

}

ROHC-Parameters-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ROHC	::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { ROHC-ExtIEs } } 		OPTIONAL

}

ROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- S

SecurityAlgorithm	::= SEQUENCE {

cipheringAlgorithm				CipheringAlgorithm,

integrityProtectionAlgorithm	IntegrityProtectionAlgorithm	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SecurityAlgorithm-ExtIEs } }	OPTIONAL,

...

}

SecurityAlgorithm-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			IntegrityProtectionIndication,

confidentialityProtectionIndication		ConfidentialityProtectionIndication,

maximumIPdatarate							MaximumIPdatarate							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SecurityIndication-ExtIEs} }	OPTIONAL,

...

}

SecurityIndication-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityInformation ::= SEQUENCE {

securityAlgorithm			SecurityAlgorithm,

uPSecuritykey				UPSecuritykey,

iE-Extensions				ProtocolExtensionContainer { { SecurityInformation-ExtIEs } }	OPTIONAL,

...

}

SecurityInformation-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityResult ::= SEQUENCE {

integrityProtectionResult			IntegrityProtectionResult,

confidentialityProtectionResult		ConfidentialityProtectionResult,

iE-Extensions						ProtocolExtensionContainer { {SecurityResult-ExtIEs} }	OPTIONAL,

...

}

SecurityResult-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Slice-Support-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF Slice-Support-Item

Slice-Support-Item ::= SEQUENCE {

sNSSAI	SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { Slice-Support-Item-ExtIEs } }	OPTIONAL

}

Slice-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI ::= SEQUENCE {

sST			OCTET STRING (SIZE(1)),

sD			OCTET STRING (SIZE(3)) 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-ExtIEs } }	OPTIONAL,

...

}

SNSSAI-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SDAP-Configuration ::= SEQUENCE {

defaultDRB				DefaultDRB,

sDAP-Header-UL			SDAP-Header-UL,

sDAP-Header-DL			SDAP-Header-DL,

iE-Extensions			ProtocolExtensionContainer { { SDAP-Configuration-ExtIEs } }	OPTIONAL,

...

}

SDAP-Configuration-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SDAP-Header-DL	::=	ENUMERATED	{

present,

absent,

...

}

SDAP-Header-UL	::=	ENUMERATED	{

present,

absent,

...

}

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

#### -- T

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TSCTrafficCharacteristics		::= SEQUENCE {

tSCTrafficCharacteristicsUL				TSCTrafficInformation			OPTIONAL,

tSCTrafficCharacteristicsDL				TSCTrafficInformation			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TSCTrafficCharacteristics-ExtIEs } }	OPTIONAL

}

TSCTrafficCharacteristics-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCTrafficInformation			::= SEQUENCE {

periodicity						Periodicity,

burstArrivalTime						BurstArrivalTime				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TSCTrafficInformation-ExtIEs } }	OPTIONAL

}

TSCTrafficInformation-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

Periodicity					::= INTEGER	(1..640000,	...)

BurstArrivalTime			::= OCTET STRING

TraceActivation ::= SEQUENCE {

traceID								TraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL,

...

}

TraceActivation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TraceID ::= OCTET STRING (SIZE(8))

TransportLayerAddress		::= 	BIT STRING (SIZE(1..160, ...))

TransactionID				::= INTEGER (0..255, ...)

T-Reordering	::=	ENUMERATED	{ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750, ms3000, ...}

T-ReorderingTimer ::= SEQUENCE {

t-Reordering				T-Reordering,

iE-Extensions				ProtocolExtensionContainer { { T-ReorderingTimer-ExtIEs } }	OPTIONAL,

...

}

T-ReorderingTimer-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Transport-Layer-Address-Info ::= SEQUENCE {

transport-UP-Layer-Addresses-Info-To-Add-List	Transport-UP-Layer-Addresses-Info-To-Add-List	OPTIONAL,

transport-UP-Layer-Addresses-Info-To-Remove-List	Transport-UP-Layer-Addresses-Info-To-Remove-List	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs} }	OPTIONAL,

...

}

Transport-Layer-Address-Info-ExtIEs	E1AP-PROTOCOL-EXTENSION	::=	{

...

}

Transport-UP-Layer-Addresses-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Add-Item

Transport-UP-Layer-Addresses-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Remove-Item

Transport-UP-Layer-Addresses-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- U

UE-Activity	::=	ENUMERATED	{

active,

not-active,

...

}

UE-associatedLogicalE1-ConnectionItem ::= SEQUENCE {

gNB-CU-CP-UE-E1AP-ID		GNB-CU-CP-UE-E1AP-ID	 OPTIONAL,

gNB-CU-UP-UE-E1AP-ID		GNB-CU-UP-UE-E1AP-ID	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-associatedLogicalE1-ConnectionItemExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalE1-ConnectionItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-Configuration	::=	ENUMERATED	{

no-data,

shared,

only,

...

}

ULDataSplitThreshold	::=	ENUMERATED {b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800, b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400, b6553600, infinity, ...}

UP-Parameters ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF UP-Parameters-Item

UP-Parameters-Item ::= SEQUENCE {

uP-TNL-Information			UP-TNL-Information,

cell-Group-ID				Cell-Group-ID,

iE-Extensions				ProtocolExtensionContainer { { UP-Parameters-Item-ExtIEs } }	OPTIONAL,

...

}

UP-Parameters-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

UPSecuritykey	::= SEQUENCE {

encryptionKey				EncryptionKey,

integrityProtectionKey		IntegrityProtectionKey		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UPSecuritykey-ExtIEs } }	OPTIONAL,

...

}

UPSecuritykey-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

UP-TNL-Information		::= 	CHOICE {

gTPTunnel		GTPTunnel,

choice-extension		ProtocolIE-SingleContainer	{{UP-TNL-Information-ExtIEs}}

}

UP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

UplinkOnlyROHC ::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UplinkOnlyROHC-ExtIEs } } 		OPTIONAL

}

UplinkOnlyROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- V

#### -- W

#### -- X

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

#### -- Common definitions

--

-- **************************************************************

E1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-CommonDataTypes (3)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= 	ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global			OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..maxProtocolExtensions)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info								ProtocolIE-ID ::= 86

id-QoSMonitoringRequest										ProtocolIE-ID ::= 87

id-RedundantCommonNetworkInstance								ProtocolIE-ID ::= xx

id-redundant-nG-UL-UP-TNL-Information							ProtocolIE-ID ::= xx

id-redundant-nG-DL-UP-TNL-Information							ProtocolIE-ID ::= xx

id-RedundantQosFlowIndicator									ProtocolIE-ID ::= xx

id-TSCTrafficCharacteristics									ProtocolIE-ID ::= xx

id-CNPacketDelayBudgetDownlink									ProtocolIE-ID ::= xx 
id-CNPacketDelayBudgetUplink									ProtocolIE-ID ::= xx

id-ExtendedPacketDelayBudget									ProtocolIE-ID ::= xx

id-AdditionalPDCPduplicationInformation						ProtocolIE-ID ::= xx

END

-- ASN1STOP

### 9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

#### -- Container definitions

--

-- **************************************************************

E1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

E1AP-PROTOCOL-IES ::= CLASS {

&id					ProtocolIE-ID			UNIQUE,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

E1AP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

E1AP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container { E1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer { E1AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field { E1AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				E1AP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		E1AP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			E1AP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, E1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer { E1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField { E1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					E1AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			E1AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		E1AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container { E1AP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field { E1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				E1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality		E1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value			E1AP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

End of Text Proposal for TS 38.463