3GPP TSG-RAN3 Meeting #107bis-e	R3-202353

E-Meeting, 20 – 30 April, 2020

Title: 	(TP for NPN BL CR for TS 38.401)  Support of RAN sharing with NPN: F1 aspects

Source: 	Huawei

Agenda item:	16.8

Document for:	Discussion

1. Introduction

In this paper, we discuss NPN in RAN sharing for disaggregated RAN cases.

2. Discussion

2.1 Initial Registration

As discussed in our companion paper [1], the gNB-DU may connect to an unsuitable gNB-CU who does not support the selected SNPN before the gNB-DU receives the RRC Setup Complete message, thereby the gNB-DU needs to send the F1AP UE CONTEXT RELEASE REQUEST message to the unsuitable gNB-CU with cause "SNPN not served by the CU".

Introduce a cause "NID not served by the CU" over F1. (Same as Proposal 3 in [1].)

2.2 RRC Connection Reestablishment

For RRC Resume Request or RRC reestablishment Request procedure, the CAG or NID should be considered as well.

First in DL RRC message transfer, the PLMN assistance information is provided to the DU for redirection. The NID should be included in the PLMN assistance information.

Second, the new gNB-CU sends Xn Retrieve UE Context Request message to the old gNB. After receiving Retrieve UE Context Setup Request message, the old gNB may decide whether to send the UE context to the new gNB-CU based on the stored UE context and the support CAG List of the new gNB-CU. It is possible that there is no overlap between the CAG List supported by the serving PLMN of the new cell identified by the New cell ID and the UE’s Allowed CAG List (stored in the old gNB) supported by the serving PLMN. In this case, the Xn Retrieve UE Context Failure is triggered, even though the old gNB stores the UE context.

The NID should be included in the DL RRC Message Transfer for PLMN redirection.

Xn Retrieve UE Context Failure is triggered by the old gNB when there is no overlap between the CAG List supported by the serving PLMN of the new cell identified by the New cell ID and the UE’s Allowed CAG List supported by the serving PLMN.

3. Conclusion

In this paper, we have discussed NPN in RAN sharing, and have the following proposals,

- Introduce a cause "NID not served by the CU" over F1. (Same as Proposal 3 in [1].)

- The NID should be included in the DL RRC Message Transfer for PLMN redirection.

- Xn Retrieve UE Context Failure is triggered by the old gNB when there is no overlap between the CAG List supported by the serving PLMN of the new cell identified by the New cell ID and the UE’s Allowed CAG List supported by the serving PLMN.

The corresponding TP for TS 38.401 is provided in the Annex, and the corresponding TP for TS 38.473 is provided in [2].

4. Reference

- R3-202348, (TP for NPN BL CR for TS 38.473): Further support of NPN over F1, Huawei, China Telecom

- R3-202354, (TP for NPN BL CR for TS 38.473): Support of RAN sharing with NPN, Huawei, China Telecom

Annex – TP for TS 38.401 (on the top of BL R3-201591)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

8.11	Support of Network Sharing with multiple cell-ID broadcast

8.11.1	General

This section describes necessary additions as compared to the case where network sharing with multiple cell-ID broadcast is not applied.

The signalling flows in the subsequent sections assuming 2 sharing operators, A and B. The F1-C signalling transport deployment used is indicated within the subsequent sections.

8.11.2	Initial Registration – separate PLMN signalling

The signalling flow for Initial Registration for network sharing with multiple cell-ID broadcast with separate per-PLMN signalling is shown in Figure 8.11.2-1.

In this example message flow

-	the UE is assumed to not provide an ue-Identity from which the DU is able to deduce the PLMN ID.

-	each F1-C interface instance uses a separate signalling transport or share signalling transport with other F1-C interface instances.

-	the gNB-DUA/B entity shown in Figure 8.11.2-1 is a simplified representation of the gNB-DUA of PLMN A, the gNB DUB of PLMN B and respective radio resources of the shared cell.

Figure 8.11.2-1: UE Initial Access procedure and network sharing with multiple cell-ID broadcast

NOTE:	Steps 1-5 are defined in clause 8.1. Note, that the selectedPLMN-Identity is provided in step 5.

6.	The gNB-DUA sends the F1AP UE CONTEXT RELEASE REQUEST message to the gNB-CUA including a Cause set to "PLMN not served by the CU" or "NID not served by the CU".

7.	The gNB-DUB sends the F1AP INITIAL UL RRC MESSAGE to the gNB-CUB. including the NR CGI associated with PLMNB, the C-RNTI indicated by the gNB-DUA at step 2, and the RRC-Container IE and the RRC-Container-RRCSetupComplete IE with the RRC message received in step1 and step 5 respectively. The RRC-Container-RRCSetupComplete IE are included in the INITIAL UL RRC MESSAGE TRANSFER for the case of network sharing and shall contain the RRC messages received via the RRC UL-DCCH-Message IE from the UE, but never previously sent to the gNB-CUB.

8. The gNB-CUA triggers the F1AP UE Context Release procedure.

NOTE: Initiating procedures from gNB-DUA towards gNB-CUA and from gNB-DUB to gNB-CUB in parallel is not precluded.

8.11.3	RRC Connection Reestablishment – separate PLMN signalling

The signalling flow for RRC Connection Reestablishment for network sharing with multiple cell-ID broadcast with separate per-PLMN signalling is shown in Figure 8.11.3-1.

In this example message flow

-	each F1-C/Xn-C interface instance uses either a separate signalling transport or a share signalling transport with other interface instances.

-	the New gNB-DUA/B entity shown in Figure 8.11.3-1 is a simplified representation of the New gNB-DUA of PLMN A, the New gNB DUB of PLMN B and respective radio resources of the shared cell.

Figure 8.11.3-1: RRC Connection Reestablishment and network sharing with multiple cell-ID broadcast

1.	The UE sends the RRCReestablishmentRequest.

2A-5A.	Depicts the case where the UE context could not be retrieved by the new gNB-CUA. In step 2A, the NR CGI associated to PLMNA is indicated. In step 5A, the gNB-CUA would prepare the possibility to revert back to normal RRC Connection Establishment, indicating that the UE Context was not retrieveable and may include the re-directed RRC message as received in step 1. After step 5A, the gNB-DUA may redirect the UE towards the PLMN indicated in DL RRC message transfer, if the PLMN assistance information is provided by the gNB-CUA; or the gNB-DUA may redirect the UE towards the SNPN indicated in DL RRC message transfer, if the SNPN assistance information are provided by the gNB-CUA. If the New gNB-DUA/B was not able to deduce the RRC message from step 1, this indicator triggers step 2B. The New gNB-DUA is supposed to trigger the release the UE-associated signalling connection (not shown).

2B-5B.	Depicts the case where the UE context was retrieveable by the New gNB-CUB. In step 2B, the NR CGI associated to PLMNB is indicated. Step 2B also includes the C-RNTI allocated at reception of step 1.

6-8.	The RRC Connection Reestablishment continues with the New gNB-CUB.

NOTE:	If all gNB-CUs indicate that the UE context is not retrievable, the RRC connection reestablishment falls back to RRC Connection setup, as described in section 8.11.2.

NOTE: Initiating procedures from gNB-DUA towards gNB-CUA and from gNB-DUB to gNB-CUB in parallel is not precluded.

NOTE: Step 4A may be triggered if there is no overlap between the CAG List supported by the serving PLMN of the new cell identified by the New cell ID and the UE’s Allowed CAG List supported by the serving PLMN.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>