3GPP TSG-RAN WG3 Meeting #108-e	R3-204500

Online, 1st - 10th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | 0068 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR to 38.470: Addition of MDT feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE, Nokia, Nokia Shanghai Bell，CATT, Samsung, Huawei, Ericsson, Qualcomm Incorporated, LG Electronics, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2020-06-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of MDT function |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are necessary: - Add cell traffic trace procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDT features not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 0492 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Add more co-source companies |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////Change Start ///////////////////////////////////////////////////////////////////////////

### 5.2.5	Paging function

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters provided.

The gNB-CU provides paging information to enable the gNB-DU to calculate the exact PO and PF. The gNB-CU determines the PA. The gNB-DU consolidates all the paging records for a particular PO, PF and PA, and encodes the final RRC message and broadcasts the paging message on the respective PO, PF in the PA.

### 5.2.6	Warning messages information transfer function

This function allows to cooperate with the warning message transmission procedures over NG interface. The gNB-CU is responsible for encoding the warning related SI message and sending it together with other warning related information for the gNB-DU to broadcast over the radio interface.

### 5.2.7	Remote Interference Management (RIM) message transfer function

This function enables the transfer of Remote Interference Management (RIM) backhaul messages between the gNB-CU and the gNB-DU. RIM messages are transferred over F1-C. The gNB-CU acts as a coordinator on behalf of its affiliated gNB-DUs, by merging the RIM information received from its gNB-DUs and forwarding the merged information to the target gNBs or gNB-CUs, transparently via the core network. Similarly, a gNB-CU distributes an incoming RIM backhaul message to all its concerned gNB-DUs.

### 5.2.8	Trace function

The Trace function provides means to control trace sessions for a UE over F1 interface.

## 5.3	F1-U functions

### 5.3.1	Transfer of user data

This function allows to transfer of user data between gNB-CU and gNB-DU.

### 5.3.2	Flow control function

This function allows to control the downlink user data flow to the gNB-DU. The detailed protocol is specified in TS 38.425 [7].

## 5.4	TEIDs allocation

The gNB-DU is responsible for the allocation of the F1-U DL GTP TEID for each data radio bearer.

# 6	Procedures of the F1 interface

## 6.1	Control plane procedures

### 6.1.1	Interface Management procedures

The F1 Interface management procedures are listed below:

-	Reset procedure

-	Error Indication procedure

-	F1 Setup procedure

-	gNB-DU Configuration Update procedure

-	gNB-CU Configuration Update procedure

-	gNB-DU Resource Coordination procedure

-	gNB-DU Status Indication procedure

-	F1 Removal procedure

-	Network Access Rate Reduction procedure

### 6.1.2	Context Management procedures

The F1 Context management procedures are listed below:

-	UE Context setup procedure

-	UE Context Release Request (gNB-DU initiated) procedure

-	UE Context Release (gNB-CU initiated) procedure

-	UE Context Modification (gNB-CU initiated) procedure

-	UE Context Modification Required (gNB-DU initiated) procedure

-	UE Inactivity Notification procedure

-	Notify procedure

### 6.1.3	RRC Message Transfer procedures

The F1 RRC message transfer procedures are listed below:

-	Initial UL RRC Message Transfer procedure

-	UL RRC Message Transfer procedure

-	DL RRC Message Transfer procedure

-	RRC Delivery Report procedure

### 6.1.3A	Warning Message Transmission procedures

The F1 Warning message transmission procedures are listed below:

-	Write-Replace Warning procedure

-	PWS Cancel procedure

-	PWS Restart Indication procedure

-	PWS Failure Indication procedure

### 6.1.4	System Information procedures

The F1 System information procedures are listed below:

-	System Information Delivery procedure

### 6.1.5	Paging procedures

The F1 Paging procedures are listed below:

-	Paging procedure

### 6.1.6	Void

### 6.1.7	Radio information transfer procedures

The F1 Radio information transfer procedures are listed below:

-	DU-CU Radio Information Transfer procedure

- 	CU-DU Radio Information Transfer procedure

### 6.1.8	UE Tracing procedures

The following procedures are used to trace the UE:

- Trace Start procedure

- Deactivate Trace procedure

- Cell Traffic Trace procedure

////////////////////////////////////////////////////////////////////////Change End ///////////////////////////////////////////////////////////////////////////