3GPP TSG-RAN WG3 Meeting #108-e	R3-20xxxx

Online, 1st - 10th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.460 | CR | 0038 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR to 38.460: Addition of MDT feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE, Nokia, Nokia Shanghai Bell，CATT, Samsung, Huawei, Ericsson, Qualcomm Incorporated, LG Electronics, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2020-06-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of MDT function |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are necessary: - Add cell traffic trace procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDT features not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.463 CR 0477 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Add more co-source companies |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////Change Start ///////////////////////////////////////////////////////////////////////////

### 5.1.2	E1 bearer context management function

The establishment of the E1 bearer context is initiated by the gNB-CU-CP and accepted or rejected by the gNB-CU-UP based on admission control criteria (e.g., resource not available).

The modification of the E1 bearer context can be initiated by either gNB-CU-CP or gNB-CU-UP. The receiving node can accept or reject the modification. The E1 bearer context management function also supports the release of the bearer context previously established in the gNB-CU-UP. The release of the bearer context is triggered by the gNB-CU-CP either directly or following a request received from the gNB-CU-UP.

This function is used to setup and modify the QoS-flow to DRB mapping configuration. The gNB-CU-CP decides flow-to-DRB mapping and provides the generated SDAP and PDCP configuration to the gNB-CU-UP. The gNB-CU-CP also decides the Reflective QoS flow to DRB mapping. For each PDU Session Resource to be setup or modified, the S-NSSAI, shall be provided in the E1 bearer context setup procedure and may be provided in the E1 bearer context modification procedure by gNB-CU-CP to the gNB-CU-UP.

This function is used for the gNB-CU-CP to send the security information to the gNB-CU-UP.

This function is used for the gNB-CU-UP to notify the event of DL data arrival detection to the gNB-CU-CP. With this function, the gNB-CU-UP requests gNB-CU-CP to trigger paging procedure over F1 or Xn to support RRC Inactive state.

This function is used for the gNB-CU-UP to notify the gNB-CU-CP that an UL packet including a QFI value in the SDAP header not configured by the Flow Mapping Information IE is received for the first time at the default DRB. The gNB-CU-CP can take further action if needed.

This function is used for the gNB-CU-UP to notify the event of user inactivity to the gNB-CU-CP. With this function, the gNB-CU-UP indicates that the inactivity timer associated with a bearer, a PDU session or a UE expires, or that user data is received for the bearer, the PDU session or the UE whose inactivity timer has expired. The gNB-CU-CP consolidates all the serving gNB-CU-UPs for the UE and takes further action.

This function is used for the gNB-CU-UP to report data volume to the gNB-CU-CP.

This function is used for the gNB-CU-CP to notify the suspension and resumption of bearer contexts to the gNB-CU-UP.

This function also allows to support CA based packet duplication as described in TS 38.300 [6], i.e. one data radio bearer should be configured with two GTP-U tunnels between gNB-CU-UP and a gNB-DU.

### 5.1.3	Trace function

The Trace function provides means to control trace sessions for a UE over E1 interface.

## 5.2	TEIDs allocation

The gNB-CU-UP is responsible for the allocation of the F1-U UL GTP TEID for each data radio bearer.

The gNB-CU-UP is responsible for the allocation of the S1-U DL GTP TEID for each E-RAB and the NG-U DL GTP TEID for each PDU Session.

The gNB-CU-UP is responsible for the allocation of the X2-U DL/UL GTP TEID or the Xn-U DL/UL GTP TEID for each data radio bearer.

# 6	Procedures of the E1 interface

## 6.1	Interface Management procedures

The E1 interface management procedures are listed below:

-	Reset procedure

-	Error Indication procedure

-	gNB-CU-UP E1 Setup procedure

-	gNB-CU-CP E1 Setup procedure

-	gNB-CU-UP Configuration Update procedure

-	gNB-CU-CP Configuration Update procedure

-	E1 Release procedure

-	gNB-CU-UP Status Indication procedure

## 6.2	Bearer Context Management procedures

The E1 bearer management procedures are listed below:

-	Bearer Context Setup procedure

-	Bearer Context Release Request (gNB-CU-UP initiated) procedure

-	Bearer Context Release (gNB-CU-CP initiated) procedure

-	Bearer Context Modification (gNB-CU-CP initiated) procedure

-	Bearer Context Modification Required (gNB-CU-UP initiated) procedure

-	DL Data Notification procedure

-	Bearer Context Inactivity Notification procedure

-	Data Usage Report procedure

-	MR-DC Data Usage Report procedure

## 6.3	UE Tracing procedures

The following procedures are used to trace the UE:

- Trace Start procedure

- Deactivate Trace procedure

- Cell Traffic Trace procedure

////////////////////////////////////////////////////////////////////////Change End ///////////////////////////////////////////////////////////////////////////