3GPP TSG-RAN WG3 #109	R3-204654

# Online, 17th - 28th August 2020

Agenda item:	31.2.1

Source:	NTT DOCOMO, INC., Ericsson

Title:	Support beam level inter-RAT measurement report in E-CID measurement in LPPa

Document for:	Discussion and Decision

1	Introduction

In the last RAN3 meeting, we agreed that both cell-level and beam-level measurements for RSRP and RSRQ are supported in NR E-CID over NRPPa as follows:

-----------------------------------------------------Excerpt from [1] ---------------------------------------------

According to RAN3 agreements, the following information may be transferred from gNB to LMF:

-	SS Reference Signal Received Power (SS-RSRP)

-	SS Reference Signal Received Quality (SS-RSRQ)

-	CSI Reference Signal Received Power (CSI-RSRP)

-	CSI Reference Signal Received Quality (CSI-RSRQ)

-	NR Cell Global Identifier / Physical Cell ID

-	Cell Portion ID

Both cell-level and beam-level measurements for RSRP and RSRQ are supported.

---------------------------------------------------------------------------------------------------------------------

In this paper we further discuss the necessity of supporting beam level inter-RAT measurement in E-CID over LPPa.

2	Discussion

2.1 Inter-RAT measurement report for E-CID in LPPa

As we could observe from the following excerption that in current LPPa spec [2], inter-RAT measurement report already includes NR measurement (i.e. NR ARFCN, NR PCI, NR SS-RSRP, NR SS-RSRQ). Regarding the SS-RSRP, SS-RSRQ measurement, it is not clear whether beam level measurement is supported. Similar as NR E-CID measurement over NRPPa, we understand SS-RSRP/RSRQ is per resource measurement as defined in TS 38.215, so both cell-level and beam-level should be supported. In addition, as an operator, we see big benefits of supporting NR beam level measurement reporting for EN-DC deployment, which could help enhance the accuracy of legacy E-CID positioning with relatively low extra development cost.

Observation1: In current LPPa spec, for inter-RAT measurement report, it is not clear whether NR beam level measurement is supported.

Proposal1: Support NR beam level measurement report in inter-RAT measurement report in E-CID measurement over LPPa.

------------------------------------------------------ Excerpt from [2] clause 9.1.1.2----------------------------------------------------

9.1.1.2	 E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by eNB to indicate that the requested E-CID measurement is successfully initiated.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| eNB UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Inter-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.15 |  | YES | ignore |

------------------------------------------------------------End of Excerption--------------------------------------------------------------

-----------------------------------------------------Excerpt from [2] clause 9.2.13-------------------------------------------------------

9.2.13	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. |
| >>Result NR | M | 1..<maxNRMeas> |  |  |
| >>>NR ARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>NR SS-RSRP | O |  | INTEGER (0..127) |  |
| >>>NR SS-RSRQ | O |  | INTEGER (0..127) |  |

----------------------------------------------------------End of Excerption------------------------------------------------------------

2.2 Contents of NR beam level measurement in LPPa

In current LTE RRC spec [3], as we could observe from the following excerption, for inter-RAT measurement reporting, UE could report NR SSB measurement result, while CSI-RS measurement result is not included. Therefore, for contents of beam level measurement, SSB-Index, SSB-RSRP, SSB-RSRQ could be included in inter-RAT measurement in E-CID measurement over LPPa.

---------------------------------------------------Except from [3] clause 6.3.5-----------------------------------------------------------

MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency and inter- RAT mobility.

----unrelated parts are omitted-----

MeasResultCellNR-r15 ::=			SEQUENCE {

pci-r15								PhysCellIdNR-r15,

measResultCell-r15					MeasResultNR-r15,

measResultRS-IndexList-r15			MeasResultSSB-IndexList-r15				OPTIONAL,

...,

[[	cgi-Info-r15						CGI-InfoNR-r15				OPTIONAL

]]

}

MeasResultNR-r15 ::=				SEQUENCE {

rsrpResult-r15						RSRP-RangeNR-r15						OPTIONAL,

rsrqResult-r15						RSRQ-RangeNR-r15						OPTIONAL,

rs-sinr-Result-r15					RS-SINR-RangeNR-r15						OPTIONAL,

...

}

MeasResultSSB-IndexList-r15::=		SEQUENCE (SIZE (1..maxRS-IndexReport-r15)) OF MeasResultSSB-Index-r15

MeasResultSSB-Index-r15 ::=		SEQUENCE {

ssb-Index-r15						RS-IndexNR-r15,

measResultSSB-Index-r15				MeasResultNR-r15					OPTIONAL,

...

}

----unrelated parts are omitted-----

----------------------------------------------------------End of Excerption------------------------------------------------------------

Observation2: In current LTE RRC Spec, UE could report inter-RAT measurement of NR SSB measurement results (i.e. SSB-Index, SSB-RSRP, SSB-RSRQ).

Proposal2: Include SSB-IndexList, SSB-Index, SS-RSRP beam value, SS-RSRQ beam value in inter-RAT measurement result in E-CID measurement over LPPa.

A CR to TS 36.455 for agreement is proposed in [4]

3	Conclusion

This contribution discussed the necessity of introducing beam level measurement in inter-RAT measurement in E-CID measurement over LPPa.

Following observations and proposals were obtained.

Observation1: In current LPPa spec, for inter-RAT measurement report, it is not clear whether NR beam level measurement is supported.

Observation2: In current LTE RRC Spec, UE could report inter-RAT measurement of NR SSB measurement results (i.e. SSB-Index, SSB-RSRP, SSB-RSRQ).

Proposal1: Support NR beam level measurement report in inter-RAT measurement report in E-CID measurement over LPPa.

Proposal2: Include SSB-Index, SSB-RSRP, SSB-RSRQ in inter-RAT measurement result in E-CID measurement over LPPa.

A CR to TS 36.455 for agreement is proposed in [4]

References

- R3-202646, LS on support for UL NR E-CID

- TS 36.455

- TS 36.331

- R3-204656, Introducing NR beam level measurement in inter-RAT measurement in E-CID measurement over LPPa.