3GPP TSG-RAN3 Meeting #109-e	R3-205075

E-meeting, 17 - 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0443 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Failure case of user location report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Location Reporting Request Type IE is included in the INITIAL CONTEXT SETUP REQUEST message and HANDOVER REQUEST message. If the location report request type contains multiple areas of interest set with the same Location Reporting Reference ID, or some other reasons, the NG-RAN can not perform the location report, and need to inform the AMF. However it is not clear which option the NG-RAN shall perform: - Option1: fail the initial UE context setup or handover procedure with failure response. - Option 2: succeed the initial UE context setup or handover procedure. Then the NG-RAN sends a LOCATION REPORTING FAILURE INDICATION message to the AMF with a cause value. - Option 3: introduce a new indicator in the successful response message to inform AMF (new IE is needed), but not fail the procedures. But currently the Location Reporting Failure Indication is used on in case of the Location Reporting Control Procedure as follows. - The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the Location Reporting Control procedure has failed. The procedure uses UE-associated signalling. Hence it is proposed to update the purpose of the Location Reporting Failure Indication procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the purpose of the Location Reporting Failure Indication procedure to allow the NG-RAN to report in case that the requested location reporting action has failed, so as to cover all location reporting failure cases Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it corrects the NG-RAN behaviour when it is not able to perform the location report signalled in the INITIAL CONTEXT SETUP REQUEST message and HANDOVER REQUEST message. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous for the NG-RAN operation when the NG-RAN can not perform the location report signalled in the INITIAL CONTEXT SETUP REQUEST message and HANDOVER REQUEST message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CRxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change begins

8.12.2	Location Reporting Failure Indication

8.12.2.1	General

The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the requested location reporting action has failed. The procedure uses UE-associated signalling.

8.12.2.2	Successful Operation

Figure 8.12.2.2-1: Location reporting failure indication

The NG-RAN node initiates the procedure by sending a LOCATION REPORTING FAILURE INDICATION message to the AMF. Upon reception of the LOCATION REPORTING FAILURE INDICATION message the AMF shall, based on the failure reason indicated by the Cause IE, take appropriate action.

8.12.2.3	Abnormal Conditions

Void.

Change ends