3GPP TSG-RAN WG3 #111-e																		  R3-210181

25 January – 4 February 2021

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | Draft | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 37.340 for support of SCG activation/deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following agreement has been achieved in RAN2 and RAN3. - MN can initiate SCG (de)activation during SN addition procedure, SN can decide whether to accept or reject SCG (de)activation request after receiving SN addition request message, FFS on how to reject it. - MN initiated SN modification procedure can be used for support of SCG (de)activation, and SN can decide whether to accept or reject SCG (de)activation request after receiving SN modification request message. - Activity Notification message sent from SN to MN, can be used for the MN to make final decision on SCG (de)activation. It is FFS whether no spec impacts or the Activity Notification message shall be enhanced, e.g., add a new SCG (de)activation suggestion IE. - SCG activation state (activated/deactivated) can be configured at PSCell addition/change, RRC resume or HO (RAN2). . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce MN initiated SCG activation/deactivation procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCG activation/deactivation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------Start of the change---------------------------------------------------

10.xx Support of SCG activation and deactivation

10.xx.1 MN initiated SCG activation and deactivation

The following figures are used for support of MN initiated SCG (de)activation.

Figure 10.xx.1-1: MN initiated SCG (de)activation through SN addition

1.	The MN decides to configure SCG activation state, at PSCell addition/change, RRC resume or HO scenarios.

2. The MN triggers the MN initiated SN Addition procedure, requesting the SN to (de-) activate SCG.

3. The RRC (Connection) Reconfiguration procedure commences

4. The following steps are the same as those in SN addition procedure in Section 10.2.

Figure 10.xx.1-2: MN initiated SCG (de)activation through SN modification

1.	The SN notifies the MN about user data inactivity for SN terminated bearers (or about SCG activity suggestion).

2.	The MN decides to keep the UE in RRC_CONNECTED and request SCG deactivation.

3/4.	The MN triggers the MN initiated SN Modification procedure, and request the SN to de-activate SCG.

5/6.	The UE is kept in RRC_CONNECTED and de-activates SCG (FFS: depending on RAN2).

7/8.	After a period of SCG deactivation, upon activity notification from the SN, the MN decides to request SCG re-activation.

9/10.	The MN initiates the MN initiated SN modification procedure to re-activate SCG..

11.	The RRC (Connection) Reconfiguration procedure commences. If the SCG configuration is to be updated, the new configuration is provided in the RRC(Connection)Reconfiguration message.

12. The following steps are the same as those in SN modification procedure in Section 10.3.

------------------------------------------------------End of the change---------------------------------------------------