3GPP TSG-RAN WG3 Meeting #111-e	R3-210495

25 January – 4 February 2021

Agenda item:	20.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Feeder Link Switchover

Document for:	Discussion & decision

# 1	Introduction

Last RAN3 meeting agreed

NTN encompasses NTN-GW(s) deployed on ground, NTN payload on board space/airborne vehicle(s) and functions to control the vehicles as well as the radio resources of the NTN payload(s) are out of 3GPP scope.

The feeder link switch-over is controlled by NTN control functions which are out of 3GPP scope.

It is assumed that the gNB can be informed about the scheduling of switch over events and usable radio resources and possibly the update of neighbouring gNBs

The execution of feeder link switch over may involve procedures over Xn and/or NG interfaces

This contribution further analyses the technical detail.

# 2	Discussion

## 2.1.	Soft switch and Hard switch

RAN2 is studying the Feeder link switch, including the soft feeder link switch and hard link switch. The related analysis can also be found in ([1])

- Soft switch

Figure 8.7.1.1-2 in TR 38.821 shows one solution that Feeder link switch over for LEO transparent satellite with two feeder links serving the satellite during the switch. At time T1.5, the satellite is connected with 2 two GWs and may potentially connected with 2 gNBs. The cell signalling from both gNBs are transmitted by the satellite.

Figure 8.7.1.1-2: Feeder link switch over for LEO transparent satellite with two feeder links serving the satellite during the switch

It is up to the implementation of the satellite to transfer the cell signalling from both gNBs over the air. During this period, gNB1 can initiate the handover procedure to “move” the UEs to gNB2. From RAN3 perspective, current NG/Xn based handover can be reused.

- Hard switch

Figure 8.7.1.1-3 in TR38.821 shows another solution Feeder link switch over for LEO transparent satellite with one feeder links serving the satellite during the switch. At time T1, the satellite stops to transfer the signalling from the serving GW1 and gNB1. At time T2, the satellite starts to transfer the signalling from the target GW2 and gNB2.

Figure 8.7.1.1-3: Feeder link switch over for LEO transparent satellite with one feeder links serving the satellite during the switch

## 2.2.	Impact to RAN3

- The distribution of cell signalling to a NTN-GW and Satellite.

In both soft switch and hard switch for Earth Moving Cell or Earth Fixed Cell, it is required that the signal of a specific cell is sent to a specific NTN GW then to a related satellite at the right time. It is either the gNB to be preconfigured with the information, e.g. transmit the signal of cell#1 to a specific transport network address (e.g. an IP address, or other transport network identifier which can identity the NTN GW and/or the satellite), or this distribution of cell signalling may also be performed by the NTN GW according to the preconfigured information. It is also possible that the gNB or NTN-GW may receive the signalling from a node controlling the satellite, e.g. NTN Control Center as discussed in ([3]).  Considering 1) the NTN-GW is a transport network node; 2) last RAN3 meeting agreed “The feeder link switch over is a Transport Network Layer procedure.” and 3) 3GPP network architecture does not consider the NTN Control Center, the distribution of cell signalling to NTN-GW and satellite can be considered as implementation detail, and no need specification work in RAN3.  There is no impact to NGAP/XnAP layer when the feeder link switch over is performed.

- Handover procedure for the UE

In both soft switch and hard switch that the UE’s serving gNB is changed, the handover procedure needs to be initiated by the serving gNB. As discussed in our RAN2 contribution ([4]), the source gNB can initiate the hand-over for the CONNECTED UEs based on radio measurements and/or feeder link switch timing information.

In last meeting, some proposals ([5][6][7]) were proposed to enhance the Xn procedure for Feeder Link Switchover, but it is unclear why the enhancements are needed and the issues to be addressed. For example, it was proposed to introduce a new Xn Message to trigger establishment of the second link for soft feeder link switch. Depend on the radio condition, the satellite may connect with a new NTN GW early or later. The satellite has the best knowledge to know when it needs to establish the connection with a new NTN GW. It is unclear why the gNB need to know when it needs to connect a new satellite. It may be argued that this is event-driven, but before the satellite can have a connection with a NTN GW, it is not possible to ask the target gNB to setup the connection with a satellite earlier, especially the satellite connects with the NTN GW, rather directly connect with the gNB.

In our understanding, current Xn/NG based HO procedure can be reused. There is no need to introduce new Xn/NG procedure for NTN.

- Feeder link switch based on accurate time control or conditional RRC re-establishment.

As described in TR38.821, two potential options were proposed, e.g.

Solution 1: Feeder link hard switch procedure is based on accurate time control

Solution 2: Feeder link hard switch procedure is based on conditional RRC re-establishment

In Solution 1, the HO Command message may need to be enhanced, but it is part of RRC that is RAN2 scope.

In Solution 2, it may be required for network to provide assistance information (e.g. next cell identity and/or reestablishment conditions) to trigger UE RRC reestablishment instead. TR38.821 also describes the assistance information can be sent to UE via SIB instead of dedicated signalling respectively, as a result, the signalling overhead caused by the large number of UEs can be effectively reduced. It is up to RAN2 to decide which solution is adopted. Even in case Solution 2 is agreed, it is RAN2 to decide how to provide the assistance information. In case a new SIB is introduced, it may require small update to F1 interface.

In a summary, there may be no impact to RAN3 specification to support Feeder Link Switch. Up to RAN2 discussion on the trigger on feeder link switch, it may require small update to F1 specification. RAN3 can wait for RAN2 discussion.

Observation 1: There is no impact to RAN3 XnAP specification and NGAP specification to support feeder link switch.

Observation 2: In case RAN2 agreed Solution 2 “Feeder link hard switch procedure is based on conditional RRC re-establishment”, small update may be needed to F1AP specification.

Proposal 1: To Support feeder link switch, RAN3 agrees current NG/Xn based HO procedure can be reused, and no need to introduce enhancement to XnAP specification and NGAP specification.

The BL Stage-2 CR need to be updated, for example, “It is up to the gNB’s implementation or the configuration to the gNB on when transferring established connection for the affected UEs between two gNBs during the feeder link switchover.”

Proposal 2: To Support feeder link switch, RAN3 agrees to wait for RAN2 decision regarding the impact to F1AP specification.

# 3	Conclusions

In this contribution, we briefly analyzed the impact to RAN3 to support feeder link switch. Our proposals are:

Observation 1: There is no impact to RAN3 XnAP specification and NGAP specification to support feeder link switch.

Observation 2: In case RAN2 agreed Solution 2 “Feeder link hard switch procedure is based on conditional RRC re-establishment”, small update may be needed to F1AP specification.

Proposal 1: To Support feeder link switch, RAN3 agrees current NG/Xn based HO procedure can be reused, and no need to introduce enhancement to XnAP specification and NGAP specification.

The update to the BL Stage-2 CR can be found in the annex section.

Proposal 2: To Support feeder link switch, RAN3 agrees to wait for RAN2 decision regarding the impact to F1AP specification.

# References

- R2-2009773, On Feeder Link Mobility in Transparent Satellite Payload Scenarios (Nokia, Nokia Shanghai Bell)

- TR 38.821

- R3-205173, On NTN Feeder link switch over

- R2-21xxxxx, On Feeder Link Mobility in Transparent Satellite Payload Scenarios (Nokia, Nokia Shanghai Bell)

- R3-205964, On Feeder link Switch (InterDigital)

- R3-206278 Discussion on feederlink switch (CATT)

- R3-206404 Feeder Link Switchover Support (Ericsson LM)

Text proposal to Stage-2 BL CR

### 4.x.5	Feeder link switch over

#### 4.x.5.1 Definitions

A feeder link switch over is the procedure where the feeder link is changed from a source NTN gateway to a target NTN gateway. The feeder link switch over is a Transport Network Layer procedure.

#### 4.x.5.2 Assumptions

A feeder link switch may result in transferring established connection for the affected UEs between two gNBs.

For soft feeder link switch over, a space-borne vehicle is able to connect to more than one NTN-GW during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, a space-borne vehicle only connect to one NTN-GW at any given time i.e. a radio link interruption may occur during the transition between the feeder links.

Editor’s Note: Some clarification on example of the temporary overlap and the interruption time may be provided later

#### 4.x.5.3 Operations [FFS]

#### 4.x.5.4 Procedures

It is up to the gNB’s implementation or the configuration to the gNB on when transferring established connection for the affected UEs between two gNBs during the feeder link switchover. The Xn based handover procedure or N2 based handover procedure can be reused to transfer the established connection for the affected UEs between two gNBs during the feeder link switchover.