3GPP TSG-RAN WG3 Meeting #111-e	R3-210663

E-meeting, 25 January – 4 February, 2021

Agenda item:	10.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for MDT BL CR to TS 38.401) Impact of IDC on MDT

Document for:	Text Proposal

# 1	Introduction

This paper follows up on discussion at RAN3#110-e related to how to handle MDT reporting to the TCE for UEs that report IDC (In-Device Coexistence).

# 2	Discussion

The IDC indication from the UE is carried over RRC in the UEAssistanceInformation message containing the idc-Assistance IE. This IE carries carrier frequency information and information about the victim radio type ("interferenceDirection "). From TS 38.331:

From the above it can be seen that NR reception is interfered if the indicator indicates "nr" or "both".

It seemed acknowledged at RAN3#110-e that the following measurements in the gNB-CU-UP are potentially impacted by NR impacting IDC problem:

⁻	M4: PDCP SDU Data Volume measurement separately for DL and UL, per DRB per UE, see TS 28.552 [17].

⁻	M5: Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB, see TS 28.552 [17].

⁻	M6: Packet Delay measurement separately for DL and UL, per DRB per UE, TS 28.552 [17] and TS 38.314 [18].

⁻	M7: Packet loss rate measurement separately for DL and UL, per DRB per UE, TS 28.552 [17] and TS 38.314 [18].

It has also been pointed out that the EM may configure management-based MDT directly to the gNB-CU-UP, without any reporting being requested from the gNB-CU-CP. On the other side, the CU-CP will be aware of the management based MDT session due to Cell Traffic Trace signalling. Furthermore, TS 37.320 specifies for LTE logged MDT that IDC "polluted" data (in-device coexistence problem) is not included in the UE's log. However, the same constraints would not apply to e.g. immediate MDT RAN measurements, where it could be an option to always report this data together with an IDC indication.

Also, for split architecture scenario we believe that the TCE could be located within the OAM system of the CU-UP, and the standard should therefore not require access to this TCE from the CU-CP although such access might exist in many cases. It therefore seems preferable to support IDC indication over E1, enabling the CU-UP to further transfer the IDC indication to the TCE.

Proposal 1: In order to support deployments with exclusive CU-UP access to the TCE, an IDC indication should be introduced over E1.

This leaves three options for handling in the CU-UP in case of IDC problem:

- Option 1: The CU-UP reports all configured measurements + IDC indication to the TCE

- Option 2: The CU-UP only reports IDC indication to the TCE (allowing the TCE to know that the UE remains in RRC connected mode), but not measurements M4-M7 if configured.

- Option 3: No reporting of M4-M7 measurements from the CU-UP to the TCE during IDC.

In some cases the M4-M7 measurements may be of use for operators even if impacted by IDC, and if they are not useful the IDC indicator would enable the TCE to filter them out. We therefore propose option 1.

Proposal 2: In case of IDC, the CU-UP reports all configured measurements + IDC indication to the TCE.

# 3	Conclusion

We have made the following two proposals:

Proposal 1: In order to support deployments with exclusive CU-UP access to the TCE, an IDC indication should be introduced over E1.

Proposal 2: In case of IDC, the CU-UP reports all configured measurements + IDC indication to the TCE.

Corresponding TP to TS 38.401 is provided in annex of this paper. We have also submitted related TPs to TS 37.320 (MDT stage 2) in R3-210749 and TS 38.463 in R3-210750.

# Annex	- TP for MDT BL CR to TS 38.401

<<< start of changes >>>

### 8.13.1	Signalling based MDT activation

The signalling flow for Signalling based MDT activation involving E1 and F1 is shown in Figure 8.13.1-1.

Figure8.13.1-1	Signalling based MDT Activation

1.	The AMF starts a trace session and sends a TRACE START message to the gNB. The AMF shall consider the MDT user consent information when activating an MDT trace session for the UE as defined in TS32.422 [20]. TRACE START message includes the parameters for configuring MDT measurements.

2. 	The gNB-CU-CP decides if the gNB-CU-UP, or the gNB-DU, or both, should be involved in the MDT measurement. If the gNB-CU-UP should be involved in the MDT measurement, the gNB-CU-CP sends TRACE START message to the gNB-CU-UP, including MDT configuration parameters.

3.	If the gNB-DU should be involved in the MDT measurement, the gNB-CU-CP sends TRACE START message to the gNB-DU, including MDT configuration parameters.

If the MDT measurement is performed in the gNB-CU-CP but not in the gNB-CU-UP, the gNB-CU-CP forwards in-device coexistence interference information received from the UE to the TCE as described in TS 37.320 [xx]. If the MDT measurement is performed in the gNB-CU-UP, the gNB-CU-CP updates the gNB-CU-UP with in-device coexistence interference information received from the UE. The gNB-CU-UP forwards the received in-device coexistence interference information to the TCE as described in TS 37.320 [xx].

<<< next change >>>

#### 8.13.2.1	General

In Management Based Trace Activation towards a gNB-CU-CP, gNB-CU-UP or a gNB-DU can be fulfilled with the Cell Traffic trace functionality defined in TS32.422 [20]. The configuration parameters of the Trace Session that are received by a node in split RAN architecture are defined in TS32.422 [20].

The following description is valid for both an en-gNB and a gNB.

If the MDT measurement is initiated by the EM towards the gNB-CU-CP, and if the activation involves measurements collected by multiple nodes under the same gNB-CU-CP control in a split RAN architecture, the EM sends MDT measurement activation to the gNB-CU-CP and the gNB-CU-CP may further decide which gNB-DU(s) or which gNB-CU-UP(s) to perform the MDT measurement.

When gNB-CU-CP or a gNB-DU receive the Trace Session Activation message from the management system for a given cell or a list of cell(s) under its control, the gNB-CU-CP or gNB-DU shall start a Trace Session for the given cell or list of cell(s). For Management Based MDT sent directly to a gNB-CU-UP, no MDT Area Configuration (apart from PLMN IDs) is to be included in the MDT activation indication.

Each node receiving an MDT activation indication reports the measurements collected according to such activation directly to the TCE the node has been configured with. If the node receives in-device coexistence interference information, it also forwards this information to the TCE as described in TS 37.320 [xx].

The signalling flow for management based MDT in gNB-CU-CP, gNB-DU and gNB-CU-UP is shown in Figure 8.13.2.2-1,  Figure 8.13.2.3-1 and in Figure 8.13.2.4-1 respectively.

<<< next change >>>

#### 8.13.2.4	Management based MDT Activation in gNB-CU-UP

The signalling flow for Management based MDT Activation in gNB-CU-UP is shown in Figure 8.13.2.4-1.

Figure 8.13.2.4-1		Management based MDT Activation in gNB-CU-UP

1.	The gNB-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP, including Management based MDT PLMN List.

2.	The gNB-CU-UP sends BEARER CONTEXT SETUP RESPONSE message to the gNB-CU-CP.

3.	The EM sends a Trace Session activation request to the gNB-CU-UP. This request includes the parameters for configuring UE measurements.

4.   The gNB-CU-UP shall select the suitable UEs for MDT data collection. If the serving PLMN is not within the Management Based MDT PLMN List the UE shall not be selected by the gNB-CU-UP for MDT data collection as defined in TS32.422 [20].

For each selected UE, the gNB-CU-UP may send CELL TRAFFIC TRACE message to the gNB-CU-CP in the E1 UE associated signalling, including Trace ID for MDT

5.	Upon reception of a CELL TRAFFIC TRACE message from E1, the gNB-CU-CP shall send CELL TRAFFIC TRACE message to the AMF for this UE, including Trace ID for MDT. The AMF forwards Trace ID and UE identity to the TCE.

The gNB-CU-CP updates the gNB-CU-UP with in-device coexistence interference information received from the UE.

<<< end of changes >>>