3GPP TSG-RAN WG3 Meeting #111e	R3-210738

# Online Meeting, 25th January – 5th February 2021

Agenda Item:	9.3.8.1

Source:	Ericsson, CATT, ZTE

Title: 	Including the Redundant UL NG-U UP TNL Information in the Modify Request

Document for:	Discussion and Approval

# 1	Discussion

At the last meeting, Ericsson submitted a correction CR in [1]

The issue is that in PDU Session Resource Modify Request Transfer, the “Additional Redundant UL NG-U UP TNL Information” is defined. But there is no IE for Redundant UL NG-U UP TNL Information either in Tabular or ASN.1 code.

This is very strange as in all the other messages, the two IEs Redundant UL NG-U UP TNL Information and the Additional Redundant UL NG-U UP TNL Information are presented as a “pair”, for the purpose of redundant QoS flow transmission, and the redundant QoS flow transmission when the additional NG-U tunnel is deployed.

During the discussion last meeting, there is a view that the existing Redundant UL NG-U UP TNL Information can be used.

Please note that is IE is included in the “UL NG-U UP TNL Modify List” which is supposed to be included (optionally) when the UP addresses are to be modified.

Now suppose we are to set up a new QoS flow, and indicated that the QoS flow has redundant transmission, we would need a corresponding new Redundant UL NG-U UP TNL Information.

And if the split PDU session is to be used, there is also a need for a corresponding new Additional Redundant UL NG-U UP TNL Information. But this one is already included.

In our view it is obvious when we did the feature, we simply forgot the IE Redundant UL NG-U UP TNL Information in the Modify Request for the new established QoS flow.

#### 9.3.4.3 PDU Session Resource Modify Request Transfer This IE is transparent to the AMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. PDU Session Aggregate Maximum Bit Rate; O; 9.3.1.102; YES; reject. UL NG-U UP TNL Modify List; 0..1; YES; reject. >UL NG-U UP TNL Modify Item; 1..<maxnoofMultiConnectivity>; -. >>UL NG-U UP TNL Information; M; UP Transport Layer Information 9.3.2.2; UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs.; -. >>DL NG-U UP TNL Information; M; UP Transport Layer Information 9.3.2.2; Identifies the NG-U transport bearer at the NG-RAN node.; -. >>Redundant UL NG-U UP TNL Information; O; UP Transport Layer Information 9.3.2.2; UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission.; YES; ignore. >>Redundant DL NG-U UP TNL Information; O; UP Transport Layer Information 9.3.2.2; Identifies the NG-U transport bearer at the NG-RAN node for the redundant transmission.; YES; ignore. Network Instance; O; 9.3.1.113; This IE is ignored if the Common Network Instance IE is included.; YES; reject. QoS Flow Add or Modify Request List; 0..1; YES; reject. >QoS Flow Add or Modify Request Item; 1..<maxnoofQoSFlows>; -. >>QoS Flow Identifier; M; 9.3.1.51; -. >>QoS Flow Level QoS Parameters; O; 9.3.1.12; -. >>E-RAB ID; O; 9.3.2.3; -. >>TSC Traffic Characteristics; O; 9.3.1.130; This IE may be present in case of GBR QoS flows and is ignored otherwise.; YES; ignore. >>Redundant QoS Flow Indicator; O; 9.3.1.134; This IE indicates whether this QoS flow is requested for the redundant transmission.; YES; ignore. QoS Flow to Release List; O; QoS Flow List with Cause 9.3.1.13; YES; reject. Additional UL NG-U UP TNL Information; O; UP Transport Layer Information List 9.3.2.12; UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of UL PDUs for split PDU session.; YES; reject. Common Network Instance; O; 9.3.1.120; YES; ignore. Additional Redundant UL NG-U UP TNL Information; O; UP Transport Layer Information List 9.3.2.12; UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of redundant UL PDUs for split PDU session.; YES; ignore. Redundant Common Network Instance; O; Common Network Instance 9.3.1.120; YES; ignore.

Proposal 1: RAN3 to agree to including the Redundant UL NG-U UP TNL Information in the Modify Request.

The CR is submitted in [2].

# 2	Proposals

Proposal 1: RAN3 to agree to including the Redundant UL NG-U UP TNL Information in the Modify Request.

# 3	Reference

[1]	R3-206610	Including the Redundant UL NG-U UP TNL Information in the Modify Request, Ericsson

[2]	R3-210737	Including the Redundant UL NG-U UP TNL Information in the Modify Request, Ericsson, CATT, ZTE