3GPP TSG-RAN WG3 Meeting #111-e	R3-210750

E-meeting, 25 January – 4 February, 2021

Agenda item:	10.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for MDT BL CR to TS 38.463) Introduction of IDC indication for MDT

Document for:	Text Proposal

# 1	Introduction

As discussed in R3-210663, this paper proposes E1AP signalling for in-device coexistence interference (IDC) information reported by the UE. The proposed signalling is based on the GNB-CU-CP Measurement Results Information signalling procedure.

# Annex	- TP for MDT BL CR to TS 38.463

<<< start of changes >>>

#### 9.2.2.19	GNB-CU-CP MEASUREMENT RESULTS INFORMATION

This message is sent to the gNB-CU-UP to provide the measurement result received by the gNB-CU-CP.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Measurement Results Information List |  | 1 |  |  | YES | reject |
| >DRB Measurement Results Information Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >> UL D1 Result | O |  | INTEGER (0 .. 10000,…) | The unit is: 0.1ms | - |  |
| IDC Information | O |  | ENUMERATED (nr, not nr, …) | Value 'nr' indicates that NR radio in the UE is victim of IDC interference. Value 'not nr' indicates that NR radio is not victim of IDC interference. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

<<< end of changes >>>