3GPP TSG-RAN WG3 #113-e	R3-214096

Online, Aug 16th – Aug 26th, 2021 e-Meeting

Title:	Discussion on Mobility with Service continuity

Source:	CMCC

Agenda item:	22.3.1

Document for:	Discussion

Introduction

Mobility between MBS supporting nodes was discussed in RAN3#112e and the related work assumptions are as follows:

- WA aim to support lossless handover

- MRB data forwarding is supported. PDCP SN synchronization between source and target is needed for data forwarding.

- WA: source and target gNBs derive synchronized PDCP SN from sequence number in NG-U. The NG-U sequence number selection (between QFI SN and GTP-U SN) is FFS.

In this contribution, we provide our view on mobility between MBS supporting nodes and the service continuity during the handover.

Discussion

Mobility between MBS supporting nodes

In RAN2#112e meeting, RAN2 already agreed to support lossless handover for MBS-MBS service, PDCP SN synchronization and continuity between the source cell and the target cell. In last meeting, we were unable to reach any conclusion in this topic. In RAN3#113e meeting, we hope to align with agreements in RAN2.

In this section, we will discuss the message flow between MBS supporting nodes for RRC_CONNECTED UEs. We take the message flow of inter-gNB handover in TS 38.300 as baseline and make some modifications. The message flow of UE handover between MBS supporting nodes is shown as Figure1. Before handover, target gNB needs to know MBS context information UE joined in source gNB through the Handover Request message. MBS context information includes MBS session ID, MBS QoS flow ID, TMGI and the mapping rules of MBS QoS flow to MRB.

Proposal 1: The message flow of UE handover between MBS supporting nodes is taken the message flow of inter-gNB handover in TS 38.300 as baseline and get enhancement on this basis.

After receiving the handover request from source gNB, target gNB sends MBS configuration in Handover Request Acknowledgement message, which includes a transparent container to be sent to the UE as an RRC message to perform the handover. Reply message also includes MRB transmission status information of target gNB. Once receiving the Handover Request Acknowledgement, the source gNB triggers the Uu handover by sending an RRCReconfiguration message to the UE.

Observation 1: After receiving the handover request from source gNB, target gNB sends MBS configuration in Handover Request Acknowledgement message, which includes a transparent container to be sent to the UE as an RRC message to perform the handover.

Next, the UE initiates handover and gets access to the target gNB. The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey MRB transmission status information of source gNB. Meanwhile, data forwarding seems not necessary. Whether to perform data forwarding should be discussed in next chapter on a case-by-case basis.

Observation 2: The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey MRB transmission status information of source gNB.

Then, the UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB. The target gNB sends a PATH SWITCH REQUEST message to AMF to trigger 5GC to switch the DL data path towards the target gNB. 5GC switches the DL data path towards the target gNB and the AMF confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

Finally, we would like to introduce a new reply message from the target gNB to the source gNB over Xn interface. Reply message will include the current buffer status of target gNB, unreceived and lost packet numbers. If data forwarding is available, instead of end marker, this message represents an indication to stop the data forwarding.

Proposal 2: Introduce a new reply message from the target gNB to the source gNB over Xn interface.

Figure1 Message Flow of UE handover between MBS supporting nodes

Data Forwarding

Aiming to support lossless handover, data forwarding is demanded between two MBS-supporting nodes. As discussed in another paper for PDCP SN synchronization, we think the GTP-U SN is the best candidate for SN used in NG-U tunnel. For convenience’s sake, we assume each MRB has only one MBS flow. We also ignore the time of UE taken during handover. Next, we will discuss two possible scenarios in data forwarding.

Figure 2 Transmission rate in source gNB faster than the rate in target gNB

In the first scenario, it is assumed that the transmission rate in source gNB is faster than the rate in target gNB. The PDCP SN of the packet latest received in source gNB is larger than the ones being delivered by target gNB. As figure 2 illustrated, source gNB has already sent packets from packet1 to packet8, and target gNB has already sent packets from packet1 to packet5. Since the rate in source is faster than the rate in target, UE will repeatedly receive packets from packet6 to packet8 in target side and discard those duplicated packets with the same SN. Data forwarding is not required. The above is in an ideal scenario, it is noted that data loss may occur:

- If packet loss occurs.

- If target gNB maintains a huge buffer containing all the data packets and the target sent the packet with the same number, target needs to inform source that its own buffer status. Data forwarding is not required.

- If lost packet is not stored in the buffer of the target gNB and target sent the packet with the same number, target needs to inform source that its own buffer status and lost packet number. Data forwarding is required.

- If lost packet is not stored in the buffer of the target gNB and target does not send the packet with the same number, UE will repeatedly receive packets in the future. Data forwarding is not required.

In the scenario of the rate in source faster than the rate in target, whether to perform data forwarding is correlated with the buffer size.

Figure 3 Transmission rate in source gNB is slower than the rate in target gNB

In the second scenario, it is assumed that the transmission rate in source gNB is slower than the rate in target gNB. The PDCP SN of the packet latest received in source gNB is smaller than the ones being delivered by target gNB. As figure 3 illustrated, source gNB has already sent packets from packet1 to packet5, and target gNB has already sent packets from packet1 to packet8. Since the rate in source is slower than the rate in target, UE will continue to receive packets from packet6 to packet8 to fill the transmission gap between source gNB and target gNB. It is noted that data loss may occur, we analyse the possible situation independently:

- If packet loss does not occur.

- If target gNB maintains a huge buffer containing all the data packets and the target sent the packet with the same number, target needs to inform source that its own buffer information its own buffer status. Data forwarding is not required.

- If lost packet is not stored in the buffer of the target gNB and target already sent the packet with the same number, target needs to inform source that its own buffer status and unreceived packet numbers. Data forwarding is required.

- If packet loss occurs.

- If target gNB maintains a huge buffer containing all the data packets, target needs to inform source that its own buffer information its own buffer status. Data forwarding is not required.

- If lost packet is not stored in the buffer of the target gNB and unreceived packet is stored in the buffer of the target gNB, target needs to inform source that its own buffer status and lost packet number. Data forwarding is required.

- If lost packet and unreceived packet are not stored in the buffer of the target gNB, target needs to inform source that its own buffer status, unreceived packet number as well as lost packet number. Data forwarding is required.

Observation 3: Since the rate in source is slower than the rate in target, UE will fill the transmission gap between source gNB and target gNB.

In the scenario of the rate in source slower than the rate in target, whether to perform data forwarding is also correlated with the buffer size.

Proposal 3: Whether to perform data forwarding is also correlated with the buffer size.

Conclusion

In this paper, we provide our view on mobility between MBS supporting nodes and the data forwarding during the handover. We propose message flow of UE handover between MBS supporting nodes and introduce the reply message from target gNB to source gNB. We reach the following observations and proposals:

Observation 1: After receiving the handover request from source gNB, target gNB sends MBS configuration in Handover Request Acknowledgement message, which includes a transparent container to be sent to the UE as an RRC message to perform the handover.

Observation 2: The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey MRB transmission status information of source gNB.

Observation 3: Since the rate in source is slower than the rate in target, UE will fill the transmission gap between source gNB and target gNB.

Proposal 1: The message flow of UE handover between MBS supporting nodes is taken the message flow of inter-gNB handover in TS 38.300 as baseline and get enhancement on this basis.

Proposal 2: Introduce a new reply message from the target gNB to the source gNB over Xn interface.

Proposal 3: Whether to perform data forwarding is also correlated with the buffer size.

References

- TR 23.757 Study on architectural enhancements for 5G multicast-broadcast services

- TS 38.300 NR and NG-RAN Overall Description

- R3-211548, Discussion on MBS mobility procedure, ZTE

- R3-212430, (TP to TS38.300 BL CR) Data forwarding between MBS supporting nodes, Huawei