3GPP TSG-RAN WG3 Meeting #112e	R3-214378

Online, 16 - 27 August 2021

Agenda item:	  22.2.2

Source:	  Lenovo, Motorola Mobility, Huawei, Nokia, Nokia Shanghai Bell, CMCC

Title:		(TP to TS 38.420 BL CR) Group paging for Multicast Session Activation Notification

Document for:		Other

1	Introduction

In this contribution, the text proposal are provided for the TS 38.420 BL CR according to the proposal in [1]；

A new class 2 XnAP RAN MBS Grouping Paging procedure is introduced, of which the contents include a list of UE Identity Index values and paging DRX (optional) of the UEs in the MBS group, RAN paging area, and multicast session ID.

[1] R3-213743. Discussion on Group paging for Multicast Session Activation Notification. Lenovo, Motorola Mobility.

2	Text Proposal

----------------------------------------------------------Change Start --------------------------------------------------------------

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SCTP	Stream Control Transmission Protocol

Xn-C	Xn Control plane

Xn-U	Xn User plane

MBS	Multicast Broadcast Service

----------------------------------------------------------Next Change--------------------------------------------------------------

6.2.X	MBS Management procedures

The MBS management procedures are used to manage the MBS Session:

-	RAN Multicast Group Paging procedure