3GPP TSG-RAN WG3 #113-e							                                              R3-214471

16-26 Aug 2021, Online

Title:	Reply LS on QoE configuration and reporting related issues

Response to:	R2-2106776/R3-213124 LS on QoE configuration and reporting related issues

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN3

To:	RAN2, SA4, SA5

Cc:

Contact person:

Name:	Xingyu HAN

Tel. Number:

E-mail Address:	hanxingyu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN3 would like to thank RAN2 for the LS on QoE configuration and reporting related issues.

With respect to the questions (related with RAN3) raised in the LS, RAN3’s answers are as follows,

Issue 1: Modify the QoE measurement configuration to UE

RAN2 is discussing QoE configuration signalling support, and RAN2 agreed QoE configuration are encapsulated in a transparent container in the RRC messages. RAN2 does not see the scenario that a QoE measurement configuration already configured in the UE will be modified for e.g., a certain service type or a QoE Reference, and assumes modification is not supported in RRC signalling. RAN2 would like SA5/RAN3 to confirm this assumption.

Answer: RAN3 is also discussing the similar issue, and also waiting for the feedback from SA4 in an earlier related LS about QoE configuration modification in R3-212953.

However, RAN3 would like to further check opinions with SA4 and SA5 on slice scope change. Slice scope is a list of S-NSSAIs that is included in QoE configuration provided by OAM and associated with a unique QoE Reference, and the NG-RAN node will check the criteria (e.g. including area scope and slice scope) to decide whether to send the QoE configuration to the UE. RAN3 would like to know whether there is a need to support modification in cases of slice scope change.

Issue 2: Provide multiple QoE measurement configurations for one certain service type

RAN2 is discussing QoE configuration and reporting signalling support, and some companies mention it is possible that multiple QoE measurement configurations can be provided to UE for one certain service type, e.g. different QoE measurement configurations for different slices may be applied to one service type, or different QoE measurement configurations may be applied for different application providers. RAN2 would like to check with SA5/RAN3 whether it is possible to provide multiple QoE measurement configurations for one certain service type?

Answer: From RAN3 perspective, it is possible to provide multiple QoE measurement configurations for one certain service type. But it’s up to RAN2 and SA5 to make the final confirmation.

Regarding the slice example in Issue 2, the condition is dependent on whether QoE measurement is configured per service type, and whether different slices for the same service type are provided with the same QMC configuration container. RAN3 can confirm that QoE measurement configurations are configured per service type, but RAN3 is unable to confirm that different slices for the same service type will be provided with the same container, so further confirmation from SA4 is needed.

Regarding the application provider example in Issue 2, from RAN3 perspective, each QoE measurement configuration is associated with a unique QoE Reference and only one QMC MCE address. And RAN3 assumes it is possible that different slices for the same service type are configured with different QMC MCE addresses, which is to be confirmed with SA5. So in this case, multiple QoE measurement configurations may be provided for a certain service type.

As a summary, RAN3 would like to ask SA4 and SA5 the following questions:

Q1: Whether there is a need to support modification in cases of slice scope change.

Q2: Whether different slices for the same service type are provided with the same content within the QoE configuration container.

Q3: Whether it is possible that different slices for the same service type can be configured with different QMC MCE addresses.

2	Actions

To RAN2

ACTION: RAN3 respectfully asks RAN2 to take the above information into account.

To SA4, SA5

ACTION: RAN3 respectfully asks SA WGs to provide feedback on Q1-Q3 above.

3	Dates of next RAN3 meetings

3GPP TSG RAN3#114e		11/2021	E-Meeting

3GPP TSG RAN3#114bis-e		01/2022	E-Meeting