3GPP TSG-RAN WG3 Meeting #113-e	R3-213691

Online, 16 – 26 August, 2021

Agenda Item:	13.2.1.1

Source: 	Lenovo, Motorola Mobility

Title: 	Remaining issues for IAB inter-donor migration

Document for:	Discussion and decision

- Introduction

In R16, IAB intra-donor migration has been specified, and in the R17 the scenario has been extended to inter-donor migration which is the key feature of the IAB R17. After discussion by three e-meetings, the procedure of the IAB inter-donor migration becomes more and more clear, and some phased agreements have been achieved in the last RAN3 111-e and RAN3 112-e meetings [1][2].

RAN3 111-e: For a single-connected IAB-MT: The procedure for inter-donor migration of a (top-level) migrating IAB-MT supports: - reuse Xn handover procedure of the (top-level) migrating IAB-MT between two parent nodes connected to different IAB-donors, and - the migration of F1 transport path for the collocated and all descendent IAB-DUs (i.e. the anchor nodes for the logical F1 connection do not change) Inter-donor migration may terminate after top-level IAB-MT migration WA: migration of collocated IAB-DU after the migration of the (top-level) migrating IAB-MT, is not precluded If collocated IAB-DU is migrated, the Inter-donor migration procedure involves, among others: - the establishment of an F1-C association to the target donor, and - the context migration of the IAB-DU’s UEs and child IAB-MTs to the target CU. RAN3 112-e: For IP address assignment of boundary IAB node (outer IP address assignment for IPSec tunnel mode) during inter-donor migration (regardless of Partial migration or Full migration) - IP address request via RRC container relies on RAN2 inputs - The new IP address(es) should be explicitly provided to the source donor CU for IPSec transport mode (non-IPSec case FFS). -- FFS on which signaling is used (Handover Request ACK message vs. GNB-DU CONFIGURATION UPDATE message) -- FFS on whether it is applied for IPSec tunnel mode - FFS on providing the coupling of IP addresses used in the CU1 network and in the CU2 network - FFS on updating IP address of source IAB donor CU St2 procedure for the inter-donor migration to include the following phases: Phase 1: Serving cell change of the boundary IAB-MT using Xn handover. Phase 2: Migration of F1 transport path. UEs accessing to boundary IAB node and to descendant node(s) shall not be impacted by the F1 transport path migration. The following two implementation alternatives, which involve two logical IAB-DUs at the boundary IAB node, are to be further discussed in the scope of Full Migration: - Alt1: the two logical DUs use separate physical cell resources - Alt2: the two logical DUs use the same physical cell resources

In this contribution, some aspects for IAB inter-donor migration are further discussed, including IP address management, and the routing and bearer mapping for the boundary IAB node.

- Discussion

IP address management

In this last RAN3 112-e meeting, several agreements had been achieved for IP address management. However, there still some issues remain FFS and should be addressed in this meeting.

As for the explicit IP address provision to the source donor CU, it had been agreed that the new IP address(es) should be explicitly provided to the source donor CU for IPSec transport mode. In addition, for the IPsec tunnel mode, source donor CU also needs to communicate with boundary IAB node via the new outer IP address(es), e.g. the DL destination IP address(es) need to be updated by the source CU for the DL data transport.

Proposal 1: The new IP address(es) should be explicitly provided to the source donor CU for IPSec tunnel mode.

While for the detail signaling to transport the new IP address(es), target donor CU can directly inform the new IP address(es) to the source donor CU via the enhanced X2AP message. Alternatively, source donor CU can obtain the new IP address(es) from the boundary IAB node via the existing F1AP message, e.g. GNB-DU CONFIGURATION UPDATE or IAB UP CONFIGURATION UPDATE RESPONSE, after the boundary IAB node has been configured the new IP address(es). For the GNB-DU CONFIGURATION UPDATE, the Transport Layer Address Info IE can be reused but it only applied for the IPsec tunnel mode. And for the IAB UP CONFIGURATION UPDATE RESPONSE, it may need source donor CU to trigger the IAB UP configuration update procedure to obtain the updated IP address(es) from the boundary IAB node, and the time to trigger the procedure can be implemented timely since the RRCreconfiguration which includes the new IP address(es) is also controlled by the source donor CU.

Two approaches can work well for provision the IP address(es) to source donor CU. But to minor the standard impacts, existing F1AP message, e.g. GNB-DU CONFIGURATION UPDATE or IAB UP CONFIGURATION UPDATE RESPONSE is proposed for transporting the new IP address(es).

Proposal 2: Existing F1AP message, e.g. GNB-DU CONFIGURATION UPDATE or IAB UP CONFIGURATION UPDATE RESPONSE is used for transporting the new IP address(es).

For the coupling of IP addresses used between in source donor CU and target donor CU, since the duty of the target donor CU in the IP request procedure is just to allocate the number of IP addresses requested by the source donor CU, while for the coupling relation, it may need to be determined by the source CU or the boundary IAB node.

Proposal 3: Target donor CU doesn’t need to provide the coupling of IP addresses to source target CU, and the coupling relation may be determined by the source donor CU or the boundary IAB node.

Partial migration

Figure 1: An example for top-level IAB-MT inter-donor migration

Inter-donor migration can terminate after partial migration, then the boundary IAB-MT connects to the target donor CU while the boundary IAB-DU connects to the source donor CU. In this stage, resource collision problem will emerge and have been analyzed by another agenda. While for the routing and bearer mapping configuration for boundary IAB node, further discussion is necessary.

In R16, BAP routing and bearer mapping for BAP entity are configured by the IAB-donor-CU via F1AP signaling both for the UL(MT) and DL(DU), except for the UL default configuration which configured by RRC. And IAB node delivers the UL BAP routing and bearer mapping from DU to the collocated MT via the internal interface.

However, for the boundary IAB node which MT and DU subject to different CUs, the legacy way for UL BAP routing and bearer mapping configuration is no longer suitable. Firstly, when it comes to the configuration entity, target donor CU is preferred than source donor CU since MT has migrated from source donor CU to target donor CU.

And for the boundary IAB-DU, the DL routing and bearer mapping configurations are still determined by source donor CU.

Proposal 4: For the IAB node which MT and DU subject to different donor CUs, the UL routing and bearer mapping configurations are determined by the CU which MT connected, while the DL routing and bearer mapping configurations are determined by the CU which DU connected.

But the target donor CU doesn’t have the F1 connection with boundary IAB node and the configuration cannot be transported to IAB node via the F1AP signaling. Thus, how to transport the configuration between boundary IAB node and target donor CU need to be studied. One straightforward method is to use the RRC message directly between target donor CU and boundary IAB-MT. Alternatively, the configuration which generated by the target donor CU can be firstly transported to source donor CU and then forward to the boundary IAB-node via F1AP.

Proposal 5: The generated UL routing and bearer mapping configurations can be delivered to the boundary IAB-MT via the following two candidate options.

- Opt1: the target donor CU configure the configuration to the boundary IAB-MT directly via RRC

- Opt2: the configurations are sent to source donor CU and then forward to the boundary IAB-node via F1AP

Full migration

In last several meetings, full migration had been discussed and still no consensus had been achieved on whether to support the full migration. In addition, there still some technical issues need to be resolved by other working groups and an LS [3] was sent to RAN1, RAN2 and RAN4. Therefore, whether to support the full migration need to wait for the inputs from other working groups.

Proposal 6: Whether to support the full migration need to wait for the inputs from other working groups.

# Conclusion

This contribution aims to analyze the remaining issues for IAB inter-donor migration. And following observations and proposals are concluded.

Proposal 1: The new IP address(es) should be explicitly provided to the source donor CU for IPSec tunnel mode.

Proposal 2: Existing F1AP message, e.g. GNB-DU CONFIGURATION UPDATE or IAB UP CONFIGURATION UPDATE RESPONSE is used for transporting the new IP address(es).

Proposal 3: Target donor CU doesn’t need to provide the coupling of IP addresses to source target CU, and the coupling relation may be determined by the source donor CU or the boundary IAB node.

Proposal 4: For the IAB node which MT and DU subject to different donor CUs, the UL routing and bearer mapping configurations are determined by the CU which MT connected, while the DL routing and bearer mapping configurations are determined by the CU which DU connected.

Proposal 5: The generated UL routing and bearer mapping configurations can be delivered to the boundary IAB-MT via the following two candidate options.

- Opt1: the target donor CU configure the configuration to the boundary IAB-MT directly via RRC

- Opt2: the configurations are sent to source donor CU and then forward to the boundary IAB-node via F1AP

Proposal 6: Whether to support the full migration need to wait for the inputs from other working groups.

# Reference

- Chairman notes of RAN3 111 e-meeting.

- Chairman notes of RAN3 112 e-meeting.

- R3-212981. LS on Inter-donor migration.