3GPP TSG-RAN WG3 Meeting #113-e	R3-214078

E-meeting, 16-26 Aug 2021

Title: 	Further discussions on general principles and framework

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

1. Introduction

The general principles and functional framework for data collection was discussed during RAN3#112-e. In R3-212688 [1], the summary of the offline discussion was presented. This paper tried to have further discussions on the remaining open issues.

2. Background

-	Mark the Model Performance Feedback in the Functional Framework (Figure 4.2-1) as FFS and continue discussions on what such transfer of information should entail and for which purpose.

To be continued...

-	The definition of the Model Deployment/Update function is FFS. Discussions need to be continued to identify what information will “Model Deployment/Update” transfer, whether this information will need to be standardised and, if not, what are the assumptions on this information

To be continued...

-	When discussing use cases, check on each use case the feasibility of a “validity time” (i.e. “best before” for the prediction result) as additional information provided by the Model Inference function together with the Inference output.

To be continued...

-	Discussions should be continued on the following principle, especially concerning what the level of accuracy is:

If the inference function provides output predictions, an optional indication of the accuracy level for which the inference model is trained should be indicated to the nodes that request/subscribe to this information.

To be continued...

Open issues that may be further discussed:

- output from one model as input to another

- high-level principles for inference function

To be continued...

3. Discussion

For the model performance feedback, the following issues should be considered:

- In the last meeting, the group agreed to change the performance feedback to feedback with the clarification that the feedback includes the information that may be needed to derive training or inference data or performance feedback. With this change, the feedback from actor to data collection is not only limited to model performance. For example, the feedback can be the new state that the “Subject of action” transitions to after the action in case of reinforcement learning.

Observation 1: Feedback from actor to data collection is already there in current functional framework.

- Model performance feedback is used for transferring the model performance from the model inference to the model training. For example, it is used by the model training to learn new knowledge from new training data. However, there is no good understanding about the model performance by the model inference itself. This is typically done only after the action is taken can the model be evaluated. In this case, the model performance feedback can be transferred from data collection to model training to support online training or reinforcement learning.

Observation 2: Model inference could not provide the model performance feedback to the model training without the help of action.

- Data collection is responsible for collecting all types of data, and it can be located in any entities providing the data related to the AI model training. Therefore, the data collection does already support the transferring performance feedback from actor to model training or model inference, and model performance feedback from model inference to model training. There’s no need to duplicate model performance feedback from model inference to model training, and performance feedback from action to model training.

Observation 3: Data collection could be responsible for collecting data which could be used for training and or inference.

Therefore, we propose to remove the model performance feedback harrow. We also propose to describe the data collection function in further details.

Remove the “model performance feedback” arrow.

For the model deployment/update, it is conceptually correct from the functional framework perspective. However, it is argued what information will “Model deployment/update” transfer, whether this information will need to be standardized and if not, what are the assumptions on this information.

Before we jump into this discussion, we should mention that the group agreed the following high level principles:

- The detailed AI/ML algorithms and models for use cases are out of RAN3 scope.

The reason behind that is the AI/ML algorithms and models are implementation specific and as such the corresponding details of the AI/ML algorithms and models are various and software and hardware dependent, which is difficult to be standardized. For example, the AI/ML algorithms are highly related to the model technologies (e.g., gradients in the learning setting) and model execution environment, which are not expected to be specified in 3GPP.

However, since AI model and related algorithm play very important role concerning the final performance, implementation thus has to guarantee that model deployment update would take effect.

Implementation should guarantee that model deployment update should take effect.

For the level of accuracy for the output prediction, it is helpful for the receiver to understand the prediction performance of the algorithm that generate it, in order to understand how to trust the prediction. However, we should mention that the accuracy of the prediction is exactly the confidence, but not the accuracy of the AI/ML algorithm and model.

- Accuracy: it defines the skill of the learning algorithm to predict accurately. It defines the percentage of correct predictions made from all predictions. It is used to describe the trained AI/ML model at the model training function. For example, if the algorithm makes 970 good predictions out of 1000 examples, it means the algorithm accuracy is 97%. Typically, during the model training phase, a portion of the sample data is saved for validation purpose.

- Confidence: AI/ML is probabilistic and the probability of each single prediction can be indicated by the confidence score. In other words, the confidence scores reflect the chances that the AI/ML is correct. Typically, the confidence score can be calculated for single input as well giving the meaning as how much the algorithm is confident for that class. If a class has high probability then it has high confidence.

As we could see that for either accuracy or confidence, it is to evaluate the efficiency or performance of the model training which is also algorithm independent, and it seems to be used for the model training or inference to evaluate itself.

Observation: Technically, accuracy is to describe the precision of a model training, while confidence is to indicate the probability of inference output approaching the reality; but they seem to be used for the model training or inference to evaluate its performance by itself.

For the validity time for which the output information is valid, it may not be always needed for some use cases.

- The assumption by RAN3 so far is that the received information are valid before a new version of such information is received. For example, in the resource status reporting procedure, the NG-RAN node will use the latest received cell load information exchanged over the Xn interface with the neighbour NG-RAN node to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cell or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to achieve network energy saving.

- The output information can be reported to the subscribed node with a given periodicity, which means the output information is valid during the time period (validity time). For example, in the resource status reporting initiation procedure, the NG-RAN node can request the target node to report the load information with the reporting periodicity, e.g., 500ms, 1000ms, 2000ms.

Therefore, there is no need to include the validity time together with the output information every time. Instead, it can be considered on a case by case manner when discuss the use case and solution.

No need to include the validity time together with the output information every time.

For the output from one model as input to another, it is very naturally needed to be supported. For example, the output from the AI/ML models for prediction making such as predicted traffic load could be utilized as the input for the AI/ML model used for decision making on the load balancing.

Output from one model could be used as input to another model.

The last open issue is about the data preparation. The data preparation may include the data pre-processing and feature engineering. The data pre-processing is to discard privacy, irrelevant and redundant information or noisy and unreliable data. The feature engineering includes feature extraction/ normalization and feature selection. The feature extraction is to derive feature values. The feature normalization is to transform the sample data set from one range to another. The feature selection is to determine the most relevant features in order to prevent over-fitting or under-fitting.

However, the data preparation always highly depends on the model design, and there is no common processing method. For example, the stopping criteria for selecting the best subset of features in Wrapper methods (e.g., forward selection, backward elimination, Bi-directional elimination) are usually pre-defined such as when the performance of the model decreases or a specific number of features has been achieved. Another example is the Embedded methods (e.g., regularization, tree-based methods), where the feature selection is blended as part of the learning algorithm thus having its own built-in feature selection methods.

Confirm that the data preparation is part of model training function or model inference function.

As discussed before, since RAN is an intermediate pipe for transmission which should not act as a storage or memory, the main intention is to maintain user data privacy, we think this should also be a general principle to follow and respect.

RAN should not act as a data storage or data memory, user data privacy should be respected during AI/ML operation.

4. Conclusion

Based on the discussion in this paper, we propose the following:

Observation: Technically, accuracy is to describe the precision of a model training, while confidence is to indicate the probability of inference output approaching the reality; but they seem to be used for the model training or inference to evaluate its performance by itself.

Proposal 1:	Remove the “model performance feedback” arrow.

Proposal 2:	Implementation should guarantee that model deployment update should take effect.

Proposal 3:	No need to include the validity time together with the output information every time.

Proposal 4:	Output from one model could be used as input to another model.

Proposal 5:	Confirm that the data preparation is part of model training function or model inference function.

Proposal 6:	RAN should not act as a data storage or data memory, user data privacy should be respected during AI/ML operation.

5. Reference

- R3-212688, SoD data collection principles and framework, Ericsson.

- 3GPP TS 23.288, Architecture enhancements for 5G System (5GS) to support network data analytics services.

Annex – TP

START OF CHANGE

4	General Framework

Editor Note: high level principles for RAN intelligence enabled by AI, the functional framework (e.g. the AI functionality and the input/output of the component for AI enabled optimization)

4.1	High-level Principles

The following high level principles should be applied for AI-enabled RAN intelligence:

- The detailed AI/ML algorithms and models for use cases are out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

- The input/output and the location of Model inference function should be studied case by case.

- RAN3 should focus on the analysis of data needed at the Model training function from external functions, while the aspects of how the Model training function uses inputs to train a model are out of RAN3 scope.

- Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

- The Model training and Model inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the algorithm.

- The Model inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g. via subscription), or nodes that are subject to actions based on the output from model inference.

- NG-RAN is prioritized; EN-DC is included in the scope. FFS on whether MR-DC should be down-prioritized.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

- RAN should not act as a data storage or data memory, user data privacy should be respected during AI/ML operation.

4.2	Functional Framework

Figure 4.2-1: Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1.

- Data Collection is a function that provides input data to Model training and Model inference functions. AI/ML algorithm specific pre-processing of data is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, performance feedback, AI/ML model output.

  - Training Data: information needed for the AI/ML model training function.

  - Inference Data: information needed as an input for the Model inference function to provide a corresponding output.

- Model Training is a function that performs the training of the ML model. The Model training function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation of raw data), if required.

- Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions). The Model inference function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation of raw data), if required. The AI/ML model inference output may be used as input for another Model inference function.

- Actor is a function that receives the output from the Model inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

- Feedback: Information that may be needed to derive training or inference data or performance feedback or model performance feedback.

NOTE:	Implementation should guarantee that model deployment update should take effect.