3GPP TSG-RAN WG3 Meeting #114-e	R3-215938

# E-meeting, 1-11 Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0693 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Redundant network instance for split PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT, CMCC, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At last RAN3-113-e meeting, the CR R3-214406 was agreed, to add the common network instance into the UP Transport Layer Information List IE. But the procedural text does not clearly state to which exact scenario it applies i.e. whether this applies only to the Additional UL NG-U UP TNL Information IE or also to the Additional Redundant UL NG-U UP TNL Information IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the Common Network Instance IE included in the UP Transport Layer Information List IE can be used for both the Additional UL NG-U UP TNL Information IE or also to the Additional Redundant UL NG-U UP TNL Information IE . Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the redundant data transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous whether the common network instance included in the UP Transport Layer Information List applies only the to the Additional UL NG-U UP TNL Information IE or also to the Additional Redundant UL NG-U UP TNL Information IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423 CR0696 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-215366 Rev1: R3-215938 Update the procedure texts based on online discussion. |  |  |  |  |  |  |  |  |

Change Begins

8.2.1	PDU Session Resource Setup

8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

<Unchanged Text Omitted>

For each PDU session, if the Additional UL NG-U UP TNL Information IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may allocate for this split PDU session resources for an additional NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Setup Request List IE and it shall indicate these QoS flows in the Additional DL QoS Flow per TNL Information IE in the PDU Session Resource Setup Response Transfer IE. In case the Additional DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional UL NG-U UP TNL information as available again.

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and the Common Network Instance IE is not present, the NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [9].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE or in the Additional UL NG-U UP TNL Information IE, or in the Additional Redundant UL NG-U UP TNL Information IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource for the concerned NG-U transport bearer as specified in TS 23.501 [9].

For each PDU session, if the Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for this PDU session for the redundant transmission and it shall include the Redundant QoS Flow per TNL Information IE in the PDU Session Resource Setup Response Transfer IE as described in TS 23.501 [9].

For each PDU session, if the Additional Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may allocate for this split PDU session resources for an additional redundant NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Setup Request List IE and it shall indicate these QoS flows in the Additional Redundant DL QoS Flow per TNL Information IE in the PDU Session Resource Setup Response Transfer IE. In case the Additional Redundant DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional Redundant UL NG-U UP TNL information as available again.

For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [9].

<Unchanged Text Omitted>

8.2.3	PDU Session Resource Modify

8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

<Unchanged Text Omitted>

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message and the Common Network Instance IE is not present, the NG-RAN node shall, if supported, use it as specified in TS 23.501 [9].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE or in the Additional UL NG-U UP TNL Information IE, or in the Additional Redundant UL NG-U UP TNL Information IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource for the concerned NG-U transport bearer as specified in TS 23.501 [9].

For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, use it for the redundant transmission as specified in TS 23.501 [9].

<Unchanged Text Omitted>

Change Ends