3GPP TSG-RAN WG3 Meeting #114-e	R3-215143

E-meeting, 01 - 11 Nov 2021

Title: 	(TP to TS 38.425 BL CR) Flow Control for MBS

Source: 	Huawei, CBN, China Unicom, China Telecom

Agenda item:	22.2.4

Document Type:	other

1. Introduction

In previous RAN3 meetings, the following agreements related to Flow Control were made:

Agreements: - WA: Standard shall enable a one to one mapping between an MRB and a shared F1-U tunnel - Applicability of current flow control is FFS - Flow control should be enabled for an MRB established for a broadcast MBS session. - WA: For Broadcast and Multicast, optional use of DL flow control in the shared F1-U tunnel. - WA: For split MRB with common PDCP, shared F1-U tunnel is used, existing NR user plane protocol functions need to be reviewed for their applicability for MBS.

In this contribution, we will further discuss the Flow Control for Multicast and Broadcast.

2. Discussion

## 2.1 Introduction of flow control

For current flow control, the Downlink Data Delivery Status (DDDS) is used to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the downlink user data flow via the corresponding node for the respective data radio bearer and control the successful delivery of DL control data to the corresponding node.

As mentioned in TS 38.425, in case of RLC UM, the corresponding node reports the highest NR PDCP PDU sequence number transmitted to the lower layers among those NR PDCP PDUs received from the node hosting the NR PDCP entity. Once the ‘Highest transmitted NR PDCP Sequence Number’ is reported to the node hosting the NR PDCP entity, the corresponding node may remove the respective NR PDCP PDUs after transmitting to lower layers. In case of RLC AM, the corresponding node reports the highest NR PDCP PDU sequence number successfully delivered to the UE in DDDS. Once the node hosting the NR PDCP entity receive the DDDS, the node hosting the NR PDCP entity and the corresponding node are allowed to remove the respective NR PDCP PDUs.

The existing flow control mechanism is designed on data radio bearer granularity. For MBS service, RAN3 has the agreement/assumption that flow control should be enabled for broadcast MRB and multicast MRB with shared F1-U tunnel. Yet there are ambiguity on DDDS should be per DU level or per cell level. RAN3 should make the decision on that:

- Option 1: shared F1-U tunnel is per cell, and DDDS is also per cell level.

- Option 2: shared F1-U tunnel is per DU, and DDDS is also per DU level.

In the last meeting, most companies support option 1, since per cell-level flow control is more accurate. So, at least, it is fine to support per cell shared F1-U tunnel and flow control as baseline.

For option 2, the CU sends one piece of data to the DU, which consumes little bandwidth. What’s more, RAN#93-e has agreed for configurable scrambling sequence initialization and DMRS sequence generator initialization for PDCCH/PDSCH, and configuring TRS as QCL sources, in order to enable support of intra-DU SFN for broadcast transmission. For intra-DU SFN scenario, the transmission progress of different cells are shared. Therefore, the Option2 may also can be supported to adopt per DU-level flow control.

Proposal 1: For MRB, per cell level flow control applies as baseline, and per DU level flow control can also apply.

## 2.2 Flow control for broadcast session

In the previous RAN2 & RAN3 meeting, there were agreements:

- RAN2: only PTM is applied by broadcast service;

- RAN3: Flow control should be enabled for an MRB established for a broadcast MBS session.

Thus, the broadcast service only supports RLC-UM mode. As common understand, for broadcast service, all of the UEs that are receiving a broadcast service via MRB receives a single copy of each MBS data packet over radio. There is no RLC feedback from UE, the delivery status of all UEs of the MRB shall be same. So we do not see the need for per UE flow control.

In detail, the transmitted status of the packets should be sent to CU for broadcast session, the legacy DDDS frame format as bellow can be reused for broadcast, as CU only needs to obtain the transmitted status via PTM transmission once. After the “Highest transmitted NR PDCP Sequence Number” of the MRB is reported from the DU to the CU, the DU could remove the respective PDCP PDUs.The DDDS of MRB only needs to be sent to the CU via the shared tunnel in the granularity of cell or DU rather than UE. In per DU level F1 tunnel scenario, the reported value should be the smallest sequence number among “Highest transmitted NR PDCP Sequence Number” of the cells within the DU.

| Desired buffer size for the data radio bearer | 4 |
| --- | --- |
| Desired Data Rate | 0 or 4 |
| Highest transmitted NR PDCP Sequence Number | 0 or 3 |

Observation 1. For a broadcast session, the existing PDU Type 0 and Type 1 can be reused for the MRB, in per cell or DU granularity.

## 2.3 Flow control for multicast session

In the last RAN2 meeting, there was agreements that “In RRC signalling, one MRB can be configured with PTM only or PTP only or both PTM and PTP”.  Then, we discuss the flow control mechanisms for different type of MRB separately.

- MRB configured with PTM leg only

For multicast, the RLC-UM mode is allowed for PTM transmission only. Similar to flow control for broadcast mentioned above, CU only need to obtain the transmitted status of “MRB configured with PTM leg only”. Thus the legacy DDDS frame format can be reuse for “MRB configured with PTM leg only”. After the “Highest transmitted NR PDCP Sequence Number” of the MRB is reported in per cell or DU granularity from the DU to the CU, the DU could remove the respective PDCP PDUs.

Observation 2. For a multicast MRB configured with PTM leg only, the existing PDU Type 0 and Type 1 can be reused for the MRB, in per cell or DU granularity.

- Split MRB with a common PDCP

In the previous RAN3 meeting, there was agreement that “A shared F1-U tunnel is established between gNB-CU and gNB-DU for PTP/PTM transmission associated with split MRB with common PDCP.” So we can infer that the PTM and PTP transmission share the same MBS date buffered in the CU. So the per UE DDDS is not necessary as CU may remove the buffered NR PDCP PDUs until received DDDSs for all UEs. For multicast service, as the PTP/PTM transmissions for different UEs might be different, per UE DDDS will increase the complexity significantly. And another reason is that the DU decides to use the PTP/PTM transmission for each PDCP PDUs, and DU does not notify the CU about the DUs (PTP/PTM) decision. If per UE DDDS is reported to the CU, the CU might be confused about whether the respective PDCP PDUs was transmitted or delivered to all UEs belongs to the corresponding MBS session. So for a multicast session, flow control should be per cell or per DU level, i.e., DU sends a converged DDDS to the CU for an MRB, in per cell or per DU granularity.

Observation 3. For a split MRB with a common PDCP, the flow control should also be supported in per cell or per DU granularity, by reusing existing PDU Type 0 and Type 1.

In the last RAN3 meeting, there was agreement that “WA: For split MRB with common PDCP, shared F1-U tunnel is used, existing NR user plane protocol functions need to be reviewed for their applicability for MBS.”

The usage of the PDCP SN provided in DDDS, includes:

- CU may calculate the starting point of data transmission in F1, according to the PDCP SN.

- CU is allowed to remove the buffered NR PDCP PDUs of a RLC AM bearer, according to the PDCP SN.

- DU removes the buffered NR PDCP PDUs for both RLC UM and RLC AM, according to the PDCP SN.

Figure 1: Flow control mechanism for split MRB.

For flow control of split MRB, the DU should send a converged DDDS via the shared F1-U tunnel in order to remove the buffered PDUs in CU or DU and provide the assistance information of starting point of data transmission. As shown in Figure 1, the DU should report the flow control taking the delivery status of all legs into account. It is needed to clarify how to provide the PDCP PDU SN to achieve the purpose of DDDS.

To remove the buffered data in DU, the data which have already been transmitted via all legs to all UEs belongs to the MBS session can be removed. Thus, the lowest transmitted status of different RLC legs, i.e. L in figure 1, should be reported. For details, considering that the PTP leg could be either RLC UM or RLC AM, we would like to discuss the details of frame format for two cases:

Case 1: For split MRB configured with PTM-UM leg and PTP-UM leg(s)

For this case, it should be clarified that the “Highest transmitted NR PDCP Sequence Number” should be used to indicate the data transmission status via the PTM-UM leg and the PTP-UM legs to all UEs belongs to the MBS session, and the data packets taken into account should be the ones which have already been transmitted via the PTM leg and all the PTP-UM legs to all UEs.

Observation 4. For a split MRB with PTM-UM leg and PTP-UM leg(s), the existing field designed for RLC UM mode in DDDS can be reused with some clarifications.

Case 2: For split MRB configured with at least one PTP-AM leg

For this case, there could be multiple types of legs at the same time, i.e. PTM UM leg, PTP UM leg, PTP AM Leg, there are two possible ways to report both the transmitted status of the RLC-UM leg(s) and the successfully delivered status of the RLC-AM leg(s):

- Solution 1: Reuse the existing fields in DDDS

As the CU is not aware of which packets are transmitted via PTP or PTM, it is impossible to reuse both the existing fields designed for RLC-UM and the fields designed for RLC-AM to support this case2. Therefore, in this solution, we either reuse the existing fields designed for RLC-UM, or reuse the existing fields designed for RLC-AM.

Considering of the PTP-AM leg, it is preferred to reuse the fields designed for RLC-AM to support the DDDS in case 2. And for the existing fields reused for this kind of MRB, it is needed to clarify that it indicates the status for both the PTM-UM leg and PTP-AM leg(s) to all UEs belongs to the MBS session.

- Solution 2: Add new field in DDDS

In this solution, it is needed to add a new field to indicate the transmitted/delivered status for all UEs, such as “Highest transmitted or successful delivery NR PDCP Sequence Number”, and it is needed to clarify that it indicates the status for both the PTM-UM leg and PTP-AM leg(s) to all UEs belongs to the MBS session.

Considering the simplicity of the frame format, we prefer to reuse the existing fields designed for RLC AM mode, i.e. “Highest successfully delivered NR PDCP Sequence Number”, with some clarifications.

Observation 5. For a split MRB with at least one PTP-AM leg, the existing fields designed for RLC AM mode in DDDS can be reused with some clarifications.

Based on these discussion, it could be understood that the existing PDU Type 0 and Type 1 can be reused for broadcast PTM MRB, and for multicast MRB with PTM leg, the ‘Highest transmitted/delivery NR PDCP Sequence Number’ could be reused to indicate the Highest transmitted/delivery NR PDCP Sequence Number of the slowest UE within the cell or the DU.

Proposal 2: Reuse existing PDU Type 0 and Type 1 to support flow control for multicast and broadcast. For MRB with PTM leg, the ‘Highest transmitted/delivery NR PDCP Sequence Number’ could be reused to indicate the Highest transmitted/delivery NR PDCP Sequence Number of the slowest UE within the cell or the DU.

Proposal 3: in case there is no PTP-AM leg, reuse the ‘Highest transmitted NR PDCP Sequence Number’, in case there is PTP-AM leg, reuse ‘Highest delivery NR PDCP Sequence Number’

For another question, different RLC legs may be configured to different DRX parameters. So, the lowest transmitted status of all RLC legs may limit the fastest transmission procedure of correlative RLC leg. The DU cannot obtain the required data in time. To deal with that, there are some options:

- Solution1: The CU sends related data based on the implementation.

As the CU knows which packets have been sent via the F1 tunnel, the CU can send data starting from the last NR PDCP PDU SN that has been transmitted to the DU.

- Solution2: extend the buffer size for common buffer in DU

If the buffer size of DU is extended, the CU can send more data for different RLC legs.

- Solution3: Additional feedback the fastest PDCP PDU SN

The DU may report the fastest transmission status of all RLC leg to the CU. Based on this, the CU can send data starting from the fastest NR PDCP PDU SN that the DU reported.

Proposal 4. Further discuss how to avoid affecting the transmission of RLC leg with faster transmission progress.

- MRB configured with PTP leg only

For an MRB configured with PTP leg only, which type of F1 tunnel to be used should be discussed first. There are two options about F1 tunnel for an MRB configured with PTP leg only:

- shared F1 tunnel

- UE dedicated F1 tunnel

If shared F1 tunnel is used, the flow control for an MRB configured with PTP leg only is quite similar to the flow control for a split MRB.

If dedicated F1 tunnel is used, similar to flow control for DRB of unicast, the flow control for PTP-MRB can be UE specific. Different UE can feedback UE specific DDDS to CU for flow control. Thus, the legacy flow control can be reused for a multicast MRB with PTP leg only, in per UE granularity.

Proposal 5. Further discuss whether shared F1 tunnel or dedicated F1 tunnels should be used for a multicast MRB configured with PTP leg only. If shared tunnel is used, per DU per MRB flow control applies, if per UE tunnels are used, per UE per MRB flow control applies.

3. Conclusion and Proposals

In this contribution, we discussed how to support flow control for MRBs, including both broadcast and multicast services, get the following proposals:

Proposal 1: For MRB, per cell level flow control applies as baseline, and per DU level flow control can also apply.

Proposal 2: Reuse existing PDU Type 0 and Type 1 to support flow control for multicast and broadcast. For MRB with PTM leg, the ‘Highest transmitted/delivery NR PDCP Sequence Number’ could be reused to indicate the Highest transmitted/delivery NR PDCP Sequence Number of the slowest UE within the cell or the DU.

Proposal 3: in case there is no PTP-AM leg, reuse the ‘Highest transmitted NR PDCP Sequence Number’, in case there is PTP-AM leg, reuse ‘Highest delivery NR PDCP Sequence Number’

Proposal 4. Further discuss how to avoid affecting the transmission of RLC leg with faster transmission progress.

Proposal 5. Further discuss whether shared F1 tunnel or dedicated F1 tunnels should be used for a multicast MRB configured with PTP leg only. If shared tunnel is used, per DU per MRB flow control applies, if per UE tunnels are used, per UE per MRB flow control applies.

The corresponding TP of proposal 1, 2, 3 is provided in section 4.

5. Text proposal to TS 38.425

--------------------------------Start of the Change-----------------------------

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MBS	Multicast Broadcast Service

MR-DC	Multi-RAT Dual Connectivity

MRB	MBS Radio Bearer

--------------------------------Start of the next Change-----------------------------

5.4.2	Downlink Data Delivery Status

5.4.2.1	Successful operation

The purpose of the Downlink Data Delivery Status procedure is to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the downlink user data flow via the corresponding node for the respective data radio bearer. The corresponding node may also transfer uplink user data for the concerned data radio bearer to the node hosting the NR PDCP entity together with a DL DATA DELIVERY STATUS frame within the same GTP-U PDU.

The Downlink Data Delivery Status procedure is also used to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the successful delivery of DL control data to the corresponding node.

When the corresponding node decides to trigger the feedback for Downlink Data Delivery procedure it shall report as specified in section 5.2:

a)	in case of RLC AM, the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs;

in case of split MRB configured with RLC AM PTP leg and RLC UM PTM leg, the highest NR PDCP PDU sequence number successfully delivered in sequence to all the involved UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs, and in this case the NR PDCP PDUs transmitted via RLC UM PTM leg are also reported as successfully delivered to all the involved UEs;

b)	the desired buffer size in bytes for the concerned data radio bearer;

c)	optionally, the desired data rate in bytes associated with a specific data radio bearer configured for the UE;

d)	the NR-U packets that were declared as being "lost" by the corresponding node and have not yet been reported to the node hosting the NR PDCP entity within the DL DATA DELIVERY STATUS frame;

e)	if retransmission NR PDCP PDUs have been delivered, the NR PDCP PDU sequence number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs successfully delivered to the UE in sequence of NR-U sequence number;

f)	if retransmission NR PDCP PDUs have been transmitted to the lower layers, the NR PDCP PDU sequence number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs transmitted to the lower layers in sequence of NR-U sequence number;

g)	the highest NR PDCP PDU sequence number transmitted to the lower layers among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs;

NOTE:	If a deployment has decided not to use the Transfer of Downlink User Data procedure, d), e) and f)  above are not applicable.

e)	in case of RLC AM, the NR PDCP PDU sequence number successfully delivered out of sequence to the UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs.

As soon as the corresponding node detects the successful RACH access by the UE for the corresponding data radio bearer(s), the corresponding node shall send initial DL DATA DELIVERY STATUS frame to the node(s) hosting the NR PDCP entity(ies). The node hosting NR PDCP entity may start sending DL data before receiving the initial DL DATA DELIVERY STATUS frame. In case the DL DATA DELIVERY STATUS frame is sent before any NR PDCP PDU is transferred to lower layers, the information on the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE and the highest NR PDCP PDU sequence number transmitted to the lower layers may not be provided.

The DL DATA DELIVERY STATUS frame shall also include a final frame indication when this frame is the last DL status report. When receiving such indication, the node hosting the NR PDCP entity considers that no more UL or DL data is expected to be transmitted between the corresponding node and the UE.

The DL DATA DELIVERY STATUS frame may also include an indication of detected radio link outage or radio link resume for the concerned data radio bearer. When receiving an indication of radio link outage detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is unavailable at the corresponding node both in UL and DL. When receiving an indication of radio link resume detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is available at the corresponding node both in UL and in DL. When receiving an indication of UL or DL radio link outage detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is unavailable at the corresponding node for UL or DL, depending on the indicated outage. When receiving an indication of UL or DL radio link resume detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is available at the corresponding node in UL or in DL, depending on the indicated resume.

The node hosting the NR PDCP entity, when receiving the DL DATA DELIVERY STATUS frame:

-	regards the desired buffer size under b) and the data rate under c) above as the amount of data to be sent from the hosting node:

-	If the value of the desired buffer size is 0, the hosting node shall stop sending any data per bearer.

-	If the value of the desired buffer size in b) above is greater than 0, the hosting node may send up to this amount of data per bearer starting from the last "Highest successfully delivered NR PDCP Sequence Number" for RLC AM if received, or the hosting node may send up to this amount of data per bearer starting from the last "Highest transmitted NR PDCP Sequence Number" for RLC UM if received.

-	The value of the desired data rate in c) above is the amount of data desired to be received in a specific amount of time. The amount of time is 1 sec.

-	The information of the buffer size in b) above and of the data rate in c) above is valid until the next DL DATA DELIVERY STATUS frame is received.

-	is allowed to remove the buffered NR PDCP PDUs of a RLC AM bearer, or the buffered NR PDCP PDUs of a split MRB configured with RLC AM PTP leg and RLC UM PTM leg, according to the feedback of successfully delivered NR PDCP PDUs;

-	decides upon the actions necessary to take for NR PDCP PDUs reported other than transmitted and/or successfully delivered.

In case of RLC AM, after the highest NR PDCP PDU sequence number successfully delivered in sequence is reported to the node hosting the NR PDCP entity, the corresponding node removes the respective NR PDCP PDUs. For RLC UM, the corresponding node may remove the respective NR PDCP PDUs after transmitting to lower layers.

Figure 5.4.2.1-1: Successful Downlink Data Delivery Status

--------------------------------Start of the next Change-----------------------------

5.5.3.18	Highest Delivered NR PDCP SN Ind

Description: This parameter indicates the presence of Highest successfully delivered PDCP Sequence Number.

Value range: {0= Highest successfully delivered PDCP Sequence Number not present, 1= Highest successfully delivered PDCP Sequence Number present}.

Field length: 1 bit.

5.5.3.19	Highest successfully delivered NR PDCP Sequence Number

Description: This parameter indicates feedback about the in-sequence delivery status of NR PDCP PDUs at the corresponding node towards the UE. For a split MRB configured with RLC AM PTP leg and RLC UM PTM leg, this parameter indicates the feedback about the delivered status of NR PDCP PDU sequence at the corresponding node to all the UEs belongs to the MBS session via PTP leg  and PTM leg.

Value range: {0..218-1}.

Field length: 3 octets.

5.5.3.20	Highest Transmitted NR PDCP SN Ind

Description: This parameter indicates the presence of the Highest transmitted NR PDCP Sequence Number.

Value range: {0= Highest transmitted NR PDCP Sequence Number not present, 1= Highest transmitted NR PDCP Sequence Number present}.

Field length: 1 bit.

5.5.3.21	Highest transmitted NR PDCP Sequence Number

Description: This parameter indicates the feedback about the transmitted status of NR PDCP PDU sequence at the corresponding node to the lower layers, for a data radio bearer, or a MRB configured with PTM leg only, or a split MRB configured with RLC UM PTP leg and RLC UM PTM leg.

Value range: {0..218-1}.

Field length: 3 octets.

--------------------------------End of the Changes-----------------------------