3GPP TSG-RAN WG3 Meeting #114e	R3-215468

# Online Meeting, 1st – 12th November 2021

Agenda Item:	9.3.6.1

Source:	Ericsson

Title: 	Voice EPS Fallback and VoNR

Document for:	Discussion and Approval

# 1		Information

Redirection for Voice EPS Fallback is used to indicate from AMF to NG-RAN node, to support various deployment scenarios for obtaining IMS voice service, that the UE and NG-RAN may support the mechanism to direct or redirect the UE from NG-RAN either towards E-UTRA connected to 5GC (RAT fallback) or towards EPS (E-UTRAN connected to EPC System fallback).

When NG-RAN node supports IMS voice, it is up to NG-RAN node to decide if to setup VoNR, or perform EPS Fallback, when such indication is set to possible.

# 2		Discussion

In the some scenario, for example, roaming, the operator may desire to disallow roamers to setup VoNR. It may want to steer the roamers to the EPC network.

Currently in NGAP, the Redirection for Voice EPS Fallback only indicate if the EPS Fallback is possible or not.

The IE can be extended for a addition code point to indicate that “voice fallback to EPC is possible but VoNR is not”.

#### 9.3.1.116 Redirection for Voice EPS Fallback This IE is used to indicate that the AMF and the UE support the redirection for voice for EPS Fallback. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Redirection for Voice EPS Fallback; M; ENUMERATED (possible, not-possible, …).

With the extended code point, there would be no backwards compatibility issue. The existing code point “possible” and “not possible” are used, as today, to indicate if the voice EPS fallback is possible or not. The VoNR allowed or not is unspecified, and it is totally up to NG-RAN node to decide.

Wit the third code point, “voice EPS Fallback possible, VoNR not possible”, NG-RAN node, if supporting IMS voice, will not setup VoNR, but instead using Fallback mechanism.

Proposal 1: It is proposed that RAN3 to agree to add one code point in Redirection for Voice EPS Fallback to indicate that when voice EPS fallback is possible and VoNR is not allowed.

The CR is submitted in [1] and [2].

# 3		Proposal

Proposal 1: It is proposed that RAN3 to agree to add one code point in Redirection for Voice EPS Fallback to indicate that when voice EPS fallback is possible and VoNR is not allowed.

# 4		Reference

[1]		R3-215469		Voice EPS Fallback and VoNR, Ericsson

[2]		R3-215470		Voice EPS Fallback and VoNR, Ericsson