3GPP TSG-RAN3 Meeting # 114bis-e	   R3-220307

# 17th - 26th January 2022

Agenda Item:	30.3

Source:	Ericsson, Qualcomm Inc.

Title: 	Addition of UE Subgrouping for paging

Document for:	Discussion

# Introduction

The WID of UE Power Saving enhancement has been revised to include RAN3 as impacted group [1]. From the WID [2] we can observe below the new modified bullet:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1, RAN3] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting

According to the endorsed TU schedule in [3], the UE Power saving work will start in Q1 2022 in RAN3#114bis-e meeting.

Previously, RAN3 received a LS from RAN2 in [4]. Further agreements have been communicated in a new RAN2 LS in [5] with an action to RAN:

To RAN3: RAN2 respectfully asks RAN3 to take the above information into account for their future work, and provide further information on the following issue once concluded: - Signalling between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE, e.g., to convey Subgroup ID, UE capability, etc.

RAN3 has also received a LS from SA2 with their Stage 2 agreements [6] impacting NG-AP signalling.

In this contribution, we discuss about the scope of the WI in RAN3 and provide CRs to update our specifications based on the above other WGs liaisons.

# Discussion

From the RAN2 liaison in [4-5], we can make the following observations:

- Two subgrouping approaches have been agreed by RAN2 for the UE power saving: a CN-assigned subgrouping and a UE-ID based one.

- The UE-ID (I-RNTI) approach has no CN impacts and is fully within RAN2 scope. No RAN3 impacts are foreseen for this approach.

- The CN-based subgrouping approach, on the other hand, relies on NAS signalling between UE and AMF and between AMF and NG-RAN, as described in the SA2 LS [6]:

Based on the reported UE capabilities, the AMF can determine the assistance information for NR Paging Subgroup according to the NR UE characteristics, i.e. local configuration, subscription information and/or statistical information. If the AMF has determined assistance information for Paging Subgroup, the AMF shall deliver the information to the UE during Registration and the NG-RAN in the NGAP Paging Message or RRC Inactive Assistance Information for support of paging a NR UE in CM_IDLE or CM_CONNECTED with RRC Inactive state.

We can note that RAN2 have agreed that up to maximum 8 groups can be configured by OAM to be used in the deployment. From the RAN2 LS [5]:

“The total number of CN-assigned subgroups that is used is not fixed and can be configured up to 8 (e.g. by OAM). No impact on signalling is assumed.”

This is the same as GWUS, where the number of Paging Probabilities groups is also set up via OAM and assumed to not change frequently.

Observation 1: RAN2 have agreed that up to maximum 8 groups can be configured by OAM to be used in the deployment. No impact on signalling is assumed.

Based on the above understanding, we propose to consider the following signalling-chart for supporting Paging Subgrouping in NG-RAN where the proposed changes are marked in red:

Fig. 1: Overall signalling chart to support CN-based paging subgrouping in NG-RAN (RAN3 changes are marked in red)

From the above, some enhancements are needed to support the CN-based grouping in RAN3. It is proposed to enhance the following messages and IEs, based on the SA2 agreed naming in [5]:

Proposal 1: To support RAN paging with subgrouping for a UE in RRC_Inactive:

- Add the Paging Early Indication with Paging Subgrouping (PEIPS) Assistance Information in the NGAP CN Assistance Information for RRC Inactive IE

- Add of the PEIPS Assistance Information in the F1 Paging message.

Proposal 2: To support CN paging with subgrouping for a UE in RRC_idle:

- Add of the PEIPS Assistance Information in the NGAP Paging message

- Add of the PEIPS Assistance Information in the F1 Paging message.

Proposal 3: The PEIPS Assistance Information includes the Paging Subgroup ID, up to 8.

Note that RAN2 have agreed to introduce a UERadioPagingInfo IE in the RRC UECapabilityInformation message in NR in Rel-17. Therefore, the NG-RAN can know the UE’s support of subgrouping in the UE Radio Capability for Paging IE sent during Paging and Initial Context Setup.

Observation 2: UE’s paging Subgrouping Support Indication is included in the UE Radio Capability for Paging IE

Regarding XnAP impacts, it is proposed to wait for a decision on whether the Subgrouping information transmitted within the PEI is going to be used only in the last visited cell ID, or in all cells to be paged. In the first case, the anchor node should not need to provide the subgroup information in the Xn RAN Paging message. It is proposed to wait for RAN2 progress on this aspect before impacting XnAP.

Proposal 4: RAN3 to wait for RAN2 progress before agreeing XnAP impacts for Subgrouping

A CR to NG-AP and F1AP are proposed in [7] and [8], respectively.

A draft CR to TS 38.300 is proposed in [9].

# Conclusions and Proposal

In this contribution, we presented in a high-level view the RAN3 specification impacts to support CN-subgrouping in NG-RAN. It is proposed to take this contribution as baseline for discussion.

Proposal 1: To support RAN paging with subgrouping for a UE in RRC_Inactive:

- Add the Paging Early Indication with Paging Subgrouping (PEIPS) Assistance Information in the NGAP CN Assistance Information for RRC Inactive IE

- Add of the PEIPS Assistance Information in the F1 Paging message.

Proposal 2: To support CN paging with subgrouping for a UE in RRC_idle:

- Add of the PEIPS Assistance Information in the NGAP Paging message

- Add of the PEIPS Assistance Information in the F1 Paging message.

Proposal 3: The PEIPS Assistance Information includes the Paging Subgroup ID, up to 8.

Proposal 4: RAN3 to wait for RAN2 progress before agreeing XnAP impacts for Subgrouping

Observation 1: RAN2 have agreed that up to maximum 8 groups can be configured by OAM to be used in the deployment. No impact on signalling is assumed.

Observation 2: UE’s paging Subgrouping Support Indication is included in the UE Radio Capability for Paging IE

A CR to NG-AP and F1AP are proposed in [7] and [8], respectively.

A draft CR to TS 38.300 is proposed in [9].

# References

R3-214281, LS on RAN3 work associated with UE Power Saving, RAN3

RP-212630, Revised WID: UE power saving enhancements for NR

RP-212607, Updated RAN3 time planning, Endorsed

R2-2108917, LS on UE Power Saving, RAN2

R2-2111415, LS on paging subgrouping and PEI

R3-215795, LS Reply on UE Power Saving, SA2

R3-220308, Addition of PEIPS Assistance Information to NG-AP, Ericsson, Qualcomm inc.

R3-220287, Addition of PEIPS Assistance Information to F1-AP, Qualcomm inc., Ericsson

R3-220286, Addition of PEIPS Assistance Information to TS 38.300, Qualcomm inc., Ericsson