3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-220160

E-meeting, 17 – 26 January 2022

Agenda item:	10.2.1.4

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP to SON BL CR to 38.423, NR_ENDC_SON_MDT_enh]  Completion of the work on the SCG UHI

Document for:	Endorsement

# 1	Introduction

At the RAN3 #114-bis meeting, the work needed to be done to complete the SCG UHI solution has been discussed in [R3-220159]. In this paper, the proposed changes for the XnAP BL CR are presented.

# 2	Text proposal to the BL CR to TS 38.423

The proposal below is based on the last endorsed BL CR [R3-216255]. ASN.1 will be added once the changes are agreed.

### First Change

#### 9.2.3.64	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [12].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [5].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >Last Visited Cell Information | M |  | 9.2.3.65 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |

============ Next Change ==============

#### 9.2.3.X	Last Visited PSCell Information

The Last Visited PSCell Information contains information related to the stay in the PSCell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | M |  | NG-RAN Cell Identity 9.2.2.9 |  |
| Time of cell entry | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. |

### End of Changes