3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-220577

# Online, Jan 17th – Jan 26th 2022

Agenda Item:	10.2.1.5

Source:	Ericsson

Title:	(TP for SON BL CR for TS 36.423) MLB enhancements

Document for:	Discussion, Decision

# 1	Introduction

This paper describes a sample TP for SON Baseline CR for TS 36.423 for MLB enhancements, according to the proposals in [1].

# Reference

- R3-220576 (TP for SON BL CR for TS 38.423) MLB enhancements

# Appendix A - Sample TP to X2AP (TS 36.423) for MLB

### 8.3.6	Resource Status Reporting Initiation

#### 8.3.6.1	General

This procedure is used by an eNB to request the reporting of load measurements to another eNB.

The procedure uses non UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from eNB1 to eNB2. Upon receipt, eNB2:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	if supported, stop cell measurements and terminate the reporting for cells indicated in the Cell To Report IE list, in case the Registration Request IE is set to "partial stop"; or

-	if supported, add cells indicated in the Cell To Report IE list to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add".

If the eNB2 received a RESOURCE STATUS REQUEST message, which includes the Registration Request IE set to "stop", the Cell To Report IE list shall be ignored.

If the Registration Request IE is set to "start" then the Report Characteristics IE shall be included in RESOURCE STATUS REQUEST message. The eNB2 shall ignore the Report Characteristics IE, if the Registration Request IE is not set to "start".

The Report Characteristics IE indicates the type of objects eNB2 shall perform measurements on. For each cell, the eNB2 shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the S1 TNL Load Indicator IE, if the second bit, "TNL Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Hardware Load Indicator IE, if the third bit, "HW Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the fourth bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE;

-	the ABS Status IE, if the fifth bit, "ABS Status Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1 and eNB1 had indicated the ABS pattern to eNB2;

-	the RSRP Measurement Report List IE, if the sixth bit, "RSRP Measurement Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the CSI Report IE, if the seventh bit, "CSI Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

-	the Reported Neighbor Cells List IE, if the eight bit, "Neighbor Cells Reporting" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the time interval between two subsequent RESOURCE STATUS UPDATE messages that include the Radio Resource Status IE, S1 TNL Load Indicator IE, Hardware Load Indicator IE, Composite Available Capacity Group IE, or ABS Status IE.

If the Reporting Periodicity of RSRP Measurement Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the RSRP Measurement Report List IE.

If the Reporting Periodicity of CSI Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the CSI Report IE.

If eNB2 is capable to provide all requested resource status information, it shall initiate the measurement as requested by eNB1, and respond with the RESOURCE STATUS RESPONSE message.

If eNB2 is capable to provide some but not all of the requested resource status information and the Partial Success Indicator IE is present in the RESOURCE STATUS REQUEST message, it shall initiate the measurement for the admitted measurement objects and include the Measurement Initiation Result IE in the RESOURCE STATUS RESPONSE message.

If the Minimum Neighbor Composite Available Capacity Downlink IE or the Minimum Neighbor Composite Available Capacity Uplink IE or both are included in the RESOURCE STATUS REQUEST message, eNB2 shall report in the Reported Neighbor Cells List IE only neighbor cells for which downlink Composite Available Capacity (respectively uplink Composite Available Capacity) are above the Minimum Neighbor Composite Available Capacity Downlink IE value (respectively the Minimum Neighbor Composite Available Capacity Uplink IE value).

/////////////////////////////////////// Next Change ///////////////////////////////////////////////

### 8.3.7	Resource Status Reporting

#### 8.3.7.1	General

This procedure is initiated by eNB2 to report the result of measurements admitted by eNB2 following a successful Resource Status Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: Resource Status Reporting, successful operation

The eNB2 shall report the results of the admitted measurements in RESOURCE STATUS UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Resource Status Reporting Initiation procedure, and thus not reported in the Measurement Failed Report Characteristics IE for the concerned cell in the RESOURCE STATUS RESPONSE message.

If the eNB1 receives the RESOURCE STATUS UPDATE message which includes the UE ID IE in the RSRP Measurement Report List IE, the eNB1 may use the UE ID IE to link the associated RSRP measurement report with other measurement results (e.g. CSI reports, RSRP measurement reports) of the same UE.

If the CSI Report IE including the CSI Process Configuration Index IE is received, eNB1 shall interpret this IE as an index identifying one of the CSI process configurations that can be configured for all UEs within the cell where the CSI measurements were collected. For all UEs within the cell, the maximum number of CSI process configurations is given by the maximum value of the CSI Process Configuration Index IE.

If the eNB1 receives the RESOURCE STATUS UPDATE message, which includes the Cell Reporting Indicator IE set to "stop request" in one or more items of the Cell Measurement Result IE, the eNB1 should initialise the Resource Status Reporting Initiation procedure to remove all or some of the corresponding cells from the measurement.

If the eNB1 receives the RESOURCE STATUS UPDATE message, which includes the Reported Neighbor Cells List IE as part of the Cell Measurement Result Item IE, the eNB1 should consider the cells listed in the Reported Neighbor Cells List IE as candidate for UE configuration with resource aggregation with the cell identified by the Cell ID IE in the Cell Measurement Result Item IE.

/////////////////////////////////////// Next Change ///////////////////////////////////////////////

#### 9.1.2.11	RESOURCE STATUS REQUEST

This message is sent by an eNB1 to neighbouring eNB2 to initiate the requested measurement according to the parameters given in the message.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …, partial stop, add) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the eNB2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eigth Bit = Neighbor Cells Reporting. Other bits shall be ignored by the eNB2. | YES | reject |
| Cell To Report |  | 1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Minimum Neighbor Composite Available Capacity Downlink | O |  | INTEGER (0..100) | Indicate minimum Composite Available Capacity Downlink of neighbor cells for reporting. | YES | ignore |
| >>Minimum Neighbor Composite Available Capacity Uplink | O |  | INTEGER (0..100) | Indicate minimum Composite Available Capacity Uplink of neighbor cells for reporting. | YES | ignore |
| Reporting Periodicity | O |  | ENUMERATED(1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Load Ind Periodic, HW Load Ind Periodic, Composite Available Capacity Periodic or ABS Status Periodic. | YES | ignore |
| Partial Success Indicator | O |  | ENUMERATED(partial success allowed, ...) | Included if partial success is allowed | YES | ignore |
| Reporting Periodicity of RSRP Measurement Report | O |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, ...) | Periodicity that can be used for the reporting of RSRP Measurement Report Periodic. | YES | ignore |
| Reporting Periodicity of CSI Report | O |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, ...) | Periodicity that can be used for the reporting of CSI Report Periodic. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add". |

/////////////////////////////////////// Next Change ///////////////////////////////////////////////

#### 9.1.2.14	RESOURCE STATUS UPDATE

This message is sent by eNB2 to neighbouring eNB1 to report the results of the requested measurements.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  |  |  |
| >>S1 TNL Load Indicator | O |  | 9.2.35 |  |  |  |
| >>Radio Resource Status | O |  | 9.2.37 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | YES | ignore |
| >>ABS Status | O |  | 9.2.58 |  | YES | ignore |
| >>RSRP Measurement Report List | O |  | 9.2.76 |  | YES | ignore |
| >>CSI Report | O |  | 9.2.79 |  | YES | ignore |
| >>Cell Reporting Indicator | O |  | ENUMERATED(stop request, ...) |  | YES | Ignore |
| >> Reported Neighbor Cells List |  | 0 .. < maxnoofReportedNeighborCells> |  | List of reported neighbor cells. | YES | ignore |
| >>> Reported Neighbor Cells Item | M |  |  |  | – |  |
| >>>> Target Cell Global ID | M |  | 9.2.x1 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofReportedNeighborCells | Maximum no. of reported neighbor cells for resource aggregation. Value is 8. |

/////////////////////////////////////// Next Change ///////////////////////////////////////////////

### 9.2.x1	Target Cell Global ID

This IE contains either an NR CGI or an E-UTRA CGI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Target Cell | M |  |  |  |
| >NR |  |  |  |  |
| >>NR CGI | M |  | 9.2.111 |  |
| >E-UTRA |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.2.14 |  |

/////////////////////////////////////// End Changes except ASN.1 ///////////////////////////////////////////////

/////////////////////////////////////// Start ASN.1 Changes ///////////////////////////////////////////////

-- 9.3.4	PDU Definitions

id-SFN-Offset,

id-DirectForwardingPathAvailability,

id-sourceNG-RAN-node-id,

id-MinimumNeighborCellsCACDownlink,

id-MinimumNeighborCellsCACUplink,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands,

maxnoofSSBAreas

FROM X2AP-Constants;

/////////////////////////////////////// Next ASN.1 Change ///////////////////////////////////////////////

-- **************************************************************

--

#### -- RESOURCE STATUS REQUEST

--

-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusRequest-IEs}},

...

}

ResourceStatusRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID			CRITICALITY ignore	TYPE Measurement-ID					PRESENCE conditional}|-- The IE shall be present if the Registration Request IE is set to “Stop”, “Partial stop” or to “Add”--

{ ID id-Registration-Request		CRITICALITY reject	TYPE Registration-Request			PRESENCE mandatory}|

{ ID id-ReportCharacteristics		CRITICALITY reject	TYPE ReportCharacteristics			PRESENCE optional}|

{ ID id-CellToReport				CRITICALITY ignore	TYPE CellToReport-List				PRESENCE mandatory}|

{ ID id-ReportingPeriodicity		CRITICALITY ignore	TYPE ReportingPeriodicity			PRESENCE optional}|

{ ID id-PartialSuccessIndicator		CRITICALITY ignore	TYPE PartialSuccessIndicator		PRESENCE optional}|

{ ID id-ReportingPeriodicityRSRPMR	CRITICALITY ignore	TYPE ReportingPeriodicityRSRPMR		PRESENCE optional}|

{ ID id-ReportingPeriodicityCSIR	CRITICALITY ignore	TYPE ReportingPeriodicityCSIR		PRESENCE optional}|

{ ID id-MinimumNeighborCellsCACDownlink		CRITICALITY ignore	TYPE CellToReport-List		PRESENCE optional}|

{ ID id-MinimumNeighborCellsCACUplink	    CRITICALITY ignore	TYPE CellToReport-List		PRESENCE optional},

...

}

/////////////////////////////////////// Next ASN.1 Change ///////////////////////////////////////////////

-- 9.3.5	Information Element definitions

id-TraceCollectionEntityURI,

id-SFN-Offset,

id-AdditionLocationInformation,

id-ReportedNeighborCellsList,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

/////////////////////////////////////// Next ASN.1 Change ///////////////////////////////////////////////

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofTNLAssociations,

maxnoofCellsinCHO,	maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofNRPhysicalResourceBlocks,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofReportedNeighborCells

FROM X2AP-Constants

/////////////////////////////////////// Next ASN.1 Change ///////////////////////////////////////////////

#### -- R

ReportedNeighborCells-List ::= SEQUENCE (SIZE(1..maxnoofReportedNeighborCells)) OF ReportedNeighborCells-Item

ReportedNeighborCells-Item	::= SEQUENCE {

cell-ID					Target-Cell-Global-ID,

iE-Extensions			ProtocolExtensionContainer { { ReportedNeighborCells-Item-ExtIEs} }	OPTIONAL,

...

}

ReportedNeighborCells-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

/////////////////////////////////////// Next ASN.1 Change ///////////////////////////////////////////////

#### -- T

Target-Cell-Global-ID::= CHOICE {

nrCGI				NRCGI,
eCGI				ECGI,
...

}

/////////////////////////////////////// Next ASN.1 Change ///////////////////////////////////////////////

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofPC5QoSFlows 							INTEGER ::= 2048

maxnoofSSBAreas								INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofNRPhysicalResourceBlocks				INTEGER ::= 275

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofReportedNeighborCells						INTEGER ::= 8

/////////////////////////////////////// Next ASN.1 Change ///////////////////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-IMSvoiceEPSfallbackfrom5G												ProtocolIE-ID ::= 408

id-AdditionLocationInformation												ProtocolIE-ID ::= 409

id-DirectForwardingPathAvailability											ProtocolIE-ID ::= 410

id-sourceNG-RAN-node-id														ProtocolIE-ID ::= 411

id-MinimumNeighborCellsCACDownlink											ProtocolIE-ID ::= x01

id-MinimumNeighborCellsCACUplink											ProtocolIE-ID ::= x02

id-ReportedNeighborCellsList												ProtocolIE-ID ::= x03

END

-- ASN1STOP

/////////////////////////////////////// End of ASN.1 Changes ///////////////////////////////////////////////