3GPP TSG-RAN WG3 #117bis-e	R3-225959

Online, 10. – 18. Oct 2022

Agenda Item:	14.4

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on Selective Activation (CB #MobilityEnh3_Others)

Document for:	Approval

# Introduction

CB: # MobilityEnh3_Others

- Discuss on NR-DC with selective activation, and focus on high level topics.

- Feasibility of simultaneous data transmission from the UPF to the group of nodes prepared for selective activation.

- SCG selective activation is prioritized in Rel-18?

- The source MN or the source SN can initiate the procedure for SCG selective activation?

- Supporting CPC/CPA configuration including intra-SN CPC configuration for the reduction of coordination and overhead?

- Other issues related with selective activation.

- Coexistence between Rel-17 CPAC and Rel-18 CPAC.

- Capture the agreements and open issues

# For the Chairman’s Notes

RAN3 considers the Inter-CU and Intra-CU cases with equal priority, and studies both the F1 and Xn signaling aspects. It can be revisited based on RAN2 progress. [last meeting’s WA turned into agreement]

WA: RAN3 will work to enable both indirect and direct early data forwarding in Selective Activation. At this moment, RAN3 does not foresee any scenarios where direct forwarding is not feasible/desired.

WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation.

WA: Enhance signalling for Selective Activation.

At this moment, RAN3 does not exclude any scenario for Selective Activation (for example CPA, MN-initiated CPC, SN-initiated CPC). RAN3 discussed following two sub-scenarios for Selective Activation and will further check their applicability/feasibility (the progress is also up to RAN2):

- Keeping PSCells prepared for CPA after UE accesses one of them;

- Keeping PSCells prepared for Selective Activation after DC operation is released so that they can be used for subsequent CPA (the cells prepared for Selective Activation may be prepared by the MN or the SN).

Any optimisations to the signalling are FFS. Proposals made so far:

- activation/deactivation of groups of PSCells prepared for Selective Activation

- parallel data transmission from the UPF to all prepared PSCells

# Discussion (2nd round)

## Scenarios: CPA

It has been raised in the 1st round that Selective Activation for CPA may have two flavours:

- Keeping PSCells prepared for CPA after UE accesses one of them;

- Keeping PSCells prepared for Selective Activation after DC operation is released so that they can be used for subsequent CPA (the cells prepared for Selective Activation may be prepared by the MN or the SN).

Question 9: Please, comment if any of the operations above should be postponed (or excluded) from RAN3 work.

| Company | Operation to be postponed | Possible comment |
| --- | --- | --- |
| Nokia | None, but depends on RAN2 | We agreed to have this discussed, but after a consideration, scenario (2) depends also on UE capability to keep conditional configuration after SCG release. So, we can’t really decide it in RAN3 alone. |
| ZTE | Depends on RAN2 | The target of selective activation is to save inter-node signalling and UE RRC signalling after the initial CPA execution. So that, the other candidate SN shall be kept after initial CPA execution, it will be changed to MN initiated CPC. However, RAN2 will consider the Full configuration at the candidate SN during CPA will be changed to delta configuration at the candidate SN during CPC. |
| Lenovo | 2) | In our view 2) implies frequent switch between stand alone and DC scenario, which has much higher complexity e.g., UE needs to add and release SCG bearers frequently and may also adapt MCG configuration frequently as well. Not sure about the benefit/gain. We agree it is upon RAN2 to finally decide. We must point out that according to the scenario discussion in the last two meetings, it seems RAN3 is not the right working group to exclude any possible scenario, since it’s upon RAN2 to finally decide. For the same reason, it will be very strange for RAN3 to decide work on a specific scenario without RAN2’s confirmation. What if RAN2 decides to exclude some scenario after RAN3 starts working on it? According to our observation, RAN2 will not progress much on selective activation in this meeting neither. So either RAN3 keeps making working assumptions and upon RAN2 decision, or RAN3 sends LS to RAN2 asking their view explicitly. |
| Intel | Depends on RAN2 but if have to choose to exclude, then definitely (2) | Agree with Lenovo. (2) seems too far fetched for the possibility of subsequent CPA execution. But anyway up to RAN2 to decide (not RAN3..) |
| E/// | Depends on RAN2, but 2) is a bit far.. | The use case for storing a CPA configuration at cell group changes would correspond to that the cell group change is for the MCG, since a CPA configuration is not relevant when the UE has an SCG configured. Then considering the work in Rel-18 is targeting at the cases of frequent SCG changes when operating FR2, we even do not see the need for keeping CPA configurations. |
| CATT | Depends on RAN2, The 2) can be postponed. | Agree with above analysis on the two scenarios. Anyway we cannot decide preclude cases by RAN3 only. But the 2) looks too far from WI scope |
| China Telecom | Depends on RAN2. | We think the scenario should be discussed and decided by RAN2, we can send LS to RAN2 asking their view if needed. |
| NTT DOCOMO | None | We think RAN3 could consider both cases 1) and 2) as possible cases of applicable scenario. Which case should be postponed is up to RAN2 decision. However, it seems that there is no critical issues to support both 1) and 2) from RAN3 aspect. |
| Huawei | 2), but | Up to RAN2 |
| NEC | 2), this is too far | We are in the understanding that the selective activation is for the subsequent CPC rather than subsequent CPA. But will depend on RAN2 work. |
| Samsung | Depends on RAN2 | Configurations of the candidate cells for CPA stored after UE accesses one of them could avoid signalling overhead inter-nodes and Uu. But how to be used for the subsequent CPA or CPC procedure depends on RAN2. |
| Qualcomm | Depends on RAN2 | None of the options need to be excluded now. |
| Google | 2) and depends on RAN2 | We also think that the selective activation is for subsequent CPC rather than CPA. |

## Scenarios: scale of preparation

In the 1st round, it was considered that there may be two use cases for Selective Activation:

- RAN keeps prepared a large number of SNs/PSCells, the group is quasi-stable (i.e. cells are removed or added to the group of prepared cells infrequently), while the UE roams among them. The consequence is that PSCells changes are more frequent than updates of the list of prepared PSCells; also, Selective Activation is more-or-less permanent while the UE is in DC operation.
(Please note, this scenario does not mean automatically that we start working on activation/deactivation of cell groups – this is a question on the scenario only).

- RAN prepares limited group of PSCells that may be immediately needed for the UE. The group is small and may be often updated based e.g. on measurement report from the UE. Selective Activation is a temporary config when UE is close to a cell border, otherwise prepared PSCells may be released.

- Anything else?...

Please note, we shall not rule out any of the scenarios yet. However, perhaps we may have a common understanding what is the primary objective for the solution (also, it’s good to learn all possible use cases for Selective Activation).

Question 10: Please, comment which of the use cases above should be the main scenario for RAN3 – or if both should be addressed.

| Company | Primary use case | Possible comment |
| --- | --- | --- |
| Nokia | 1 (likely) | We tend to consider scenario 1 as more relevant for Selective Activation – scenario 2 can be handled by a classic CPAC mechanism. |
| ZTE | 1 (likely) | I am not sure of the maximum number of the candidate PSCell between the case 1 and case 2. In my view, referred to the legacy, both network and UE shall have the same candidate PSCell configuration and the maximum number can be 8. |
| Lenovo | 2 | According to legacy UE capability, the maximum number of conditional configuration handling is 8. We don’t expect large number of SNs get involved. Otherwise, there will be a large amount of resources reserved/prepared for selective activation for much longer time (compared to legacy CHO/CPAC) which may make the feature less attractive. |
| Intel | 1 (likely) but | According to the WID: To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] the objective is to “allow” subsequent cell group change “without” reconfiguration or re-initiation. Hope we focus on the objective. |
| E/// | ? | The intention of discussing/prioritizing any use case here is unclear. First the number of candidate PSCell could be 8 as legacy, or extended to larger time pending in RAN2. Second how to define “large” or “small” number of SNs/PSCells in the question. If moderator would discuss whether the selective action is relatively static when UE is in DC or whether the config is temporary for the UE, we suggest brining this to RAN2. |
| CATT |  | Both 1 and 2 can use selective SCG mechanism. Considering the resource reservation for CHO/CPAC, the candidate CG number should be limited. |
| China Telecom | 2 | We think we need to limit the candidate PSCell numbers. The status of the network and the position of the UE will change, thus the configurations that remain unchanged for a long time (proposed in option1) do not apply to dynamic networks. In addition, too many candidate cells will lead to high storage pressure in NG-RAN nodes and UEs. |
| NTT DOCOMO | 1 | We prefer to consider case 1 as a objective of selective activation. Still there are several open issues (e.g. security key refresh, reconfiguration of delta config) but if these issues could be solved, theoretically, it seems that there is no strong limitation about maximum number of conditional configuration. And also, limitation of maximum number of the candidates or subsequent CPC will reduce the gain of selective activation. However, we expect that RAN2 involvement is needed if case 1 is considered, and we also think case 2 is good for basic discussion, since it aligns with legacy CHO/CPAC. |
| Huawei | Between 1 and 2 :) | In case of 1, too many candidate SNs will be involved and prepared, which is not costly and not expected. And we do not have to set the set very small like 2 or 3 PSCells, in Rel-17 we limited the PSCells to 8, this seems a balanced scale. |
| NEC | 2 | Agree with Lenovo. |
| Samsung | 1 | The candidate cells might be update frequently with FR2. To avoid RRC signalling overhead for updating the pre-configured cells, NW might pre-configure more candidate cells for UE, from which NW select the cells(e.g. no more than 8) used for the subsequent CPAC procedure. |
| Qualcomm | 1 | 1 is the scenario that seems to be of interest. However, RAN2 needs to decide about the maximum number of conditional configurations (PSCells) that can be configured to a UE (there should be a maximum for similar reasons as in Rel-16/Rel-17, i.e., limitations of UE storage/processing capabilities). |
| Google | 2 | Subject to the maximum number of conditional configuration handling |

## Scenarios: direct / indirect data forwarding

In the 1st round, it was declared that early data forwarding is critical for the Selective Activation. However, applicability of direct or indirect forwarding was put in question.

Indirect data forwarding is possible by default in DC operation, so we understand that it shall be enabled so also for Selective Activation. However, RAN3 normally tries to design the signalling so that direct data forwarding is possible (sometimes even adding dedicated signalling to enable it). Since it requires some effort, the problem is: are there any scenarios where direct data forwarding is not expected to be beneficial?

Question 11: Please, comment if you see any scenario (among those discussed so far) where direct early data forwarding is not beneficial to be enabled.

| Company | Scenario (CPA, MN-CPC, SN-CPC, other?) | Possible comment |
| --- | --- | --- |
| Nokia | None at this stage | Direct data forwarding has always been considered beneficial, so we expect it to be also in the case of Selective Activation. Thus, until some serious technical obstacle is encountered, we prefer to assume it shall not be disabled in any scenario. |
| ZTE |  | Direct data forwarding is always very beautiful, however, it depends on the IP network room, i.e., it cannot work if different IP network. Indirect data forwarding is always robust and workable. If we support both direct and indirect data forwarding, we would like to try dedicated signalling to enable it if the effort is little/simple. |
| Lenovo |  | Both direct and indirect data forwarding shall be supported if possible. |
| Intel | Seems none at this stage |  |
| E/// | Too early to conclude | Both are possible, but it does not mean both direct and indirect should be supported for selective activation. The question is a bit misleading. Before any scenario like MN-initiated or SN-initiated CPC is confirmed working with selective activation, we prefer the group to wait and check case by case in details. |
| CATT |  | Both direct and indirect data forwarding shall be supported if possible. The beneficial of the direct and indirect should be different in different cases. |
| China Telecom |  | We think both direct and indirect data forwarding should be support for the above scenarios. |
| NTT DOCOMO |  | Same view as Lenovo |
| Huawei |  | Same view as Lenovo |
| NEC |  | Both direct and indirect data forwarding shall be considered. One possible none technical obstacle may be the limited time to complete the WI. |
| Samsung |  | Both should be supported |
| Qualcomm | None at this stage |  |
| Google | None at this stage |  |

## Scenarios: intra-SN vs inter-SN

At RAN3 #117, a WA was made:

WA: RAN3 considers the Inter-CU and Intra-CU cases with equal priority, and studies both the F1 and Xn signaling aspects. It can be revisited based on RAN2 progress.

Question 12: Please, comment if RAN3 may turn the WA into agreement.

| Company | Yes / No | Possible comment |
| --- | --- | --- |
| Nokia | Yes | We don’t think there is any RAN2 dependence, is there? From UE perspective, PSCell change is nearly the same if it is intra-SN or inter-SN change, right? |
| ZTE | Yes | Agree with Nokia. |
| Lenovo | No | It might impact RRC design, e.g., if SN can configure the execution condition for selective activation, considering the “source” SN may change during the selective activation. Thus, we don’t think it is entirely upon RAN3, and some RAN2 discussion last meeting touched upon the issue without conclusion yet. Since RAN2 is the leading group, if we really want to agree on it, it is necessary to send LS to RAN2 checking their view. |
| Intel | Yes | The agreement is to “consider” both inter-CU and intra-CU cases and to “study” both F1/Xn impacts + revisit based on RAN2 progress. No harm to agree. |
| E/// | Neutral | The WA seems enough for current RAN3’s work. If companies like to turn it into agreement, no strong view. Then the group need to keep in mind that there are quite some dependencies with RAN2’s discussion, thus RAN3 would keep aligned with their progress. |
| CATT | Yes |  |
| China Telecom | Yes |  |
| NTT DOCOMO | Yes | Agree with Nokia. Now, we don’t see any intra-SN or inter-SN specific critical issues. |
| Huawei | No | Let’s wait RAN2 progress first. |
| NEC |  | May be need to check the RAN2 status whether they will de-prioritize any scenario. |
| Samsung | Yes | Agree with Intel |
| Qualcomm | Yes | Agree with Ericsson and others that there are significant dependencies on the discussion in RAN2. However, turning it into an agreement seems fine. |
| Google | Yes |  |

# Discussion (1st round)

## Initiating node and general operation

In [5373], [5418] and [5770] the initiative to start Selective Activation is discussed:

- [5373] and [5770] propose to prioritise or focus exclusively on MN-initiated Selective Activation operation;

- [5418] on the other hand proposes to work on solution that would enable both, the MN- and SN-initiated Selective Activation operation.

It seem therefore, there is no doubt to work on MN-initiated Selective Activation operation. The question concerns SN-initiated operation. However, as observed in [5770], in case of MN-initiated operation, it may be both, CPC and CPA. We have therefore 3 types of Selective Activation operation:

- MN-initiated CPA

- MN-initiated CPC

- SN-initiated CPC

Question 1: Please, comment if any of the operations above should be postponed (or excluded) from RAN3 work.

| Company | Operation to be postponed | Possible comment |
| --- | --- | --- |
| Nokia | None | Postponing/deprioritising will likely mean excluding it from Rel.18. However, all of the operations, including SN-initiated CPC, are likely relevant. |
| CATT | None | We may support all the scenarios. But we need to treat them in different priority |
| Huawei | 3)? | It is better to focus on 1) and 2) first, and only discuss 3) if time allows. |
| Lenovo | upon RAN2? | They are all technically possible, however, it doesn’t seem a wise decision for RAN3 to dive into detailed procedures. We can keep an eye on RAN2 discussion during this meeting. Or, we can send an LS to check RAN2’s view or to trigger the scenario discussion there. |
| Qualcomm | None | All the above types of operations seem relevant. |
| ZTE | 1) | We wonder CPA, because when after first CPA execution and subsequent selective activation, the other candidate SN will be changed to CPC, which is different from MN/SN initiated CPC, especially for the delta SN configuration. More, we think we can coordinate with RAN2’s view. |
| Intel | None from R3 point of view | But tend to agree with Lenovo. We think RAN2 needs to progress before we dive into details for this subsequent CPAC. |
| China Telecom | None | All the scenarios should be supported. |
| NEC | None | All are relevant. But agree with Lenovo that we may need to align with RAN2 if RAN2 will prioritize. In [5373] we did not propose to exclude any scenario, but we discuss who to decide to apply the selective activation, and both MN and S-SN is possible while not persuade to let T-SN to take the decision. |
| NTT DOCOMO | None | Agree with the prioritization as 1) – 3). |
| Google | None | The RAN2 progress should be monitored |
| Samsung | None | But we suggest 1) and 2) have higher priority. |
| E/// | None | May down-prioritize 1). Need to sync with RAN2’s progress. |

In [5423], [5713] and [5770], it is discussed if the solution for Selective Activation shall enable keeping the last serving PSCell as prepared cell after UE changes to another PSCell.

Question 2: Please, comment if you agree that the last serving (source) PSCell should be possible to be kept as prepared after the UE changes the serving PSCell?

| Company | Yes / No | Possible comment |
| --- | --- | --- |
| Nokia | Yes | The last serving (source) cell/node is a close neighbout after UE changes the cell, so keeping it prepared is relevant from the Selective Activation perspective. |
| CATT |  | We may study the options: keep it or release/add by MN, which option is more efficient and signalling saving if we keep it, the configuration should be updated and set condition for it. |
| Huawei | Yes | It is possible for UE to switch back to the source PSCell when executing subsequent PSCell change. |
| Lenovo | Yes, but upon RAN2 | Although we have the same understanding, it is essentially upon RAN2 conclusion since it will impact UE behaviour as well. |
| Qualcomm | Yes | Agree with Nokia’s comment. |
| ZTE | Yes | The last serving PSCell can be the new candidate PSCell, but the initiating node can finally decide it, i.e., keep it or release it. |
| Intel |  | Should this be decided by RAN2? |
| China Telecom | Yes | UE may handover back to the source PSCell when performing subsequent PSCell change, so configuring the source PSCell as a new candidate PSCell for the UE is reasonable. |
| NEC | Yes |  |
| NTT DOCOMO | Yes | There can be some cases in which keeping source PSCell as candidate target PSCell in subsequent CPC is helpful. |
| Google | Yes |  |
| Samsung | Yes | Same view as Nokia |
| E/// | Yes |  |

## Coexistence

At the last meeting, RAN3 made following agreement:

From RAN3 point of view, Rel-16/Rel-17 CPAC procedures are considered as start point for the Rel-18 work.

This hints, the Rel.16/17 procedures will be enhanced to support Selective Activation operation and thus the initiation will be action of X2/Xn signalling. However, in [5367], it is suggested initiation of Selective Activation operation may require pre-configuration of the information if the possible target SN supports it.

Question 3: Please, comment if you expect that OAM support will be necessary to initiate Selective Activation operation? Do you have any other comments concerning OAM support?

| Company | Comment |
| --- | --- |
| Nokia | No, we don’t think that initiation of the Selective Activation operation must rely on OAM. Similarly like with other features, Selective Activation will likely require adding new IEs, which, based on criticalities, will enable a node to handle a scenario where the target node does not support Rel.18 Selective Activation. |
| CATT | We don’t think the OAM involvement is needed |
| Huawei | No. An indication of selective activation over Xn/X2 signalling is sufficient to aid the target SN to decide whether to support the new feature. |
| Lenovo | Same view as Nokia. |
| Qualcomm | No, we don’t think OAM involvement is needed. |
| ZTE | In our contribution [5367], we ask a question, can nodes only supporting R17 CPAC and nodes supporting R18 CPAC coexist during the selective activation procedure? For instance, in case of SN initiated CPC, the initiating node (i.e., source SN) and MN support R18 CPAC, some candidate SN also support R18CPAC, but some candidate SN does not support R18 CPAC. Then after CPC execution, during selective activation period, can we release those candidate SN not supporting R18 CPAC and keep other candidate SN supporting R18 CPAC? |
| Intel | We think that, if RAN2 progresses and agrees to make this selective activation based on the explicit configuration from NW (i.e. the UE is explicitly configured to perform selective activation), then it may be better to go with having an explicit indicator for “selective activation” during SN ADD procedure even though there is nothing to enhance in terms of signalling compared to Rel-17 CPAC. With the criticality, non-supporting candidate SNs can be filtered out without relying on OAM. |
| China Telecom | Same view as Nokia. |
| NEC | As usual, OAM configuration is always the default way. Whether we need explicit signalling to indicate whether supporting the Selective Activation operation, may be this is not needed, as we will likely have new IE at least to indicate to keep the prepared CG configuration. |
| NTT DOCOMO | Same view as CATT. |
| Google | Share the same view as NEC. |
| Samsung | Same view as Nokia. |
| E/// | No. This is about node capability, which can be handled normally as other features. Any pre-configuration is leaving for implementation. |

Also, in [5367] it is clearly declared that Selective Activation operation can coexist with classic CPAC operations.

However, there are two variants of coexistence:

- Using different procedures (e.g. Rel.17 CPAC and Rel.18 SelAct) in the RAN for different UEs;

- Using different procedures (e.g. Rel.17 CPAC and Rel.18 SelAct) for the same UE at the same time.

Question 4: Please, comment if you see any issues with the coexistence of Selective Activation operations with other features within RAN for different UEs or at the same time for the same UE.

| Company | Comment |
| --- | --- |
| Nokia | For (1), we don’t see any issue with using any Rel.17 feature together with the Selective activation within RAN for different UEs. For (2), possible handing of Rel.17 CPAC and Rel.18 Selective Activation within the same UE at the same time should be discussed rather in RAN2. |
| CATT | For 1), 2), the UE should be informed which version is used. But it is RAN2 scope |
| Huawei | Agree with Nokia. |
| Lenovo | From [5367], it seems “coexist” means a candidate SN in a R17 CPAC can configure R18 SelAct during R17 CPAC preparation? If the understanding is correct, we think it should be discussed after CHO+CPAC discussion and “CPC/CPA configuration including intra-SN CPC configuration” discussion. |
| Qualcomm | For 1), we agree with Nokia. For 2), we agree that RAN2 involvement is needed. It is not clear why the network would provide the UE with both the configurations at the same time. |
| ZTE | Agree with CATT, it is RAN2’s issue for UE behaviour. We shall consider to allocate the new data forwarding address during selective activation period. |
| Intel | Agree with Nokia. |
| China Telecom | Agree with Nokia. |
| NEC | We understand 5367 is talking about the peer nodes that some support Rel-17 CPAC but some support Rel-17 CPAC+Rel-18 Selective Activation, whether the Selective Activation initiating node (e.g. MN) will need to run different procedures to handle differently. If some peer nodes do not support Rel-18 Selective Activation but support only Rel-17 CPAC, we believe it has no other choice but only to handle differently. And whether can use different procedure for different UEs, since this control is per UE context, it should have no issue using different procedures for different UEs. (May be we misunderstand the question.) |
| NTT DOCOMO | Agree with Nokia. |
| Google | Agree with Nokia. |
| Samsung | Agree with Nokia. |
| E/// | Agree with Nokia. |

## Signalling effort

Two papers address possible signalling effort related to the Selective Activation operation. In [5713] and in [5392] some optimisations are considered that help managing large groups of possibly prepared PSCells:

- In [5713], it is proposed to enable a mechanism that will enable activating or deactivating groups of prepared PSCells, so that only a limited part of them is active for the UE, while frequent reconfigurations of the set of prepared PSCells does not need to be updated.

- In [5392] early data forwarding towards prepared target nodes is considered and the effort of frequent reconfiguration of the data forwarding (in case serving SN changes more often than the set of prepared target SNs).

Even though the objectives are quite different, both papers consider that the group of prepared PSCells (or SNs) may be large and stable, i.e. SN nodes do not have to be added or removed from the set often. In [5392], also an alternative is considered: UE staying long in a given PSCell, while other SNs/PSCells are configured just temporarily, e.g. for the moment of cell border crossing.

Question 5: Please, comment on the Selective Activation scenario: do you expect Selective Activation to be used for UEs changing PSCells frequently among large group of prepared SNs? Also, do you expect that direct early data forwarding should be enabled in Selective Activation?

| Company | Comment |
| --- | --- |
| Nokia | Yes, this is one of relevant scenarios for Selective Activation, if not the most relevant one. To make an illustration, we imagine Selective Activation to be used in a large area, covered by many SNs, all of which remain prepared (and thus the group of possibly prepared nodes remain about-stable), which the UE moves among them. |
| CATT | Yes The direct data forwarding should be supported |
| Huawei | Selective activation intends to apply to the frequent PSCell change case scenario, but we are not sure if there will be large group of prepared SNs, there could be multiple prepared SCGs, but may not large number of prepared SNs, e.g. in Rel-17 there are at maximum 8 prepared SCGs for a UE. Early data forwarding is the main gain of selective activation. Without early data forwarding, selective activation is more or less meaningless, direct data forwarding or indirect data forwarding, could be further discussed case by case. |
| Lenovo | Partially, we agree UE may change PSCells frequently, but whether it is among large group of prepared SNs is unclear. How many SNs shall we expect and how large the area is? |
| Qualcomm | We agree that the scenario with UE changing among multiple prepared PSCells/SNs while it is moving is quite relevant. However, as Huawei mentions, there has to be a maximum number of PSCells configured for a UE, similar as Rel-16/Rel-17 CPA/CPC. Early data forwarding should be enabled, and direct data forwarding is also fine. |
| ZTE | Yes, in our view, the selective activation is use for this changing PSCell frequent scenario. The direct early data forwarding shall be used after R17 CPAC execution, if direct early data forwarding is already used. But if the indirect early data forwarding is used in the previous CPAC preparation, then direct early data forwarding cannot be used during selective activation period. |
| Intel | Agree with Nokia and Huawei. |
| China Telecom | We agree that the selective activation is use for this changing PSCell frequent scenario, but we think the maximum number of candidate SNs should be discussed. Since the larger the number of candidates, the more complex the network, we prefer to keep the prepared SNs in a reasonable range. For data forwarding used in frequent PSCell changes, we think the directed early data forwarding should be supported. |
| NEC | We understand that motivation of such selective activation is when the UE move here and there quite frequently then to keep the prepared configuration even if the UE has selected other cell, the purpose is to reduce signalling when again need to do the CPAC. So the answer should be Yes. |
| NTT DOCOMO | Yes. It seems that prepared cells activation/deactivation could avoid both a frequent candidate cell reconfiguration and a waste of resources in prepared cells, and this scenario (i.e. so many candidates configured) could also be possible. Agree with Huawei about early data forwarding. |
| Google | Yes and direct data forwarding can be supported. |
| Samsung | The direct early data forwarding should be supported. |
| E/// | Early data forwarding should be supported. Whether indirect or direct needs further discussion. |

Since [5392] assumes early data transmission, it is also worthwhile to confirm that it should be enabled for Selective Activation.

Question 6: Please, comment, if direct early data forwarding should be enabled in Selective Activation?

| Company | Yes / No | If the answer is ‘no’, please, explain |
| --- | --- | --- |
| Nokia | Yes |  |
| CATT | Yes |  |
| Huawei | Yes? | Early data forwarding should be enabled, but whether direct or indirect data forwarding could be further discussed case by case. |
| Lenovo | Yes, in principle | In principle yes, otherwise the service continuity will suffer. On the other hand, the complexity of possible solution and scenario needs careful assessment. |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Intel | In principle, Yes |  |
| China Telecom | Yes |  |
| NEC | Yes |  |
| NTT DOCOMO | Yes | Same view as Huawei. |
| Google | Yes |  |
| Samsung | Yes |  |
| E/// | Yes | Agree with Huawei. |

If the scenario in Q6 is likely, RAN3 shall consider if the signalling optimisation solution proposed in [5713] is worth further work.

Question 7: Please, comment if RAN3 shall work further on the signalling optimisation related to the activation/deactivation of groups of PSCells prepared for Selective Activation.

| Company | Yes / No | If the answer is ‘no’, please, explain |
| --- | --- | --- |
| Nokia | Yes, but… | We are fine to study the proposed solution further, but at this stage, it does not seem to realise any promised optimisations: activation/deactivation of groups of prepared cells may be as burdensome as preparing/releasing some targets. But we are fine to evaluate it further. |
| CATT |  | We may have more study whether the group concept is introduced. |
| Huawei | Yes | Agree with Nokia. Not at this point but for later. |
| Lenovo | Yes, after scenarios become clear |  |
| Qualcomm |  | At this stage we can probably focus on how the basic procedure looks like and discuss signalling optimizations later. |
| ZTE | Not sure | We appreciate the motivation from the contribution [5713]. However, because it shall introduce activation/deactivation group besides candidate group, so, I wonder if it will bring additional group change between the two groups, and it maybe ask RAN2 to have different RRC signalling for the two groups. |
| Intel |  | We prefer to wait for RAN2 to progress more before delving into solutions for optimizations. |
| China Telecom |  | Similar view with QC, we should focus on the basic procedures first and then evaluate whether optimization is needed. |
| NEC |  | Fine to study to see first any benefit. |
| NTT DOCOMO | Yes | Agree with Huawei. |
| Google |  | Agree with Intel. |
| Samsung | Yes | Activation and deactivation of the candidate PSCells/CGs is a solution to reduce the signalling overhead for pre-configured candidate cells change with the mobility of UE, especially with the use of FR2. |
| E/// | Yes with comments | RAN2 is going to figure out the scenarios first, we are fine to discuss some basic signalings, but prefer later. |

Then, similarly, if the scenario in Q6 is likely and direct early data forwarding possible, RAN3 may consider optimisations that will help avoiding frequent path switch at the AMF/UPF and frequent switching data forwarding source. This is, of course, needed only assuming that direct early forwarding is to be used.

Question 8: Please, comment if RAN3 shall analyse the solution for simultaneous data transmission from the UPF to all prepared SNs to avoid frequent path switch and switching of the forwarding source?

| Company | Yes / No | If the answer is ‘no’, please, explain |
| --- | --- | --- |
| Nokia | Yes | Answering ‘Yes’ does not mean, of course, that the solution is to be enabled. However, it should be analysed if it is feasible and if it offers benefits as compared to a classic direct early data forwarding. |
| CATT | No | We don’t think the SA should be involved |
| Huawei |  | We are open to evaluate it, but probably this solution is not the preferred one. |
| Lenovo |  | Not if the RAN node based solution does not work or is way too complicated. |
| Qualcomm |  | We are also open to evaluate this proposal. |
| ZTE | No | The data transmission from UPF to all of prepared SNs, then the duplicated data transmission will be happen. It is not good solution because duplication data transmission at NG interface. |
| Intel |  | We prefer to wait for RAN2 to progress more before delving into solutions for optimizations. |
| China Telecom | Prefer No | We think data forwarding optimization should be studied in RAN3, however the solution for simultaneous data transmission from the UPF to all prepared SNs will increase NG transmission load and is not optimal. |
| NEC |  | Open to discuss but involving UPF to duplicate data, may be not an easy thing to do. |
| NTT DOCOMO | Yes | Agree with Nokia. |
| Google |  | Agree with Intel. |
| Samsung |  | We are open to evaluate it. |
| E/// | No | This has been raised as bicasting from CN in Rel-16, which was not selected by SA2 as a possible solution. We don’t see much difference in this case. |

# Conclusion, Recommendations [if needed]

If needed

# References

- [5367]	R3-225367, Disscussion on NR-DC with selective activation (ZTE)

- [5373]	R3-225373, Decision on selective activation of the cell groups (NEC)

- [5392]	R3-225392, Data forwarding considerations for selective activation (Nokia, Nokia Shanghai Bell)

- [5418]	R3-225418, Configuration and activation of multiple cell groups in NR-DC (Qualcomm Incorporated)

- [5423]	R3-225423, Consideration on support of selective activation of cell groups (China Telecommunication)

- [5496]	R3-225496, (TP for TS 38.473) On SCG selective activation (Lenovo)

- [5649]	R3-225649, Discussion of selective activation (NTT DOCOMO INC.)

- [5665]	R3-225665, (TPs to TS 37.340, TS 38.423 BL CRs) Consideration on selective activation of SCGs (Huawei)

- [5713]	R3-225713, Considerations on selective activation of the cell groups (Samsung)

- [5745]	R3-225745, Considerations on NR-DC with Selective Activation (vivo)

- [5770]	R3-225770, Discussion on NR-DC with selective activation of the cell groups (CATT)