3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225996

# E-meeting, 10 – 18 October 2022	was R3-225652

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0261 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Multicast Session Establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Lenovo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | F1AP UE specific UE context management procedures were updated to include MBS related information at RAN3#116, but the call flow of Multicast MBS Session Context establishment procedure did not capture the corresponding part yet. The message flow should not depict multiple scenarios at options at once. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The message flow was changed to depict a UE joining an active multicast session as the first UE joining in its serving gNB - Move the NG-U distribution setup procedure before the UE reated context management procedures. Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause any non-backward compatible issues. This CR only have impacts on the MBS multicast context setup function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreements would not been reflected in stage 2 and the message flow would be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of the first change--------------------

#### 8.15.1.2	Multicast MBS Session Context Establishment

Figure 8.15.1.2-1 illustrates an examplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for Multicast MBS Session Context Establishment with a UE joining an active multicast session as the first UE joining in its serving gNB.

Figure 8.15.1.2-1: Multicast MBS Session Context establishment

0.	For a certain multicast MBS session (TMGI), which is currently active, no multicast MBS session context is yet established in the gNB.

1.	The UE joins the multicast session.

NOTE 1:	NAS related details for PDU Session signalling are not shown in Figure 8.15.1.2-1.

2/3. The gNB-CU-CP establishes the multicast bearer context at the gNB-CU-UP, in order to retrieve for unicast NG-U transport the GTP DL TEID, a shared resource address (GTP DL TEID).

4/5. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC.

6/7.	The gNB-CU-CP establishes the Multicast Context at the DU, providing MRB configuration. It may contain MBS Area Session ID information.

8/9. The gNB-CU-CP retrieves the MRB configuration for the joined UE from the gNB-DU via F1 UE Context Management procedures.

NOTE 2:	The interactions between the gNB-CU-CP and gNB-CU-UP for UE specific Berarer Context management before and after step 8/9 are not illustrated in this Figure.

10.	The gNB-DU triggers the establishment of F1-U resources, which are established either per DU or per cell or per MBS Area Session ID. The receiving gNB-CU-CP  fetches a gNB side F1-U TNL address information from the gNB-CU-UP by means of a E1AP MC Bearer Context Modification procedure.

7/8. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC.

11/12. The gNB-CU-UP side of the F1-U and NG-U UP entity is established by means of the E1AP MC Bearer Context Modification procedure, providing the DU side F1-U TNL address and the 5GC NG-U TNL address to the gNB-CU-UP, which provides the gNB-CU-UP side F1-U TNL address in return.

13.	The gNB-CU-UP side F1-U TNL address is provided to the gNB-DU.

Note 3: The step 10-13 may be triggered earlier than step 8 and 9.

14.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

15.	The gNB successfully terminates the NGAP procedure for establishing the multicast session context.

16.	The gNB-CU-CP RRC-configures each UE which has joined the multicast group.

17.	The multicast MBS media stream is provided to the UEs.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU- CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.

--------------End of the changes--------------------