# 3GPP TSG-RAN WG3 Meeting #119													R3-230073

# Athens, GR, 27 Feb - 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0260 | rev | 6 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of L1/L2 inter-cell mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2 |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is to introduce the L1/L2 inter-cell mobility function into the stage 2 specfication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #117bis: The message flow for intra-DU L1/L2 inter-cell mobility procedure is introduced. #118: Added the inter-DU LTM procedure in TP R3-226913. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-DU L1/L2 inter-cell mobility is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1, update according to comments in #117bis-e meeting. Rev#2, further update according to comments in #117bis-e meeting. Rev#3, further update according to comments in #117bis-e meeting. Rev#4, resubmission to #118 meeting. Rev#5, R3-226930, merge the TP in R3-226913 from #118 meeting. Rev#6, resubmission to #119 meeting. |  |  |  |  |  |  |  |  |

--------------------------------------------------------- the start of change -------------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU L1/L2 based inter-cell Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 based inter-cell mobility. Figure 8.2.1.x-y shows the intra-gNB-DU L1/L2 based inter-cell mobility procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 based inter-cell Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- WA: The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the target candidate cells. FFS: whether parallel UE Context Modification procedure should be used rather than a candidate list in a single message.

- WA: If the request for configuring L1/L2 inter-cell mobility is accepted, the gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes a generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration, which may contain a configuration per target candidate (e.g. including the lower layer RRC configurations of the candidate cells for L1/L2 inter-cell mobility. Details are FFS).

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the L1/L2 inter-cell mobility command to the UE.

- FFS: How the gNB-DU detects the UE access to the target cell is up to RAN2.

- The gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message. Th target cell ID is included.

- FFS: The gNB-CU may send the UE Context Modification message to the gNB-DU to release the resources of prepared cells.

- FFS: The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for L1/L2 Triggered Mobility. Figure 8.2.1.Y-1 shows the inter-gNB-DU L1/L2 Triggered Mobility procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells.

FFS for Step 3 and Step 4: either a single or multiple UE Context Setup procedure(s) should be used.

- If the candidate gNB-DU decides to accept the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration.

FFS: whether it is DL RRC MESSAGE TRANSFER message or UE Context Modification Request message.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE Context Modification Response message.

- The UE sends lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

FFS: Notifying the LTM cell switch decision to the other nodes as well.

- The source gNB-DU sends L1/L2 Triggered Mobility Cell Switch command to the UE.

Editor’s note: the order of step 10 and 11 is not defined. The cell switch command message needs to be updated according to RAN2’s discussion.

FFS:  how the target gNB-DU detects the UE access and whether there is an F1 impact.

FFS: For inter-DU L1/L2 Triggered Mobility, whether and how to release the source cell/prepared cells’ resources in the source gNB-DU is FFS.

--------------------------------------------------------- the end of change -------------------------------------------------------------