3GPP TSG RAN WG3 Meeting #119bis-e	R3-231197

Electronic meeting, April 17th – April 26th, 2023

Title: 	Sharing processing for both unicast reception and broadcast reception

Source: 	TD Tech, Chengdu TD Tech

Agenda item:	15.2

Document Type:	Discussion

1. Introduction

For the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services” [1], the following agreement was made in the last RAN3 meeting for the sharing processing for both unicast reception and broadcast reception. In the contribution, the sharing processing for both unicast reception and broadcast reception is further discussed with the related proposals suggested.

Agree Option4 to support shared NG-U tunnel.

For MOCN, it is up to NG-RAN node implementation to decide how many NG-U tunnels to be setup.

For MOCN, it is up to the NG-RAN node implementation on how to handle different S-NSSAI received for the same shared service from different PLMNs (i.e. same Associated Session ID).

It is agreed to transfer the Associated Session ID together the MBS Session ID.

WA: The Associated Session ID is per TMGI per Area Session ID. Therefore, it is transferred outside the N2 SM container.

2. Sharing processing for both unicast reception and broadcast reception

For a broadcast session provided by different 5GCs, it’s up to gNB implementation to decide how many NG-U tunnels to be set up. But how to send the broadcast session over F1 has not been decided. There are two options:

Option 1: One GTP-U tunnel over F1 per NG-U tunnel

Option 2: One GTP-U tunnel over F1 for all NG-U tunnels

For option 1, the user plane data from each NG-U tunnel is sent on a GTP-U tunnel over F1 to gNB-DU and thus gNB-DU receives many copies. Therefore option 1 needs more buffer space in gNB-DU and consumes more resource over F1.

For option 2, only one GTP-U tunnel over F1 for all NG-U tunnel is set up to send the data from a selected NG-U tunnel. Thus gNB-DU receives only one copy.

Compared with option 1, option 2 needs less buffer space in gNB-DU and consumes less resource over F1. If the failure happens in the user plane of the selected NG-U tunnel, option 2 only needs to establish a new GTP-U tunnel over F1 for a new selected NG-U tunnel. The delay for the new selected GTP-U tunnel over F1 is acceptable.

Based on the above discussion, the following proposals are suggested.

Proposal 1: A GTP-U tunnel over F1 for all NG-U tunnels is set up for sending the user plane data from a selected NG-U tunnel. The other NG-U tunnels have no corresponding GTP-U tunnels over F1.

Proposal 2: If the failure happens in the user plane of the selected NG-U tunnel, a new GTP-U tunnel over F1 is set up for sending the data from a new selected NG-U tunnel.

3. Conclusions

Based on the discussion in the above section, the following proposals are suggested.

Proposal 1: A GTP-U tunnel over F1 for all NG-U tunnels is set up for sending the user plane data from a selected NG-U tunnel. The other NG-U tunnels have no corresponding GTP-U tunnels over F1.

Proposal 2: If the failure happens in the user plane of the selected NG-U tunnel, a new GTP-U tunnel over F1 is set up for sending the data from a new selected NG-U tunnel.

4. References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e