3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231270

Electronic meeting, Apr. 17th – Apr. 26th, 2023

Title: 	(TPs for SON BLCRs for TS 38.300) SPR

Source: 	Huawei

Agenda item:	10.2.1

Document Type:	discussion

Introduction

In last meeting, there were some progresses on the scenarios to be studied SPR. The agreements and FFSs are showed as the following:

SPCR

Proposal 11: FFS whether to also include the following in Successful PSCell Change Report:

- PCell information, in case of MN initiated PSCell change/CPC

- Information that PSCell change was MN-initiated or SN-initiated

- Time between CPC execution and report retrieval

- C-RNTI (MN, target SN, source SN)

SPR:

For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis.

For intra-SN classic PSCell change/CPC, the source SN decides SPR triggers of T310 and T312 and performs root cause analysis.

For SPR:

Objective of SPR:

If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility.

FFS for the trigger T304, whether the the objective of SPR is to optimize PSCell change configuration during mobility or the RACH access issue or both？

Which Node decides the triggers and which node performs root cause?

If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations.

FFS Which Node decides the triggers and which node performs root cause?

Option 1: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis

Option 2：For MN-initiated classic PScell change /CPC, the source SN node decides the T310/T312 triggers and performs root cause analysis

Option 3：For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis

FFS: Fetching of the SPR or forwarding mechanism (i.e. waiting RAN2’s progress).

FFS: correlation between SPR and SCGFailureInformation

In this paper, for the agreed scenarios and FFSs, we will give some potential solutions.

Discussion

Based on the previous discussions and progresses, it seems that there are still some issues to be handled.

FFS on the trigger T304

In RAN3#119 meeting, there were some progresses and open issues left to be discussed.

For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis.

FFS for the trigger T304, whether the objective of SPR is to optimize PSCell change configuration during mobility or the RACH access issue or both？

Technically speaking, if the SPR is triggered due to the T304 trigger, it seems that the configuration of the T304 trigger and/or RACH configuration of the target SN may be suboptimal. The above configurations decided by the target SN can also be considered as part of the whole PSCell change configuration.

Besides, when the SPR is only triggered due to the trigger T304, it seems the decisions and configuration from the source SN are suitable enough. It is possible that the unreasonable configuration of the T304 trigger and/or RACH configuration from the target SN results in the SPR. Therefore, it is sufficient for the target SN to analyze the root cause and optimize related SPR trigger and/or RACH configuration. The above optimizations can also be taken as some optimization of the PSCell change configuration during PSCell change. Correspondingly, the objective of SPR is to optimize the RACH access issue as part of PSCell change configuration optimization during mobility.

Observation 1: The configurations of T304 trigger and/or RACH access are part of PSCell change configuration during mobility.

Proposal 1: For the trigger T304, the objective of SPR is to optimize the RACH access issue as part of PSCell change configuration optimization during mobility.

FFS on the trigger T310/312 for MN-initiated classic PSCell change/CPC

In RAN3#119 meeting, there were some progresses and open issues left to be discussed.

If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations.

FFS Which Node decides the triggers and which node performs root cause?

Option 1: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis

Option 2：For MN-initiated classic PScell change /CPC, the source SN node decides the T310/T312 triggers and performs root cause analysis

Option 3：For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis

For MN-initiated classic PSCell change/CPC, as the above summary, there are three options. In option 1, the MN node can decide the T310/312 triggers itself. This solution can work well if the configuration of the T310/312 triggers are relative ratio compared the related timers. In this way, the MN doesn’t need to know the exact absolute or the current value of the configured timers by the source SN. The changes of the T310/312 configurations have no impact on the MN decision on the T310/312 triggers. This will avoid unnecessary interaction between the MN and source SN. For option 2, it is questionable when the source SN node decides the T310/312 triggers and sends them back to the MN. It is possible that the MN doesn’t initiate the classic PSCell change/CPC. If the source SN node always sends the T310/312 triggers to the MN, this brings unnecessary complexity on the source SN node. For option 3, the MN will need the inputs from source SN. The inputs can be the configured values of the T310/312. When the source SN decides to cancel or change the configurations of T310/312, the source SN node should inform MN of the updated ones. This brings additional notification from source SN node to MN node.

Observation 2: Option 1 can work well and has no Xn impact.

Proposal 2: Option 1 is preferred for MN to decide the T310/312 triggers and performs root cause analysis for MN-initiated classic PSCell change/CPC.

forwarding

In RAN2, it was agreed to report the (stored) SPR via UE Information Request/Response procedure.

Agreements in RAN2#119bis-e:

6	RAN2 agree to the following:

B.	SPR is fetched via UE Information Request/Response procedure

Agreements 1 in RAN2#120:

3	Only MN can retrieve the SPR from the UE.

Agreements 2 in RAN2#120:

2	UE can send the (stored) SPR to gNB. FFS how long UE keeping SPR is FFS.

3	Only the latest successful PSCell change/addition is reported by the UE.

FFS: Fetching of the SPR or forwarding mechanism (i.e. waiting RAN2’s progress).

In RAN2#120 and 121 meeting, some agreements on the reporting of SPR was achieved.

Agreements in RAN2#120:

3	Only MN can retrieve the SPR from the UE.

Agreements in RAN2#121:

1: UE includes available location information in SPR .

2: UE stores SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE if not fetched.

It is worth noting that RAN2 agreed that the UE can store SPR at most 48 hours as RLF report. When the UE sends the stored SPR, it is only allowed to send it to the current serving node or MN node in case of DC configuration. When the serving (MN) node receives the SPR, it should forward it to the suitable node responsible for the SPR for root cause analysis.

Based on the current conclusions in RAN3, the SPR will be sent to the initiating node which configures the triggers that triggered the SRP. For the delivery of the SPR to the triggering node (MN or SN), there are two potential options:

- option 1: the SPR is directly sent to the triggering node which configured the SPR trigger that triggers the SPR

- option 2: the SPR is first sent to the MN that was serving the UE at the time of the event which forwards to the triggering node, then the MN sends the SPR to the triggering node

Proposal 3: RAN3 should discuss the forwarding mechanism at network side for SPR:

- option 1: the SPR is directly sent to the triggering node which configured the SPR trigger that triggers the SPR by the reception node

- option 2: the SPR is first sent to the MN that was serving the UE at the time of the event by the reception node , then the MN sends the SPR to the triggering node

For the procedures to forward the SPR from the reception node to the triggering node or source MN, it is an intuitive idea by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG.

Proposal 4: To forward the SPR, the ACCESS AND MOBILITY INDICATION message over Xn and the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG are reused.

Information in the SPR

In RAN3#118 meeting, there were some discussions on the contents of the SPR and the FFS is showed as below:

Proposal 11: FFS whether to also include the following in Successful PSCell Change Report:

- PCell information, in case of MN initiated PSCell change/CPC

- Information that PSCell change was MN-initiated or SN-initiated

- Time between CPC execution and report retrieval

- C-RNTI (MN, target SN, source SN)

Here, we prefer to start the discussion of the information based on the above option 1 and option 2 of SPR forwarding schemes in the aforesaid section.

As discussed in the above section, for option 1, the UE only includes the related cell information in the SPR. If the classic PSCell change/CPC is initiated by MN, the UE should include the PCell information as the source cell information. If the classic PSCell change/CPC is initiated by SN, the UE should include the source PSCell information as the source cell information. In this way, the UE must know who is the triggering node.

To assist the triggering node to identify the UE, the UE should provide the related C-RNTI per triggering node. As for the time between CPAC execution and report retrieval, together with the agreed time elapsed between the CPAC execution and reception of CPAC configuration, the network can infer when the mobility decision was made and know the corresponding mobility configuration parameters. Based on the time information and the C-RNTI, they can be considered for uniquely identify the UE to finally find the suboptimal configurations, as mentioned in the above section of SHR.

For option 2, the UE always include the PCell information no matter whether the classic PSCell change/CPC is MN initiated or not. To assist the triggering node to identify the UE, the UE should additionally provide MN C-RNTI. Base on the above analysis, the time between CPC execution and report retrieval for CPAC is also needed.

In option 2, if we assume that the MN can decide whether the MN or the source SN is the initiating node in case of classic PSCell change, the information that PSCell change was MN-initiated or SN-initiated in note needed. Otherwise, we also need UE to know the triggering node as in option 1. And we also request the UE to include explicit indicator for MN to know the triggering node.

We give a small summary of the information needed respectively below:

|  | Option 1 | Option 2 |
| --- | --- | --- |
| PCell information, in case of MN initiated PSCell change/CPC | Depend Optional, in case of MN initiated classic PSCell change/CPC | Yes, mandatory |
| Information that PSCell change was MN-initiated or SN-initiated | No need to send from UE, but need UE to be aware of the triggering node to select Cell information | Depends if the MN can know the triggering node: If MN can know: not needed If MN cannot know: Needed and requires UE to know the triggering node |
| Time between CPC execution and report retrieval | Yes | Yes |
| C-RNTI (MN, target SN, source SN) | Yes, triggering node specific C-RNTI | Yes, triggering node specific C-RNTI and mandatory MN C-RNTI |

Based on the above consideration, we believe option 1 can be considered.

Proposal 5: The SPR is directly sent to the triggering node by the reception node (Option 1)

Proposal 6: In option 1, the UE should know the initiating node in case of classic PSCell/CPC.

Proposal 7: In option 1, the following information should be provided:

- PCell information, in case of MN initiated classic PSCell addition/change/CPAC

- Time between CPC execution and report retrieval for CPAC

- C-RNTI per triggering node

Proposal 8: RAN3 should send LS to RAN2 for the above information.

Correlation between SPR and SCGFailureInformation

FFS: correlation between SPR and SCGFailureInformation

As the discussion in section 2.4, the UE can store the SPR at most 48 hours and report it to the serving node different from the one where the SPR is generated. On the contrary, the SCGFailureInformation message is sent to the related node immediately upon detecting of the SCG failure. Both SPR and SCGFailureInformation will be delivered to the right node responsible for the PSCell related mobility.

Observation 3: Both SPR and SCGFailureInformation are sent to the right node responsible for the PSCell related mobility.

As indicated in potential contents in SPR from the UE, there will be enough information to assist the node to look up the UE context. By comparing the UE information with the one in the SCG failure case, the node will correlate the two reports. Thus, it is the network implementation to perform the correlation of SPR and SCGFailureInformation.

Proposal 9: Correlation between SPR and SCGFailureInformation depends on network implementation.

Conclusion

In this paper, we discuss miscellaneous enhancements for SPR, and we have the following observations and proposals:

Observation 1: The configurations of T304 trigger and/or RACH access are part of PSCell change configuration during mobility.

Observation 2: Option 1 can work well and has no Xn impact.

Observation 3: Both SPR and SCGFailureInformation are sent to the right node responsible for the PSCell related mobility.

Proposal 1: For the trigger T304, the objective of SPR is to optimize the RACH access issue as part of PSCell change configuration optimization during mobility.

Proposal 2: Option 1 is preferred for MN to decide the T310/312 triggers and performs root cause analysis for MN-initiated classic PSCell change/CPC.

Proposal 3: RAN3 should discuss the forwarding mechanism at network side for SPR:

- option 1: the SPR is directly sent to the triggering node by the reception node

- option 2: the SPR is first sent to the MN that was serving the UE at the time of the event by the reception node , then the MN sends the SPR to the triggering node

Proposal 4: To forward the SPR, the ACCESS AND MOBILITY INDICATION message over Xn and the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG are reused.

Proposal 5: The SPR is directly sent to the triggering node by the reception node (Option 1)

Proposal 6: In option 1, the UE should know the initiating node in case of classic PSCell/CPC.

Proposal 7: In option 1, the following information should be provided:

- PCell information, in case of MN initiated classic PSCell addition/change/CPAC

- Time between CPC execution and report retrieval for CPAC

- C-RNTI per triggering node

Proposal 8: RAN3 should send LS to RAN2 for the above information.

Proposal 9: Correlation between SPR and SCGFailureInformation depends on network implementation.

A draft LS including SPR aspects are included in the annex.

Reference

[1] RAN3_119_agenda_EOM

Annex_draft LS to RAN2

Title:	LS on SPR

Response to:

Release:	Rel-18

Work Item:	         NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Henrik Olofsson

E-mail Address:	henrik.olofsson@huawei.com

- Overall Description:

RAN3 has discussed the forwarding of the SPR and identified the potential requirements for RAN2 to provide for the SPR:

  - Source PSCell information, in case of PSCell change/CPC

  - PCell information, in case of MN initiated classic PSCell addition/change/CPAC

  - target PSCell information

  - SPR cause

  - Latest measurement results

  - Time elapsed between the CPAC execution and the reception of the CPAC configuration in case of CPAC

  - C-RNTI per triggering node

## 2. Actions:

To RAN2:

ACTION: RAN3 kindly asks RAN2 to take the above into consideration and start analysis on possible Uu impacts.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #120		22 – 26 May 2022			Incheon, KR

TSG-RAN3 Meeting #121		21 – 25 Aug 2022			Toulouse, FR

Annex – TP to TS38.300 for SPR

Start of the change

#### 15.5.2.X	Successful PSCell Addition/Change Report

For analysis of successful PSCell addition/change, the UE supports Successful PSCell Addition/Change Report (SPR) based on configuration by network, if received, and makes the SPR available to the network as specified in TS 38.331 [12].

The UE stores the SPR until the SPR is fetched by the network or for 48 hours after the SPR is recorded.

When the node fetches the SPR from UE, it may forward the SPR by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG.

Upon retrieval of a SPR, the receiving node may analyse whether its mobility configuration needs adjustment.

End of the change