3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231285

E-meeting, 17 – 26 April, 2023

Agenda item:	20.2

Source:	Nokia, Nokia Shanghai Bell, Orange

Title:	(TP for TS 38.423) Discussion on MT-SDT Related Open Points

Document for:	Discussion and Decision

# 1	Introduction

The work item on Mobile Terminated Small Data Transmission has been agreed in [2].

The RAN3 scope has been defined as:

Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response;

- MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.

Note: Data transmission in DL within paging message is not in scope of this WI.

RAN3#119 meeting was the first meeting for MT-SDT and the following agreements were taken:

- MT-SDT can be triggered by DL SDT user data and/or DL SDT signaling

- E1AP: Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator.

- XnAP: When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message. The encoding and the name of MT-SDT information IE are FFS.

- XnAP: The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging.

- F1AP: Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message. FFS on F1AP MT-SDT Information.

In the RAN3#119 meeting, the following aspects of MT-SDT handling were also left FFS

FFS: whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message.

FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

FFS:  whether Data volume should also be provided from the CU to the DU in F1AP PAGING message.

FFS: whether receiving gNB-CU or receving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receving gNB to make the final decision on triggering MT-SDT Uu paging.

FFS: whether and how XnAP RETRIEVE UE CTXT REQ message should carry an MT-SDT resume indication.

We look at these open aspects in this paper and proposes Nokia’s preferred solutions.

# 2	Discussion

E1AP Aspects

- Bearer status

Currently over E1AP the CU CP can configure a bearer to be SDT or not, then it can later on resume/suspend such bearers. It was an open question whether we should allow means for configuring separately MO SDT bearers from MT-SDT bearers over E1. However, last RAN2 has agreed same configuration for MO SDT and MT SDT. Therefore, it is enough to have only SDT bearers configured over E1 which can be later resumed/suspended. No change needed.

- Definition of Data Size

The agreement at last RAN3#119 meeting was:

E1AP: Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator

We propose to confirm that an MT SDT Data Size IE is needed and to clarify also that this MT SDT Data Size can be sent defined as the cumulative data volume present at CU UP for all SDT bearers.

We also propose the unit to be expressed in bytes. A max value of 65535 bytes is proposed in the draft TP.

About the FFS on MT-SDT indicator, we assume that it is not needed because implicitly indicated through receiving the Data Size IE.

- Controlling from CU CP the sending of MT-SDT data size by CU UP

A third FFS was whether the CU CP should control the CU UP to include the data size, and was captured as:

whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message

This control could be based on CU CP determining that UE supports the MT SDT feature. However, we think that the CU UP is already configured in the existing E1 Bearer context setup/modification request whether the Bearer is SDT bearer and can simply include MT SDT Data Size whenever the bearer has been configured for SDT. There is no need to optimize further.

Proposal 1: confirm MT SDT Data Size IE sent by CU UP as E1 Assistance information in the DL DATA NOTIFICATION message for any SDT bearer. Rely on SDT bearer configuration in CU UP for the sending of MT SDT Data Size IE without need of additional control from CU CP.

Xn Aspects

- FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

It was agreed at RAN3#119 that the anchor gNB provides assistance information and new gNB takes the final decision.

XnAP: The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging.

Another main RAN3 agreement in RAN3#119 was

XnAP: When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message. The encoding and the name of MT-SDT information IE are FFS

Therefore, the only open item is the actual name and format of the MT SDT Information IE.

The new gNB can decide based on e.g. data size and cell load. We therefore propose that Xn MT-SDT Information over Xn includes MT SDT Data Size, same data size as anchor CU CP has received from anchor CU UP.

Similar as E1 interface, we also think that MT SDT Information IE just contains the MT SDT Data Size IE: no additional MT SDT indicator is needed because implicitly indicated through receiving the MT SDT Data Size IE.

Proposal 2: include MT SDT Data Size IE as Xn MT-SDT Assistance information IE as follows:

MDT-SDT Information – Optional

  - MT-SDT Data Size – INTEGER (0..65535) in bytes.

- FFS: whether and how XnAP RETRIEVE UE CTXT REQ message should carry an MT-SDT resume indication.

The new gNB should inform the anchor gNB that MT-SDT transaction has been initiated by including the MT-SDT Indicator in the Xn Retrieve UE Context Request message.  Based on this, the anchor gNB can decide whether to continue the MT-SDT transaction with or without relocating the anchor function.

If anchor gNB decides to continue “MT-SDT without anchor relocation” it needs to transfer the RLC bearer configuration for MT-SDT bearer in order for receiving gNB to be able to encode the data payload into an RLC PDU. Since RAN2 decided that the RLC bearer configuration for MT-SDT would be the same than MO-SDT, we can reuse the existing “RLC Bearer Configuration” present in the Xn Partial UE context Transfer message and the current SDT indicator in the Xn Retrieve Request.

Proposal 3: not include a dedicated MT-SDT indication in Xn retrieve request.

F1 Aspects

- FFS:  whether Data volume should also be provided from the CU to the DU in F1AP PAGING message.

- FFS: whether receiving gNB-CU or receiving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receiving gNB to make the final decision on triggering MT-SDT Uu paging.

It was agreed to send F1 Assistance Information from CU CP to DU, however this assistance information depends on which node makes the final decision to trigger MT-SDT paging:

- Option 1: if CU CP decides, the F1 MT SDT information can be a simple flag to trigger gNB-DU to build the RRC Paging including the MT SDT flag.

- Option 2: if gNB-DU decides, it needs to receive the Data size within the F1 MT SDT assistance information.

We assume that the gNB-CU CP has all the necessary information to decide whether to trigger a normal paging or a paging with “MT-SDT” indication based on e.g. total packet size, cell load. However, the gNB-DU is the node to build the RRC Paging message based on gNB-CU input. Therefore, once the gNB-CU has made its decision to use paging with MT-SDT indication it should send the “MT-SDT indicator” to the gNB-DU to be further included into the RRC Paging message.

Also, assuming that gNB-CU takes the decision, there is no need to send other information (packet size, etc..) to the gNB-DU at the time of paging.

So, we propose not to send the data volume in F1AP :PAGING message: the F1 MT SDT assistance information can be a simple MT SDT indicator.

Proposal 4: include MT SDT Indicator as F1 MT-SDT Assistance information.

# 3	Conclusion

This paper has investigated the impacts of supporting MT-SDT and remaining open issues from last RAN3 meeting. It made following proposals:

Proposal 1: confirm MT SDT Data Size IE sent by CU UP as E1 Assistance information in the DL DATA NOTIFICATION message for any SDT bearer. Rely on SDT bearer configuration in CU UP for the sending of MT SDT Data Size IE without need of additional control from CU CP.

Proposal 2: include MT SDT Data Size IE as Xn MT-SDT Assistance information IE as follows:

MDT-SDT Information – Optional

  - MT-SDT Data Size – INTEGER (0..65535) in bytes.

Proposal 3: not include a dedicated MT-SDT indication in Xn retrieve request.

Proposal 4: include MT SDT Indicator as F1 MT-SDT Assistance information.

A proposal TP for TS 38.423 is proposed below and a TP for TS 37.483 is proposed in [3].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- RP-213583, Work Item on Mobile Terminated Small Data Transmission for NR

- R3-231286, Introduction of MT-SDT

# TP for TS 38.423

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards an ng-eNB (e.g. NG-RAN node2).

When available, the NG-RAN node1 shall include the E-UTRA Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the E-UTRA Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

Not modified

#### 9.2.3.xxx	MT-SDT Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT SDT Data Size | O |  | INTEGER (0…65535, …) | Indicates the total data size for all SDT bearers. Unit: byte. |

Not modified

MaximumCellListSize ::= INTEGER(1..16384, ...)

MT-SDT-DataSize ::= INTEGER (0..65535, ...)

MT-SDT-Information ::= SEQUENCE {

mT-SDT-DataSize			MT-SDT-DataSize	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL,

...

}

MultiplexingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}