3GPP TSG-RAN WG3 Meeting #119bis-e                                                              R3-231309

Electronic meeting, April 17 – 26, 2023

Agenda item: 		13.2

Source: 		Qualcomm Incorporated

Title: 	Topology adaptation for mobile IAB

Document for:	Discussion

# 1	Introduction

The last two RAN3 meetings and the last RAN2 meeting achieved multiple agreements related to mobile-IAB inter-donor migration procedures [1] [2] [3]. The following discussion aims to make further progress on this topic.

References to specific agreements are included in the discussion below.

# 2	Discussion

## Issue 1: DU migration: Information passed when triggering F1 Setup

- RAN3 agreed in the last meeting [1]:

When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the information of target logical mIAB-DU’s CU (e.g. IP address, gNB-ID).

The question arises on the exact information on the target logical mIAB-DU’s CU to be included in this trigger message.

- The information should at least include the CU’s gNB-ID.

- It may be beneficial to include the target CU’s (inner) IP address, which the IAB-node can use for the F1 Setup procedure.

- In case IPsec tunnel mode is used, it should also be beneficial to include an outer tunnel IP address.

Proposal 1a:  When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the target CU’s gNB-ID, and optionally the CU’s IP address as well as the CU’s outer IP address for the use of IPsec tunnel mode.

The question arises how the mIAB-DU’s CU knows about these parameters. Note that the target CU’s IP addresses to be used for F1 establishment may be different that those used for Xn.

Proposal 1b:  RAN3 to discuss how the source mIAB-DU’s CU obtains the (inner and outer) IP addresses of the target mIAB-DU’s CU to be used for F1 Setup.

## Issue 2: DU migration: mIAB-MT info passed to mIAB-DU’s CU

- RAN3 agreed in November [1]:

The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed. The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID.

- RAN3 further agreed in last meeting [2]:

In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU.

- There are two aspects that need to be addressed:

- How is the mIAB-MT’s ID generated and past along through multiple migrations.

- How is the target logical mIAB-DU’s CU informed about the mIAB-MT’s CU ID and mIAB-MT ID.

- On the management of the mIAB-MT ID:

- The mIAB-MT ID needs to be known by the mIAB-MT’s current CU, so that this CU can identify the mIAB-MT associated with the mIAB-MT ID included in an Xn Transport Migration Management Request message.

- The mIAB-MT ID must further be generated by the mIAB-MT’s current CU so that potential ID collisions among IDs used for multiple mIAB-MTs are avoided.

- Observation 1: To avoid mIAB-MT ID collisions, the mIAB-MT ID used for Xn Transport Migration Management Request messages toward a CU must have been generated by this very same CU.

- In case the mIAB-MT hands over to a target CU, a new mIAB-MT ID must be generated by this target CU, which can then be used by the mIAB-DU’s CU in the Xn Migration Management Request sent the target mIAB-MT’s CU.

- Observation 2: In case the mIAB-MT hands over to a target CU, a new mIAB-MT ID must be generated by this target CU for the Xn Migration Management Request messages sent to this target CU.

- Proposal 2a: The mIAB-MT ID included in the Xn Transport Migration Management Request messages toward a CU must have been generated by this very same CU.

- On how the mIAB-MT’s CU ID and mIAB-MT ID is passed to the target logical DU’s CU:

- Figure 1: Options to pass mIAB-MT ID and MT’s gNB-ID to the mIAB-DU’s CU.

- Two options can be considered.

- Option 1: The information is passed to the target logical mIAB-DU’s CU via Xn.

- This approach aligns with RAN3’s present agreement to pass the same information from the source mIAB-MT’s CU to the source mIAB-DU’s CU. The following sub-options need to be considered:

- Option 1a: The info is sent to the target mIAB-DU’s CU by the source logical mIAB-DU’s CU.

- Option 1b: The info is sent to the target mIAB-DU’s CU by the source logical mIAB-MT’s CU.

- Option 1c: The info is sent to the target mIAB-DU’s CU by the target logical mIAB-MT’s CU.

- Option 1c was already ruled out for the passing of this info to the source logical mIAB-DU’s CU. For that reason, Option 1c will not be further discussed.

Option 1b implies that the source mIAB-MT’s CU knows about the CU selected by the source logical mIAB-DU for DU migration, which may not be the case. Option 1b can therefore be ruled out, too.

Option 1a is principally possible and shown in Fig. 1a. This option implies that two F1-terminating IAB-donor-CUs (i.e., the source mIAB-DU’s CU and target mIAB-DU’s CU) exchange information about a UE (= IAB-MT), which neither of these F1-terminating IAB-donor-CUs has a relation with (3rd message in Figure 1a). The only relationship these two donors have with each other is that their respective DUs reside on the same IAB-node. Therefore, non-UE-associated signaling needs to be used for this information exchange. While this can be supported, it will be difficult to migrate such non-UE-associated signaling to an NGAP-related exchange in case Xn is not supported (e.g., which AMF would be used?).

Option 2: The information is passed to the target logical mIAB-DU’s CU via the mIAB-node.

- The mIAB-node can be used as a vehicle to pass the mIAB-MT’s gNB-ID and the mIAB-MT ID between CUs (see Fig. 1b):

- The mIAB-MT’s CU ID is already known to the mIAB-node.

- The mIAB-MT ID can be passed by the mIAB-MT’s CU, e.g., in the handover command or the initial RRC Setup message.

- Both IDs can be passed to the source logical mIAB-DU’s CU and the target logical MIAB-DU’s CU via F1AP.

- Option 2 can use existing RRC and F1AP signaling. It does not rely on Xn as Option 1, and therefore, it also works in scenarios where Xn is not supported. Adoption of Option 2 implies reversal of RAN3’s prior agreement to pass such information via Xn.

- Option 2 is further compliant with IAB topology discovery introduced in Rel-16, where the IAB-DU sends the BAP address, which the co-located IAB-MT received via RRC, to the CU via F1.

Observation 3: Passing mIAB-MT ID and MT’s gNB-ID to the DU’s CU via Xn requires new Xn signaling, while passing via the IAB-node can reuse existing signaling, and it is better aligned with IAB’s topology discovery procedure.

Proposal 2b: In reversal of prior agreement, mIAB-MT ID and MT’s gNB-ID to be passed via the mIAB-node to the mIAB-DU’s CU(s).

## Issue 3: DU migration: Coordination on DU-migration decisions

RAN3 agreed in prior meetings [1][2]:

In case the donor of the mIAB-DU decides the F1AP setup for DU migration, the donor of the mIAB-DU triggers via F1 signalling the IAB node to perform the F1 Setup procedure for the DU migration. An OAM based solution is not excluded. The trigger for F1 setup between the mobile IAB-node’s second logical DU and its donor CU may be based on OAM or pre-configuration. Target donor CU selection for mIAB-DU migration and triggering conditions for F1 setup can be up to source CU implementation (unless it is justified that this is not possible) or based on OAM configuration at the source CU.

Based on these agreements, DU migration can be triggered by the mIAB-node or by the mIAB-DU’s CU. To avoid conflicting decisions, the mIAB-node and the mIAB-DU’s CU need to coordinate, which of the two nodes is responsible for to take DU migration decisions. For this purpose, it is best to use legacy principles:

- The mIAB-DU indicates in the F1 Setup Request, whether it is configured to take DU migration decisions.

- The CU indicates in the F1 Setup Response, whether it itself wants to take the DU migration decisions.

Proposal 3:  The mIAB-DU to indicate in the F1 Setup Request, whether it is configured to take DU migration decisions, and the CU to indicate in the F1 Setup Response, whether it itself wants to be taking the DU migration decisions.

## Issue 4: ST2 MT-migration and DU-migration procedures

Based on prior agreements, mIAB-MT migration and mIAB-DU migration can be captured as separate St2 procedures.

Proposal 4:  mIAB-MT migration and mIAB-DU migration to be captured as separate St2 procedures.

## Issue 5: Inclusion of mIAB-MT’s ULI with UE’s location update

- RAN3 replied to SA2’s LS on VMR in R3-211011 on the inclusion of mIAB-MT’s ULI with UE’s ULI:

- For point#7 (regarding KI#6): RAN3 believes that the functionality requested to provide UE location in point#7 can be accommodated within Rel-18.

In case the mIAB-DU’s CU is the same as the mIAB-MT’s CU, this CU can include its gNB-ID into location reports for UEs connected to the mIAB-DU.

In case the mIAB-DU’s CU is different form the mIAB-MT’s CU, the cell ID used by the mIAB-MT must be known to the mIAB-DU’s CU. This cell ID can be passed together the gNB ID of the mIAB-MT’s CU to the mIAB-DU’s CU.

Proposal 5a: The cell ID used by the mIAB-MT to be passed to the mIAB-DU’s CU together with the gNB-ID of the mIAB-MT’s CU.

In addition, the NGAP User Location Information IE needs to be enhanced to include the gNB ID of the IAB-donor-CU that serves the mobile IAB-MT, which is collocated with the mobile IAB-DU serving the UE.

Proposal 5b: The NGAP User Location Information IE to optionally include the mIAB-MT’s cell ID.

## Issue 6: Enhancements to BH transport

- RAN2 discussed enhancement necessary to BH transport for the support of two concurrent logical IAB-DUs on the IAB-node. RAN2 agreed [3]:

For the upstream data handling at the BAP of mobile IAB MT, one common default BAP configuration to be used by both logical DUs is the baseline. RAN2 to further discuss the need of using logical-DU-specific default BAP configuration (e.g. when the two logical DUs use different donor-DUs). For the upstream data handling at the BAP of mobile IAB MT, RAN2 assume that the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection (To be confirmed by RAN3). For the downstream data handling arriving at the mobile IAB node, RAN2 assume upper layers (e.g. IP layer) can differentiate the data to different logical DUs based on e.g. the IP address, i.e. no need to introduce logical-DU-specific BAP address. (To be confirmed by RAN3).

RAN3 can confirm RAN2’s assumptions.

Proposal 6: RAN3 confirms RAN2’s assumptions on upstream and downstream data handling at BAP layer in presence of two concurrent logical IAB-DUs on the IAB-node.

# Conclusion

This contribution discussed issues related to mobile-IAB inter-donor migration procedures. The following observations and proposals have been made:

Issue 1: DU migration: Information passed when triggering F1 Setup

Proposal 1a:  When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the target CU’s gNB-ID, and optionally the CU’s IP address as well as the CU’s outer IP address for the use of IPsec tunnel mode.

Proposal 1b:  RAN3 to discuss how the source mIAB-DU’s CU obtains the (inner and outer) IP addresses of the target mIAB-DU’s CU to be used for F1 Setup.

Issue 2: DU migration: mIAB-MT info passed to mIAB-DU’s CU

- Observation 1: To avoid mIAB-MT ID collisions, the mIAB-MT ID used for Xn Transport Migration Management Request messages toward a CU must have been generated by this very same CU.

- Observation 2: In case the mIAB-MT hands over to a target CU, a new mIAB-MT ID must be generated by this target CU for the Xn Migration Management Request messages sent to this target CU.

Observation 3: Passing mIAB-MT ID and MT’s gNB-ID to the DU’s CU via Xn requires new Xn signaling, while passing via the IAB-node can reuse existing signaling, and it is better aligned with IAB’s topology discovery procedure.

- Proposal 2a: The mIAB-MT ID included in the Xn Transport Migration Management Request messages toward a CU must have been generated by this very same CU.

Proposal 2b: In reversal of prior agreement, mIAB-MT ID and MT’s gNB-ID to be passed via the mIAB-node to the mIAB-DU’s CU(s).

Issue 3: DU migration: Coordination on DU-migration decisions

Proposal 3:  The mIAB-DU to indicate in the F1 Setup Request, whether it is configured to take DU migration decisions, and the CU to indicate in the F1 Setup Response, whether it itself wants to be taking the DU migration decisions.

Issue 4: St2 MT-migration and DU-migration procedures

Proposal 4:  mIAB-MT migration and mIAB-DU migration to be captured as separate St2 procedures.

Issue 5: Inclusion of mIAB-MT’s ULI with UE’s location update

Proposal 5a: The cell ID used by the mIAB-MT to be passed to the mIAB-DU’s CU together with the gNB-ID of the mIAB-MT’s CU.

Proposal 5b: The NGAP User Location Information IE to optionally include the mIAB-MT’s cell ID.

Issue 6: Enhancements to BH transport

Proposal 6: RAN3 confirms RAN2’s assumptions on upstream and downstream data handling at BAP layer in presence of two concurrent logical IAB-DUs on the IAB-node.

# References

- Chairman notes, TSG-RAN WG3 Meeting #118, Toulouse, France, November 2022.

- Chairman notes, TSG-RAN WG3 Meeting #119, Athens, Greece, February 2023.

- Chairman notes, TSG-RAN WG2 Meeting #121, Athens, Greece, February 2023.