3GPP TSG RAN WG 3 Meeting #119-bis electronic		TDoc R3-231326

17th - 26th April, 2023

Title:	[Draft] Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM

Response to:	LS R3-231107/R1-2302194 on L1 measurement RS configuration and PDCCH ordered RACH for LTM from RAN1

Release:	Release 18

Work Item:	NR_mob_enh2-Core

Source:	Fujitsu, CATT (to be RAN3)

To:	RAN1, RAN2

Cc:

Contact person:	Shinichiro Aikawa

saikawa @ fujitsu.com

sun jiaying

sunjiaying @ catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks RAN1 for their LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM, based on the discussion in RAN2.

RAN3 would like to provide the following feedback for RAN1’s question:

RAN 1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN 3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in the agreement described in section B.

- From RAN3 perspective, both options are feasible.

- The following aspects will be potential RAN3 specs impact.

<For with RAR and the RAR is received from the serving cell>

- RAN3 needs to specify how the RAR is transmitted from the candidate DU to the serving DU, e.g., the candidate DUs shall transmit the RARs to the serving DU by F1 interface through the CU.

<For Without RAR>

- RAN3 needs to discuss and specify how and when the TA value(s) is(are) transmitted from the candidate DU to the serving DU and how the TA value is maintained:

-	Option 1: The candidate DUs transmit the TA values of candidate cells to serving DU before LTM triggering and the serving DU maintains the TA values, e.g., the candidate DUs shall transmit the TA values to the serving DU by F1 interface through the CU.

-	Option 2: Option 2: After LTM is triggered, the serving DU requests the target DU to transmit the latest TA value which is maintained by itself before sending LTM command, e.g., the target DU shall transmit the requested TA value to the serving DU by F1 interface through the CU.

2	Actions

To RAN1 and RAN2 groups

ACTION: 	RAN3 kindly asks RAN1 and RAN2 to take the above feedback into consideration.

3	Dates of next TSG RAN WG 3 meetings

TSG RAN WG3 Meeting #120	22 – 26 May 2023	Incheon, KR

TSG RAN WG3 Meeting #121	21 – 25 August 2023	Toulouse, FR