3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231489

# Online, April 17th – 26th 2023

Agenda Item:	11.4

Source:	Ericsson

Title:	(TP for QoE BL CR for TS 38.473): Enhancements of Rel-17 QoE and RVQoE Features

Document for:	Agreement

# Introduction

In this paper we discuss the enhancements of Rel-17 QoE and RVQoE features, namely:

- DU participation in RVQoE measurements and reporting.

- Trigger-based RVQoE reporting.

- Handling of QoE reporting at RAN overload.

- Intra-5GC inter-RAT mobility support.

A TP for QoE BL CR for TS 38.473 is provided in the Annex.

# DU participation in RVQoE measurements and reporting

We discuss the following issues:

- Deactivation of RVQoE reporting towards the DU.

- DU participation in generating the RVQoE configuration.

## Deactivation of RVQoE reporting towards the DU

The RAN3#119 meeting notes capture the following agreement:

Introduce the deactivation of RAN visible QoE information transfer via F1. No need to introduce pause/resume mechanism in Release 18.

Based on the agreement, we propose to introduce a new F1AP class-2 procedure. The message should be sent from the DU to the CU, containing an indication to the CU to deactivate the RVQoE information transfer towards the DU. An example of the proposed new UE-associated procedure QoE Information Transfer Control is shown below.

Proposal 1: Introduce a new class-2 F1AP UE-associated procedure to control the transfer of RVQoE information from the CU to the DU.

## DU participation in generating the RVQoE configuration

The following is to be further discussed in RAN3:

DU participation in assembling the RVQoE configuration.

Scheduling optimization at the DU was one of the main reasons for introducing, not only the F1AP QoE Information Transfer procedure, but the entire RVQoE concept in Rel-17. In fact, depending on the use case, it might be that the DU is the only consumer (or the main consumer) of RVQoE reports, meaning that it is appropriate to let the DU have a say in assembling the RVQoE configuration.

Nevertheless, the Rel-17 specifications do not allow the DU to participate in generating the RVQoE measurement configuration. This contradiction should be addressed in Rel-18.

For RVQoE measurements, the CU receives a list of available RVQoE metrics (from the CN in case of s-based QoE, or from the OAM in case of m-based QoE). The CU configures the UE with the RVQoE metrics to be reported and, optionally, with a reporting periodicity. Even if the RVQoE metrics defined as of today are only two, the reporting periodicity is a very important parameter, particularly from the perspective of the DU as consumer of the reports. So, we think that the DU should at least be able to have a say on this configuration parameter. Let us consider the following options:

Alt1: The CU configures the reporting periodicity without consulting the DU. The DU can understand what the reporting periodicity is only after receiving two consecutive RVQoE reports (reporting periodicity = time elapsed between the two reports). If the reporting periodicity is appropriate according to the DU, then no action is needed. If not, the DU indicates its preferred reporting periodicity to the CU and the CU reconfigures the UE with a new (updated) reporting periodicity. This approach is very simple, but has some severe limitations when the reported periodicity selected by the CU is not appropriate for the DU:

- The session may be completed when the second report arrives, meaning that the reports are pretty much wasted from DU point of view

- The network needs to send to the UE two RRCReconfiguration messages: one to configure the RVQoE measurements, and another one t re-configure it, if needed.

If we consider that more RVQoE metrics will be available in the future, the same limitations are also applicable when the RVQoE metrics of interest for the DU are not identical to the ones selected by the CU.

Alt2: The CU does not configure the reporting periodicity. In this case, the RVQoE reports will be sent according to the reporting interval specified at the application layer. However, this parameter is optional (see table below from TS 26.247). If the reporting periodicity is not indicated in the XML file, the RAN will not be able to infer it from consecutive RVQoE reports because only one RVQoE report will be sent by the UE at the end of the session. In this case, configuring the RVQoE is de facto of no use for the DU.

|  | @reportinginterval | O | Indicates the time(s) reports should be sent. If not present, then the client should send a report after the streaming session has ended. If present, @reportingInterval=n indicates that the client should send a report every n-th second provided that new metrics information has become available since the previous report. For each report sent, only the newly collected information since the previous report shall be reported. |
| --- | --- | --- | --- |

Alt3: The CU configures the reporting periodicity after consulting the DU. In this case, the DU can propose a reporting periodicity (and, possibly, a list of RVQoE metrics). Only after this is done, the CU configures the UE. This can be realized with a new F1AP class-1 procedure, where the CU indicates to the DU the available QoE metrics and the reporting periodicity that it intends to use. The DU confirms the proposal or suggests some changes. Finally, the CU configures the UE. It is probably good that the CU, being the function in charge of configuring the UE, maintains the possibility to accept or discard the suggestions received from the DU. An example of the new F1AP UE-associated QMC Coordination class-1 procedure is shown in the figure below.

Alt3 has some clear advantages over the other options, and it is our preferred choice:

- The DU, as consumer of RVQoE metrics, can at least express its “opinion” in order to receive RVQoE reports that are useful from its point of view.

- No RVQoE reports are wasted.

- The UE needs to be configured only once, rather than twice (needed in Alt1).

Proposal 2: Introduce a new class-1 UE-associated F1AP procedure) initiated by the CU, where:

- The CU indicates to the DU the available RVQoE metrics and the intended reporting periodicity.

- The DU indicates to the CU its preferred available RVQoE metrics and its preferred reporting periodicity.

Overall, we support the following F1AP procedures to handle QMC configuration and reporting in Rel-18:

- One (Rel-18) class-1 procedure (e.g., QMC Coordination) initiated by the CU, used to assemble the RVQoE configuration.

- One (Rel-17) class-2 procedure, QoE Information Transfer, to transfer RVQoE reports from CU to DU

- One (Rel-18) class-2 procedure (e.g., QoE Information Transfer Control) initiated by the DU, and used to notify the CU to deactivate the transfer of RVQoE reports towards the DU.

A TP adding the two Rel-18 procedures above is provided in the Annex.

Proposal 3: Agree the TP for QoE BL CR for TS 38.473 in the Annex.

# Trigger-based RVQoE reporting

We discuss the following issues:

- Event-based RVQoE reporting trigger.

- Handling of trigger-based RVQoE reporting.

- The format of buffer level threshold in RVQoE reports.

## Event-based RVQoE reporting trigger

RVQoE reports may be useful for optimization of mobility related decisions. For example, a gNB can configure the UE to send radio measurement reports when poor coverage is detected (as in A2 event, inter-frequency A3 event, B1 event defined in TS 38.331). When the UE reports to the gNB that one of these radio related events is fulfilled, the gNB can decide to transfer the UE to a different frequency or to a different RAT. If the gNB could configure the UE to also send the RVQoE metrics observed in the “time vicinity” of one of the above radio related events (i.e., around the same time that the radio even occurs), the gNB could use this information as a tool for optimizing the mobility decisions based on such events.

For example, it might be that the decision to perform inter-frequency mobility (based on A2 event) could be postponed without impacting too much the user experience. Another example can be that the trigger point of A2 needs to be anticipated, to avoid poor signal quality.

If the UE can be configured to send an RVQoE reporting in the time vicinity of a mobility related decision, then the gNB can have a good understanding of how to optimize radio related parameters taking the user experience into account.

Observation 1: RVQoE reports triggered by radio related events (as defined in TS 38.331) can be used by the gNB to optimize mobility decisions.

To enable RVQoE reporting triggered by radio related events, the following is needed:

- The gNB can configure the UE AS to perform radio measurements as usual. For example, the UE can be configured to report fulfilment of event A2.

- The gNB can send to the UE an RVQoE configuration that is extended to include the radio related events of interest (e.g., “A2”).

  - For example, the combination “A2”+ “RVQoE report” can be sent to the gNB to indicate that the RVQoE report is triggered by an A2 event.

- When the UE AS detects that event A2 is fulfilled, the UE AS can query the application layer for an RVQoE report. The UE AS can then associate the RVQoE report to the triggering event.

- The UE AS can send the RVQoE report and the indication of the triggering event as part of the MeasurementReportAppLayer RRC message, or separately (FFS).

Proposal 4: RVQoE reporting can be triggered by a radio related event (as defined in TS 38.331). Details are FFS.

Note that there is no need for the application layer to be aware that a specific radio event is the cause for providing a RVQoE report. In other words, the mechanism to query the application layer to receive RVQoE report can be “event-agnostic”.

Observation 2: Introduction of event-based RVQoE trigger has no impact on the application layer.

However, it is essential for RVQoE report analysis that the gNB is aware of the triggering event that caused the sending of the aperiodic RVQoE report.

Proposal 5: The UE should indicate to the network the event that triggered the RVQoE reporting.

## Handling of trigger-based RVQoE reporting

The following was agreed at the RAN3#118 meeting:

Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting.

The details of implementing the agreement need to be clarified, and we think that RAN3 should discuss the following:

- Can periodic RVQoE reporting coexist with threshold-based RVQoE reporting?

- Once the threshold-based RVQoE reporting starts, does the UE send only one report, or keeps sending the reports periodically?

- If the UE sends the RVQoE reports periodically when threshold-based RVQoE reporting starts, does the reporting need to be stopped, and, if yes, how?

The first two points can be discussed together. For example, it may be interesting for the RAN to know how the buffer level evolves over time, which can be achieved by letting the reporting continue periodically after being triggered. In this way, for example, the RAN can monitor whether an action that it has taken based on the first threshold-based RVQoE report was a correct one. So, if periodic RVQoE reporting has been configured together with threshold-based RVQoE reporting, after the RVQoE reporting is triggered (e.g., by a threshold), the UE can continue sending the reports periodically, according to already established Rel-17 mechanisms.

Proposal 6: When threshold-based RVQoE reporting starts, the reports start to be sent immediately, and are sent periodically.

Regarding the third point, i.e., whether and how to stop trigger-based RVQoE reporting, we foresee the following options:

- In case of threshold-based trigger, RAN sends to the UE (in the RVQoE configuration) some parameter that will guide the UE to stop sending RVQoE reports.

  - For example, one possible way can be to use a similar strategy as for logged MDT, and RAN can indicate a certain reporting duration.

- The RAN can stop the reporting by modifying the RVQoE configuration.

Otherwise, the UE can keep sending reports until the session ends (the “do nothing” approach).

In our understanding, an explicit “stop” via the modification of the existing RVQoE configuration is already supported in Rel-17, so that looks a viable solution with no spec impact. Other options of RAN control (e.g., configuring a reporting duration) can be even better, and we are open to discuss them as further improvements.  Finally, a “do nothing” approach is always possible.

Proposal 7: Threshold-based RVQoE reporting can be stopped by modifying the RVQoE configuration. Otherwise, the RVQoE reporting continues until session end.

## The format of buffer level threshold in RVQoE reports

The RAN3#118 meeting notes capture the following agreement:

Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting.

One remaining issue is the format of the buffer level threshold. We propose to express the threshold in the form of the remaining playout time of the content currently in the buffer, and to send an LS asking RAN2 to provide the corresponding RRC signalling support.

Proposal 8: Send an LS asking RAN2 to specify in RRC signalling the buffer level threshold for RVQoE reporting expressed in terms of remaining playout time of the content currently in the buffer.

# Handling of QoE reporting at RAN overload

At the RAN3#119 meeting, the following was agreed:

In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE.

Before determining the content of the assistance information, RAN3 should discuss some aspects and issues, related to the consumer which expects to receive the QoE reports.

There can be different types of consumers (or consumer categories), for example:

- Centralized consumers (or “centralized automation functions”): this type of consumer may ask for QoE reports to perform network optimization in a “semi-static” way. For example, a consumer of this type would like to know if certain configuration parameter values lead to a better user experience compared to other configuration parameter values.

- Distributed consumers (or “distributed automation function”): this type of consumer may ask for QoE reports to perform network optimization in a more “dynamic” way. For example, a consumer of this type would like to understand if different QoS parameters can improve the user experience on a per-session basis.

Because of their different goals, the different consumers (or different types of consumers) might have different needs. It is possible that one consumer intends to optimize only certain services, while another consumer intends to optimize the overall network performance in a certain area. These, and possibly other characteristics of the consumers, need to be considered.

Proposal 9: RAN3 to discuss whether assistance information for handling of QoE reporting upon RAN overload should include the type of consumer that will receive the QoE reports, or some characteristics of the consumers.

# Intra-5GC inter-RAT mobility support

RAN3 has received an LS from RAN2 in R3-231110 on the WID objective related to continuity of QoE measurements during intra-5GC inter-RAT HO. RAN2 agreed the following:

1: RAN2 understanding is that for HO between LTE/5GC and NR, QoE continuity is done in AS layer (rather than APP layer), that means the QoE measurement continuity in application layer may not be guaranteed.

2: Agree on the principles of Option 3 and Option 4:

- -	Option 3: For HO from NR to LTE/5GC, the UE can keep and continue measurements for only one configuration for a service type supported in LTE

- -	Option 4: For HO from LTE/5GC to NR, the UE can keep and continue measurements for the ongoing configuration for a service type supported in NR

3: Option 3 and Option 4 can be worked on in this WI only if there are no impacts to LTE specifications.

According to the RAN2 agreement, Option 3 and Option 4 can be specified only if there is no impact on LTE specification, and we would like RAN3 to discuss if this is the case.

First, we note that the measurement configuration information that is stored at the UE could be passed from NR to LTE (or vice versa) using inter-node RRC signalling (pending RAN2 discussion). This means that the provision of LTE-/NR-specific configuration information to the UE is not an issue. The problem to solve is mostly related to the network side and stems from the fact that:

- In LTE, only one QoE measurement configuration at a UE at the time is possible, while, in NR, up to 16 configurations are supported.

- The format of the QoE measurement configuration information in LTE and NR is different. The NR QMC supports far more features than the LTE QMC.

Observation 3: The existing NR QoE configuration information cannot be transferred to LTE “as is”, and vice versa.

It is also not possible to transfer a “filtered” version of the NR configuration to LTE (or vice versa).

For example, when comparing the UE Application Layer Measurement Configuration Information IE for NR (XnAP) and the UE Application layer measurement configuration IE for LTE (X2AP), we observe at least the following differences:

- The LTE XML container has a maximum size of up to 1000 bytes, while its NR counterpart can be up to 8000 bytes in size.

- The presence of the XML container is optional in the NGAP and XnAP signalling, while it is mandatory in S1AP and X2AP signalling.

- The LTE Area Scope contains E-GCI, or 4G TAC, but containes no information regarding NR CGI, or 5GS TAC. The opposite holds as well - the NR Area scope contains no information about LTE cells or TAC.

- The order of the common IEs inside the LTE QMC IE and the NR QMC IE is different.

- The MCE collecting the results might not be necessarily the same for NR and LTE.

Observation 4: Current NR area scope IE contains no cell- or TA-related LTE information.

Observation 5: Current LTE area scope IE contains no cell- or TA-related NR information.

Observation 6: Information elements used both in the NR QoE configuration and in LTE QoE configuration are not compatible due to different sizes and presence.

To support Option 4 (HO from LTE to NR), at least the LTE configuration needs to be modified, which is not in line with the RAN2 agreement 3 above.

Proposal 10: Option 4 (HO from LTE/5GC to NR) is not supported.

To support Option 3 (HO from NR to LTE), some “new” NR configuration seems needed, for example to include some LTE information. To avoid LTE specification impact, such “new” NR configuration would have to be transferred “as is” to the LTE node. This might be feasible, although some “hacks” seem required. For example, the “NR” XML container carrying the configuration cannot be simply passed as “LTE” XML container, given the different sizes. Also, given the different orders of the IEs in the current configurations in the two RAT, it seems that a completely new IE needs to be defined in NR. This does not sound as an elegant solution (to say the least).

In short, Option 3 seems achievable without impact on LTE specification. However, it would require quite some effort in RAN3 to discuss the details for the solution.

Proposal 11: RAN3 to further discuss how to support Option 3 (HO from NR to LTE/5GC).

# Conclusion

In this paper we discuss the enhancements of Rel-17 features. The following was observed and proposed:

Proposal 1: Introduce a new class-2 F1AP UE-associated procedure to control the transfer of RVQoE information from the CU to the DU.

Proposal 2: Introduce a new class-1 UE-associated F1AP procedure) initiated by the CU, where:

- The CU indicates to the DU the available RVQoE metrics and the intended reporting periodicity.

- The DU indicates to the CU its preferred available RVQoE metrics and its preferred reporting periodicity.

Proposal 3: Agree the TP for QoE BL CR for TS 38.473 in the Annex.

Observation 1: RVQoE reports triggered by radio related events (as defined in TS 38.331) can be used by the gNB to optimize mobility decisions.

Proposal 4: RVQoE reporting can be triggered by a radio related event (as defined in TS 38.331). Details are FFS.

Observation 2: Introduction of event-based RVQoE trigger has no impact on the application layer.

Proposal 5: The UE should indicate to the network the event that triggered the RVQoE reporting.

Proposal 6: When threshold-based RVQoE reporting starts, the reports start to be sent immediately, and are sent periodically.

Proposal 7: Threshold-based RVQoE reporting can be stopped by modifying the RVQoE configuration. Otherwise, the RVQoE reporting continues until session end.

Proposal 8: Send an LS asking RAN2 to specify in RRC signalling the buffer level threshold for RVQoE reporting expressed in terms of remaining playout time of the content currently in the buffer.

Proposal 9: RAN3 to discuss whether assistance information for handling of QoE reporting upon RAN overload should include the type of consumer that will receive the QoE reports, or some characteristics of the consumers.

Observation 3: The existing NR QoE configuration information cannot be transferred to LTE “as is”, and vice versa.

Observation 4: Current NR area scope IE contains no cell- or TA-related LTE information.

Observation 5: Current LTE area scope IE contains no cell- or TA-related NR information.

Observation 6: Information elements used both in the NR QoE configuration and in LTE QoE configuration are not compatible due to different sizes and presence.

Proposal 10: Option 4 (HO from LTE/5GC to NR) is not supported.

Proposal 11: RAN3 to further discuss how to support Option 3 (HO from NR to LTE/5GC).

# Annex: TP for QoE BL CR for TS 38.473

-------------------------------------------Start of changes-------------------------------------------

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| >>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<< |  |  |  |
| QMC Coordination | QMC COORDINATION REQUEST | QMC COORDINATION RESPONSE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| >>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<. |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |

-------------------------------------------Next change-------------------------------------------

### 8.16.x	QoE Information Transfer Control

#### 8.16.x.1		General

This procedure is initiated by the gNB-DU to control the transfer of RAN visible QoE information from the gNB-CU to the gNB-DU.

The procedure uses UE-associated signalling.

#### 8.16.x.2	Successful Operation

Figure 8.16.x.2-1: QoE Information Transfer Control procedure.

The gNB-DU initiates the procedure by sending the QOE INFORMATION TRANSFER CONTROL message to the gNB-CU.

If the gNB-CU receives the QoE Information Transfer Action IE in the QOE INFORMATION TRANSFER CONTROL message with the value set to “deactivate”, the gNB-CU shall, if supported, consider that the termination of the ongoing QoE information transfer is requested from the gNB-DU for this UE, and act as specified in TS 38.300 [6].

### 8.16.y	QMC Coordination

#### 8.16.y.1		General

The purpose of the QMC Coordination procedure is to enable coordination of RAN Visible QoE configuration parameters for a UE between a gNB-CU and a gNB-DU.

The procedure uses UE-associated signalling.

#### 8.16.y.2	Successful Operation

Figure 8.16.y.2-1: QMC Coordination procedure.

A gNB-CU initiates the procedure by sending the QMC COORDINATION REQUEST message to a gNB-DU over the F1 interface. The gNB-DU responds with a QMC COORDINATION RESPONSE, including the appropriate information.

If the QMC COORDINATION REQUEST contains the gNB-CU RAN Visible QoE Configuration List IE, the gNB-DU shall, if supported, include in the QMC COORDINATION RESPONSE the gNB-DU RAN Visible QoE Configuration List IE, containing a RAN visible QoE measurement configuration preferred by the gNB-DU.

If the QMC COORDINATION RESPONSE contains the gNB-DU RAN Visible QoE Configuration IE, the gNB-CU may use it according to TS 38.300 [6].

-------------------------------------------Next change-------------------------------------------

#### 9.2.16.z1	QOE INFORMATION TRANSFER CONTROL

This message is sent by the gNB-DU to control the transfer of RAN visible QoE information from the gNB-CU to the gNB-DU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Visible QoE Reporting Action List |  | 0..1 |  |  | YES | ignore |
| >RAN Visible QoE Reporting Action Item |  |  | 1..<maxnoofRVQoEConfigurations> |  | Each | ignore |
| >>QoE Reference | M |  |  | Indicates the RAN visible QoE measurement configuration for which the action applies. | - | - |
| >>QoE Information Transfer Action | M |  | ENUMERATED (deactivate, …) | Indicates an action to apply to the RAN visible QoE reporting. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofRVQoEConfigurations | Maximum no. of QoE information for one UE, the maximum value is 16. |

#### 9.2.16.z2	QMC COORDINATION REQUEST

This message is sent by the gNB-CU to provide the QMC coordination information to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| gNB-CU RAN Visible QoE Configuration List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU RAN Visible QoE Configuration Item |  |  | 1..<maxnoofRVQoEConfigurations> |  | Each | ignore |
| >>QoE Reference | M |  |  |  | - | - |
| >>gNB-CU RAN Visible QoE Configuration | M |  | 9.3.1.t | Indicates the preferred RAN visible QoE configuration for the gNB-CU. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofRVQoEConfigurations | Maximum no. of QoE information for one UE, the maximum value is 16. |

#### 9.2.16.z3	QMC COORDINATION RESPONSE

This message contains the response to the QMC COORDINATION REQUEST message.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| gNB-DU RAN Visible QoE Configuration List |  | 0..1 |  |  | YES | ignore |
| >gNB-DU RAN Visible QoE Configuration Item |  |  | 1..<maxnoofRVQoEConfigurations> |  | Each | ignore |
| >>QoE Reference | M |  |  |  | - | - |
| >>gNB-DU RAN Visible QoE Configuration | O |  | 9.3.1.t | Indicates the preferred RAN visible QoE configuration for the gNB-DU. If absent, the gNB-DU is not interested to received RAN visible QoE measurements for the UE. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofRVQoEConfigurations | Maximum no. of QoE information for one UE, the maximum value is 16. |

-------------------------------------------Next change-------------------------------------------

#### 9.3.1.t	RAN Visible QoE Configuration

This IE contains the information about a RAN visible QoE measurement configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Buffer Level | M |  | ENUMERATED (true, ...) | This IE defines whether the Buffer Level can be collected as a RAN visible QoE metric from UE, for DASH streaming and VR service types. |
| Playout Delay for Media Startup | M |  | ENUMERATED (true, ...) | This IE defines whether the Playout delay can be collected as a RAN visible QoE metric from UE, for DASH streaming and VR service types. |
| Reporting Periodicity | O |  | ENUMERATED (ms120, ms240, ms480, ms 640, ms1024, …) | This IE indicates the periodicity for the reporting of RAN visible application layer measurements. Value ms120 indicates 120 ms, value ms240 indicates 240 ms and so on. |

-------------------------------------------End of changes-------------------------------------------