3GPP TSG-RAN3 #119-bis-e	R3-231543

E-meeting, 17th April – 26th April 2023

Title: 	Analysis of ACL remaining issues

Source: 	Ericsson, Deutsche Telekom, Huawei, China Telecom

Agenda item:	9.2.4

Document Type:	Discussion and Decision

# Introduction

In past meetings support of the dynamic ACL during handover and dual connectivity was discussed. In the following we will further elaborate on some remaining issues. Based on our analysis we will bring forward our proposals.

# Discussion

Most of the cases for ACL both regarding HO and dual connectivity have been analysed and well covered in previous meetings. We have identified though some cases that are not covered by the current solutions. Below we touch upon them one by one.

E-UTRAN Inter-Master Node handover with/without Secondary Node change

In the case of inter-Master Node handover with/without Secondary Node change the user plane IP information exchange for dynamic ACL is allowed for almost all supported data forwarding scenarios. The only scenario not supported concerns the case of S-SN -> T-MN direct data forwarding.

Conclusion 1:  In the case of E-UTRAN inter-Master Node handover with/without Secondary Node change the user plane IP information exchange for dynamic ACL is not allowed for the S-SN -> T-MN direct data forwarding.

The best way to support also this case would be, as was already done for other cases, to carry the source SN’s IP address in the Handover Request message.

E-UTRAN Master Node to eNB Change

In the case of EN-DC to standalone eNB handover the user plane IP information exchange for dynamic ACL is allowed for almost all supported data forwarding scenarios. The only scenario not allowed concerns the case of S-SN -> T-eNB direct data forwarding.

Conclusion 2:  In the case of E-UTRAN Master Node to eNB Change the user plane IP information exchange for dynamic ACL is not allowed for the S-SN -> T-eNB direct data forwarding.

The best way to support also this case would be, as above, to carry the source SN’s IP address in the X2: Handover Request message.

In light of the two use cases above the following is proposed:

Proposal 1: Include the source SN’s IP address in the X2: Handover Request message.

E-UTRAN RRC re-establishment

In the case of RRC re-establishment if the UE Context is not locally available, the new serving eNB requests the last serving eNB to provide the UE Context data by means of the Retrieve UE Context procedure. As a consequence of re-establishment and UE context Retrieval, the new serving eNB may receive forwarded data from the last serving eNB after sending the DATA FORWARDING ADDRESS INDICATION message where the destination IP addresses for data forwarding are included. The support for dynamic ACL is missing in that case.

Conclusion 3:  The support for dynamic ACL is missing for the case of E-UTRAN UE context retrieval in RRC re-establishment and transmission of forwarded data from last serving to new serving eNB.

The best way to support this case would be to include the last serving eNB´s source IP to be used for data forwarding in the RETRIEVE UE CONTEXT RESPONSE message.

Proposal 2: Include last serving eNB´s source IP address to be used for data forwarding in the X2: RETRIEVE UE CONTEXT RESPONSE message.

# Conclusion

In this contribution, we have discussed some remaining ACL issues. Corresponding conclusions and proposals are reported below.

Conclusion 1:  In the case of E-UTRAN inter-Master Node handover with/without Secondary Node change the user plane IP information exchange for dynamic ACL is not allowed for the S-SN -> T-MN direct data forwarding.

Conclusion 2:  In the case of E-UTRAN Master Node to eNB Change the user plane IP information exchange for dynamic ACL is not allowed for the S-SN -> T-eNB direct data forwarding.

Conclusion 3:  The support for dynamic ACL is missing for the case of E-UTRAN UE context retrieval in RRC re-establishment and transmission of forwarded data from last serving to new serving eNB.

Proposal 1: Include the source SN’s IP address in the X2: Handover Request message.

Proposal 2: Include last serving eNB´s source IP address to be used for data forwarding in the X2: RETRIEVE UE CONTEXT RESPONSE message.

CRs mirroring the proposals above are presented in R3-231544 and R3-231545