3GPP TSG-RAN WG3 #119bis                                                 R3-231557

Online, 17th – 26th April 2023

Agenda Item:	10.2.5

Source:	CATT

Title:	Discussion on SON enhancement for NR-U

Document for:	Discussion

Introduction

In previous RAN3 meeting, we did not achieve agreement on MRO and MLB for NR-U. The open issues are collected as below:

MRO:

further enhancements for RLF report:

  - addition to RLF report of indications of number of consistent LBT failures and at which granularity (e.g., per BWP)

  - addition to RLF report of EDT in UL (e.g., exact value, average, max)

  - whether LBT configuration at network side is sufficient or should be added to RLF report

  - waiting time in uplink due to LBT

further enhancements of RA report:

  - information of LBT failures occurring during the RA procedure. FFS on the granularity of this information.

  - addition of EDT in UL from UE (and which value, e.g., exact value, average, max)

  - addition of Measured RSSI

  - addition of UL LBT duration time

improvements for SCG Failure Information:

  - Measured RSSI

  - SCG failure due to consistent LBT failure

whether and how, in case of handover, the target gNB can send to the source gNB indication of DL LBT failure. For example:

  - in the Xn message, sent post HO execution, which contains the RLF report

in an Xn message, sent post HO execution, which does not contain the RLF report

MLB:

FFS on whether the values for COT UL and the EDT UL in resource status reporting to be used for MLB can be obtained by the gNB in an implementation specific way and/or based on COT UL and EDT UL provided by the UEs.

FFS whether to add in F1AP within the RESOURCE STATUS UPDATE message, a Channel Occupancy Time Percentage UL IE and/or an Energy Detection Threshold UL IE as sub-IEs of NR-U Channel Item IE.

Continue to discuss whether and how to report COT in UL and COT in DL for neighbor cells

FFS whether and how to report NR-U metrics with SSB beam level granularity.

In the document, we provide some analysis on the open issue on MRO and MLB for NR-U.

Discussion

  - MRO for NR-U

Before discussing the open issue, we may first clarify the objective of MRO for NR-U.

For legacy MRO, handover configuration is optimized to solve too early/too late/to wrong cell failure type which may lead to RLF or handover failure. But we notice that access to NR-U may also lead to RLF or handover failure because of occupancy by other network, i.e. it does not need to optimize handover configuration for the failure caused by NR-U.

For example:

1. UE0 handover from cell0 to cell1 fails and then initiate RRC reestablishment to cell2. After UE sent RLF Report to network, a handover to wrong cell failure type is detect which requires MRO for handover configuration.

2. UE1 handover from cell0 to cell1 fails due to the reason of consistent LBT failure, and then initiate RRC reestablishment to cell2. After UE sent RLF Report to network, network should not perform MRO for handover configuration because cell1 may have good signal quality and should be select as suitable handover target cell but the cell has been occupied by NR-U.

Observation 1: It does not need to optimize handover configuration for the failure caused by NR-U.

When analyzing RLF Report, one bit indicator is needed to identify whether the failure is caused by NR-U. We have agreed to introduce consistent LBT failures which can indicate the failure is caused by NR-U.

Observation 2: “Indication of consistent LBT failure” indicates the failure is caused by NR-U.

Legacy MRO function will not be impacted by using one bit indicator in RLF Report.

Proposal 1: The first objective of MRO for NR-U is to exclude the impact of RLF Report caused by accessing to NR-U network. It does not need optimize handover configuration. One bit indicator in RLF Report is already introduced to indicate the failure is caused by NR-U.

The open issue related to RLF report is as below:

further enhancements for RLF report:

  - addition to RLF report of indications of number of consistent LBT failures and at which granularity (e.g., per BWP)

  - addition to RLF report of EDT in UL (e.g., exact value, average, max)

  - whether LBT configuration at network side is sufficient or should be added to RLF report

  - waiting time in uplink due to LBT

According to the objective of MRO for NR-U in P1 and P2, the enhancements can be classified into two kinds as below:

1. Indicator to identify NR-U caused failure

addition to RLF report of indications of number of consistent LBT failures and at which granularity (e.g., per BWP)

waiting time in uplink due to LBT

For the indications of number of consistent LBT failures, we have agreed to introduce the indicator of consistent LBT failures. We do not understand the benefit of introducing precise number.

Proposal 2: One bit indicator for consistent LBT failures is sufficient, and further enhancement of precise number is not needed.

For waiting time in UL due to LBT, the first issue is how to define the waiting time.

In last RAN3 meeting, some companies propose to calculate the wait time by time point t1 – time point t2.

For example, UE want to send measurement report to trigger handover. t1 is the time point measurement report is triggered and t2 is the time point when measurement report is successfully sent to network. We think this solution cannot work for the following reason:

- For example, there are 5 times of UL LBT before successfully sending measurement report to network. 1 LBT fail ->2 LBT succeed but sending message fail ->3 LBT fail ->4 LBT fail ->5 LBT succeed and successfully sending message to network. For the second LBT, although LBT is OK, measurement report finally failed to be sent to network due to uplink radio link failure. So, the wait time (t1 – t2) is from 1 LBT fail to 5 LBT succeed and successfully sending message to network. During this period of time there are 3 times LBT failure and 1 time uplink radio link failure. In other words, (t1 - t2) is not only caused by LBT failure and it is unsuitable to use (t1-t2) for LBT wait time.

- Even assuming all failure is caused by LBT, i.e. if LBT success, UE always can sent measurement report successfully to network, it is still unsuitable to define wait time as (t1-t2). If LBT fail, UE has to wait for the next UL available occasion to trigger LBT. It is quite complicate to calculate the next available UL occasion for UL LBT which is up to many factors, for example, UE capability, SchedulingRequestConfig, frame structure, etc. So, a bigger (t1-t2) may be caused by a longer SR configuration, fewer UL available occasion, etc. we also cannot use (t1-t2) for LBT failure detection.

In one word, there may be many reasons can impact the value (t1-t2) and LBT failure is only one of them. So, we propose to not define waiting time because it is too complicated.

Proposal 3: We do not agree to introduce waiting time in UL for the following two reasons:

1. It is hard to define waiting time in UL.

2. Waiting time in UL due to LBT is also used to indicate whether channel is occupied in UL. We agreed to introduce the indication of consistent LBT failure, so waiting time in UL is not needed.

2. NR-U related configurations

addition to RLF report of EDT in UL (e.g., exact value, average, max)

whether LBT configuration at network side is sufficient or should be added to RLF report

Some companies also propose to optimize NR-U configuration. We think MRO mainly focus on mobility configuration optimization while NR-U configuration impact both mobile and static UE. For this kind of common configuration, we do not think they are in the scope of MRO since we can optimize this kind of configuration in non-mobile state which is relatively simple. But RAN3 can continue discussing these configurations as the open issues.

Observation 3: The objective of MRO for NR-U is not NR-U related configuration optimization because these configurations impact both mobile and static UE and can be optimized in non-mobile scenarios.

As for including EDT in UL in RLF report, we first look at how UE generate EDT in UL.

After receiving energyDetectionConfig from network, UE may select its own EDT based on it. For example, maxEnergyDetectionThreshold in energyDetectionConfig, as the text in TS37.213:

A UE accessing a channel on which UL transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold XThresh_max

Some companies think it is useful for UE to send its own EDT in UL to network in order to optimize energyDetectionConfig. We think access successful rate and LBT detection result would be used to optimize energyDetectionConfig. The channel is considered to be idle if the energy detected by UE is less than energy detection threshold, i.e. EDT in UL. Otherwise, the channel is considered busy. Therefore, if the EDT in UL is set by UE a little bit higher than normal, channel is more likely to be considered as idle, but it may lead to access failure due to incorrect LBT detection result. On the contrary, if the EDT in UL is set lower, most of time channel is more likely to be considered as busy which will lead to more LBT detection failure. Network can count access successful rate and LBT detection result for a period of time and set a suitable energyDetectionConfig to achieve balance between access successful rate and LBT detection result, i.e. EDT in UL is not needed to optimize energyDetectionConfig.

Some companies believe EDT in UL can be used to detect the strength of signal, but we think RSSI measurement result has been introduced for Signal Strength in RLF Report, so, EDT in UL is not needed.

Observation 4: Access successful rate and channel occupation detection result shall be used to optimize energyDetectionConfig rather than EDT in UL.

Proposal 4: It is proposed to not include EDT in UL in RLF Report.

further enhancements of RA report:

- information of LBT failures occurring during the RA procedure. FFS on the granularity of this information.

- addition of EDT in UL from UE (and which value, e.g., exact value, average, max)

- addition of Measured RSSI

- addition of UL LBT duration time

For measured RSSI in RA report, we think it is better to include measured RSSI in RLF Report.

For LBT duration time and EDT in UL from UE, as discuss before, we do not think it is needed.

Proposal 5: It is proposed to not include measured RSSI in RA Report because it has been included in RLF Report.

Proposal 6: It is proposed to not include LBT during time and EDT in UL from UE in RA Report for the same reason as P3 and P4.

For the granularity of LBT failure, either per RA attempt or per RA procedure shall be selected?

According to TS38.321, for each Random Access Preamble transmission, LBT failure indication may be received by UE MAC. Let’s consider an example of RACH procedure during handover, before T304 expires, UE perform 4 times RA attempt: 1 LBT fail ->2 LBT succeed ->3 LBT fail ->4 LBT succeed.

For the second and fourth RA attempt, although LBT succeed, RACH may fail due to other reason. If introducing LBT failure per RA procedure, it is hard to say that RA procedure failure is caused by LBT failure because LBT is successful finally. It also hard to say that RA procedure failure has no impact on LBT failure because LBT may fail in most of RA attempts and cause there is no enough time for the left RA procedure. Only all of the LBT failure during a RA procedure, we can decide a LBT failure for a RA procedure.

Observation 5: During a RA procedure, there would be many times of RACH attempt. Only all of RACH attempt failed due to LBT failure, we can decide it is a LBT failure for a RA procedure.

It is hard to decide whether LBT failure or other reason leads to RA procedure fail when there is partial LBT fail and partial LBT success in a failure RA procedure. The better method is to record each LBT result for RA attempt.

Proposal 7: There are many times of RACH attempt during a RA procedure which may be partial LBT fail and partial LBT success. It is better to indicate LBT failure per RA attempt in RA Report.

whether and how, in case of handover, the target gNB can send to the source gNB indication of DL LBT failure. For example:

  - in the Xn message, sent post HO execution, which contains the RLF report

  - in an Xn message, sent post HO execution, which does not contain the RLF report

During RACH procedure, network shall perform DL LBT before sending MSG2 when receiving MSG1. While for contention-based RACH, network does not aware of UE ID at that time and cannot record DL LBT failure information. After receiving MSG3, network can identify the UE ID and begin to record LBT failure information. So, for contention-based RACH procedure, target RAN node may be not able to collect complete LBT failure information.

Observation 6: During contention-based RACH procedure, target RAN node may be not able to collect complete LBT failure information.

Proposal 8: It is proposed not to introduce DL LBT failure because target RAN node may be not able to collect complete LBT failure information in contention-based RACH procedure.

improvements for SCG Failure Information:

  - Measured RSSI

  - SCG failure due to consistent LBT failure

We are ok to introduce above information in SCG failure information message, but we may wait for the result of RLF Report, and then discuss SCG failure case.

Proposal 9: It is proposed to wait for the result of RLF Report, and then discuss SCG failure case.

  - MLB for NR-U

FFS on whether the values for COT UL and the EDT UL in resource status reporting to be used for MLB can be obtained by the gNB in an implementation specific way and/or based on COT UL and EDT UL provided by the UEs.

FFS whether to add in F1AP within the RESOURCE STATUS UPDATE message, a Channel Occupancy Time Percentage UL IE and/or an Energy Detection Threshold UL IE as sub-IEs of NR-U Channel Item IE.

DU shall report the UL COT to CU in resource status report procedure when it is requested by CU, how to obtained this information by DU is up to implementation.

The granularity of UL COT reporting in F1 interface should be NR-U channel level.

In the previous meetings, we added COT UL and the EDT UL in resource status reporting in XnAP and FFS on whether they can be obtained by gNB or based on UE report. Given that the gNB only knows the max EDT in UL, the main point is whether gNBs need to exchange the exact EDT in UL via Xn. If we support exact EDT in UL, UE has to report the exact value, it may increase UE complexity. Also, it may not very useful because the selection of EDT in UL will not have a significant difference between UEs. The EDT in UL reflects the threshold about whether the NR-U channels can be access hence small different between UEs will not impact neighbour cell decision too much. Generally, we propose to transfer the average EDT in UL to neighbour gNB and how to obtain average EDT in UL is based on gNB implementation.

Proposal 10: It is propose to transfer the average EDT in UL to neighbour gNB and how to obtain average EDT in UL is based on gNB implementation.

Continue to discuss whether and how to report COT in UL and COT in DL for neighbor cells

The NR-U channel can be occupied/shared by more than one cells e.g., cell 1 and cell 2 shared NR-U channel 1. In mostly case, neighbour cell 3 will receive two cell measurement result items that indicate the COT and EDT of cell 1 and cell 2 in NR-U channel 1, respectively. And neighbour cell 3 can consider them together.

For example, neighbour cell 3 receives (cell1 NR-U channel 1: COT 20%, EDT -80) and (cell 2 NR-U channel 1: COT 80%, EDT -70), neighbour cell 3 may decide not to offload traffic to cell 2 and choose cell 1 as target cell. In general, neighbour cell 3 does not care about who occupied NR-U, it only considers whether this cell is frequently occupied by NR-U by analyse COT and EDT and perform offloading.

Proposal 11: It does not need to report COT in UL and COT in DL for neighbour cells because cell/gNB only need to know whether neighbour cell is occupied by NR-U.

Conclusions

Observation 1: It does not need to optimize handover configuration for the failure caused by NR-U.

Observation 2: “Indication of consistent LBT failure” indicates the failure is caused by NR-U.

Proposal 1: The first objective of MRO for NR-U is to exclude the impact of RLF Report caused by accessing to NR-U network. It does not need optimize handover configuration. One bit indicator in RLF Report is already introduced to indicate the failure is caused by NR-U.

Proposal 2: One bit indicator for consistent LBT failures is sufficient, and further enhancement of precise number is not needed.

Proposal 3: We do not agree to introduce waiting time in UL for the following two reasons:

1. It is hard to define waiting time in UL.

2. Waiting time in UL due to LBT is also used to indicate whether channel is occupied in UL. We agreed to introduce the indication of consistent LBT failure, so waiting time in UL is not needed.

Observation 3: The objective of MRO for NR-U is not NR-U related configuration optimization because these configurations impact both mobile and static UE and can be optimized in non-mobile scenarios.

Proposal 4: It is proposed to not include EDT in UL in RLF Report.

Proposal 5: It is proposed to not include measured RSSI in RA Report because it has been included in RLF Report.

Proposal 6: It is proposed to not include LBT during time and EDT in UL from UE in RA Report for the same reason as P3 and P4.

Observation 4: Access successful rate and channel occupation detection result shall be used to optimize energyDetectionConfig rather than EDT in UL.

Observation 5: During a RA procedure, there would be many times of RACH attempt. Only all of RACH attempt failed due to LBT failure, we can decide it is a LBT failure for a RA procedure.

Proposal 7: There are many times of RACH attempt during a RA procedure which may be partial LBT fail and partial LBT success. It is better to indicate LBT failure per RA attempt in RA Report.

Observation 6: During contention-based RACH procedure, target RAN node may be not able to collect complete LBT failure information.

Proposal 8: It is proposed not to introduce DL LBT failure because target RAN node may be not able to collect complete LBT failure information in contention-based RACH procedure.

Proposal 9: It is proposed to wait for the result of RLF Report, and then discuss SCG failure case.

Proposal 10: It is propose to transfer the average EDT in UL to neighbour gNB and how to obtain average EDT in UL is based on gNB implementation.

Proposal 11: It does not need to report COT in UL and COT in DL for neighbour cells because cell/gNB only need to know whether neighbour cell is occupied by NR-U.

# 4. Reference