3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231721

April 17-26, 2023

Agenda item:	14.4

Source:	Samsung

Title:	Considerations on selective activation of the cell groups

Document for:	Discussion & Decision

Introduction

In RAN3 119, the following agreements were achieved.

Enhance XnAP and F1AP signaling to support NR Selective Activation.

Introduce a new indicator to the S-NODE ADDITION REQUEST message over Xn to indicate that the request is for Selective Activation.

Further check the activation/deactivation of candidate PSCells for NR Selective Activation.

Keep the WA for early data forwarding.

Wait for RAN2’s progress on the scenarios.

In this contribution some considerations will be given from our side.

Discussion

On Enhance signalling for Selective Activation

In RAN3, the following working assumption is agreed.

WA: RAN3 will work to enable both indirect and direct early data forwarding in Selective Activation. At this moment, RAN3 does not foresee any scenarios where direct forwarding is not feasible/desired.

According to the working assumption, early data forwarding is agreed to be supported.

Therefore in the selective activation of SCGs if CPC procedure is executed, the source SN should perform early data forwarding to the candidate SNs by indirect or direct data forwarding mechanism. And the candidate SNs should will store the received data and send to UE if UE selects it as the target SN.

After the CPC execution, UE accessing to the target SN, the data forwarded to the other candidate SNs seems no need to be stored and should be discarded to avoid as the new data sent to UE when UE accessing to one of them as the new target SN. Therefore, there needs an indication sent to the candidate SNs to discard the data stored after UE accessing to the target PSCell by CPC execution.

Observation1: There needs an indication sent to the candidate SNs to discard the data stored after UE accessing to the target PSCell by CPC execution.

Proposal1: Enhance XnAP and F1AP signaling to support the candidate SNs to discard the stored data received from early data forwarding.

Activation and deactivation of candidate PSCells

According to the conclusion of the last meeting, this issue needs further check.

In selecctive activation of SCGs, to reduce the signalling overhead and interrupting time for SCG change, candidate PSCells are pre-configured for CPAC and kept by UE for the subsequent CPAC procedures after the execution of CPAC.

With UE’s mobility, there might be no proper candidate PSCell in the pre-configured candidate pscells for the subsequent CPC procedure, such as the quality of the candidate cell is not good enough or the load of the candidate cell is heavy. Therefore, the operations to modify or release the prepared PSCells will result in RRC signaling overhead.

If the more candidate PSCells can be pro-configured, UE will have enough proper candidate PSCells for the subsequent CPC procedures. According to UE’s capability, the network could select the proper number of  cells for UE for the subsequent CPC procedure from the pre-configured candidate PSCells if the candidate PSCell is selected for the subsequent, it is set in activated state,  otherwise it is set in deactivated state, which is the the mechanism of activation and deactivation of candidate PSCells.

With the activation/deactivation mechanism, UE only needs to evaluate the candidate PSCells in the activated state for the subsequent CPC procedure. The selected PSCells to be activated can be based on such as UE’s location, the overload of the cell.

The mechanism of activation/deactivation of the candidate PSCells is like that of SCell activation/deactivation in carrier aggregation. The state of the candidate PSCells can be indicated to UE by MAC CE or L1 signalling, therefore the signalling overhead to modify/release the pre-configured candidate PSCells can be reduced.

Observation 2: With activation and deactivation of candidate PSCells, the signaling overhead to modify/release the pre-configured candidate PSCells can be reduced.

Proposal 2: Support activation/deactivation of candidate PSCells in Selective Activation of the cell groups.

Proposal 3: RAN3 study the impact work on Xn and F1 to support activation/deactivation of candidate PSCells.

Discussion for the WA “last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation”

For the below WA which is noted with “up to RAN2’s discussion” and no further decision now.

WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation.

However after checking RAN2#121 meeting, RAN2 made one following agreement on the subsequent CPAC after SCG change [1].

It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell.

Based on RAN2’ agreement, we would propose this WA into Agreement.

Proposal 4: RAN3 agree that the network could explicitly configure last serving (source) PSCell as a candidate PSCell for subsequent SCG change.

Further checking, we can start to discuss when to configure UE on source PSCell can be used one candidate PSCell for the subsequent SCG change. After looking through the whole subsequent SCG change procedure (e.g., MN initiated subsequent SCG change procedure in Fig.1), we can see that possible configuration time is before or after execution of CPC.

-Alternative#1: configuration time is before CPC execution

Even though UE received execution condition of source PSCell in advanced, UE shouldn’t evaluate the CPC execution condition of the source PSCell until the source PSCell becomes candidate PSCell after CPC execution.

-Alternative#2: configuration time is after CPC execution

If network want to configure it as one candidate PSCell for subsequent SCG change, the UE receives the configuration of the source PSCell after CPC execution. And then UE starts to evaluate the CPC execution condition of source PSCell.

Fig 1.MN initiated subsequent SCG change procedure

Compared with the above two schemes, we prefer the Alternative#1 because source PSCell can be configured using the same RRCReconfiguration message simultaneously including configuration of other candidate PSCells. However, in Alternative#2, a separate RRCReconfiguration will be needed for configuration of source PSCell. But we would like to say that when to configure source PSCell is decided by network, it is possible for network to configure it before or after CPC execution.

Based on above discussion in this section 2.3, we have a below proposal on configuration time for NW to configure initial source PSCell as one of the candidate PSCells for the subsequent SCG change.

Proposal 5: When does NW explicitly configure initial source PSCell as one of the candidate PSCells for the subsequent SCG change is FFS.

Conclusion

Based on the discussion, we have the following proposals.

Observation1: There needs an indication sent to the candidate SNs to discard the data stored after UE accessing to the target PSCell by CPC execution.

Proposal1: Enhance XnAP and F1AP signaling to support the candidate SNs to discard the stored data received from early data forwarding.

Observation 2: With activation and deactivation of candidate PSCells, the signaling overhead to modify/release the pre-configured candidate PSCells can be reduced.

Proposal 2: Support activation/deactivation of candidate PSCells in Selective Activation of the cell groups.

Proposal 3: RAN3 study the impact work on Xn and F1 to support activation/deactivation of candidate PSCells.

Proposal 4: RAN3 agree that the network could explicitly configure last serving (source) PSCell as a candidate PSCell for subsequent SCG change.

Proposal 5: When does NW explicitly configure initial source PSCell as one of the candidate PSCells for the subsequent SCG change is FFS.

References

Chairman notes for RAN2#121 meeting	RAN2.