3GPP TSG-RAN3 Meeting #119bis-e	R3-231757

# Online, April 17th –April 26th , 2023

Agenda Item:	17.2

Source: 	Huawei

Title: 		Further discussion on NG HO

Document for:	Discussion

Introduction

In the last meeting the following agreement has been achieved：

Agreement： Introducing time-based parameters for NG HO follows legacy CHO configuration over Uu interface without any RAN2 impact.

In this contribution, we will further analyse potential impacts to NG.

Discussion

Xn interface

One remaining issue which has not been discussed in length is whether Xn interface will be deployed in a transparent payload scenario.

In our understanding, whether Xn interface between NTN gNBs over long distance could exist depends on the real deployment of different operators. For now, there is no proof that the Xn interface will never be deployed. There is nothing preclude Xn deployment in rel-17. Hence, unless strong concerns are raised from operators, we may slightly de-prioritize Xn but not preclude it in Rel-18.

Proposal 1: The existence of Xn interface depends on real deployment of different operators, which should not be precluded in Rel-18.

NG for NTN-NTN hand-over in addition to T1 and T2

In the last meeting, it was agreed that time-based parameters for NG HO follows legacy CHO configuration over Uu interface without any RAN2 impact. In the following, we analyze potential procedures which may need an enhancement, while keep in mind the IEs are introduced in the source to target container.

- Parallel transaction:

There is only one handover preparation procedure ongoing at the same time to a single cell for a certain UE according to the latest TS 38.413.

Figure 1: Parallel Transactions

As illustrated in Figure 1, if multiple candidate cells are permitted, parallel transaction for the UE handover to the target gNBs should be supported. To support parallel transaction, core network should be able to uniquely identify each handover procedure.

The existing mechanism allows the source gNB and source AMF to allocate respectively the RAN UE NGAP ID and AMF UE NGAP ID to the UE during UE context setup procedure, where the two IDs can uniquely identify one UE in gNB side and core network side correspondingly. However, if the per-cell HANDOVER REQUIRED messages are sent to multiple candidate cells which belong to the same target gNB, refer to Xn CHO procedure, we may introduce target cell global ID (CGI) in HANDOVER REQUIRED and HANDOVER REQUEST message to avoid confusion.

Furthermore, in Xn CHO, different with legacy HO, the candidate target cell CGI(s) are included in the HO REQUEST ACK and HANDOVER COMMAND messages. The purpose of transferring the CGI to source gNB is to avoid confusion in source gNB side when there are several HO REQUEST ACK messages from different cells of the same target gNB. Also, transferring CGI helps source to generate different conditional execution conditions for each cells. Therefore, RAN3 should also discuss the necessity and feasibility of introducing target cell CGI in HANDOVER COMMAND and HANDOVER REQUEST ACK message.

Observation1: If more than one candidate cell is supported in the time-based NG HO, enhancement to parallel transaction should be considered, e.g., include the target CGI IE in the whole HO preparation procedures.

- Handover success and Handover Cancel procedure

HANDOVER SUCCESS procedure is used during a conditional handover to enable a target gNB to inform the source gNB node that the UE has successfully accessed the target gNB. In return, the source gNB sends the SN   STATUS TRANSFER message or initiates late data forwarding as soon as the source gNB receives the HANDOVER SUCCESS message.

We note that NG DAPS procedure has introduced the HANDOVER SUCCESS message, which can be reused with some IE enhancement dedicated for the time-based NG HO, i.e., adding the Requested Target Cell ID to the message follow same principle as Xn CHO. With the “Requested Target Cell ID”, the source gNB can know which cell it should forward the data to and which cell(s) need to be informed to cancel the time-based HO.

In the meanwhile, there seems less urgency to introduce also the HANDOVER CANCEL message, since the candidate gNBs can anyway cancel time-based NG HO after T2 by themselves. We note in CHO, the CONDITIONAL HANDOVER CANCEL message can also be used by the target node to trigger the CHO modification. In our view, for NG interface, the NG HO is not the full duplication of Xn CHO, and there is no need to support similar HO modification feature.

Observation2: Consider reusing HANDOVER SUCCESS message introduced for NG DAPS by adding the CGI of the successfully attached target cell.

Observation 3: There is no need to introduce HANDOVER CANCEL message over NG interface.

- Data forward enhancement

CN needs to establish more than one forward tunnels with several target candidate gNBs per PDU session if the tunnel is a PDU session-level forward tunnel or per DRB if the tunnel is a DRB level forward tunnel during CHO preparation. Up to now during legacy handover CN just needs to prepare one forward tunnel per PDU session or per DRB with one target gNB for one UE. SA2 needs to evaluate whether multiple forwarding tunnels can be established with multiple gNBs for one PDU session or for one DRB. If there is no support of multiple forwarding tunnels, packet loss can happen.

Observation 4: RAN3 considers to send LS to SA2 to evaluate whether multiple forwarding tunnels can be established with multiple gNBs for one PDU session or for one DRB.

Based on above analysis, there are three aspects to be analysed before time-based NG HO can be introduced, if the two IEs are added for a purpose similar to CHO.

Proposal 2: RAN3 should discuss if any enhancement is needed for the time-based Handover

There is another use of time-based parameters which will introduce less impact compared to the above time-based NG HO and more efficiently. That is, the time-based parameters are only introduced for resource reservation, and source gNB instead of UE decides when to do the HO execution. Below is the details.

- IEs introduced for resource reservation in legacy HO（time-based HO）

According to our understanding, when two time-related IEs are used for legacy HO, they are passed to the target node to pre-configure handover resources for UE in advance. The UE will appear in the target cell within the period of [T1, T2] and the candidate nodes only reserve the resource for the period T1 and T2. Then the source node makes the handover decision by choosing one of the candidate cells as the target cell and sends the handover command to UE directly. In addition to the benefit of resource reservation, another benefit is reducing the signaling exchange time after a handover decision made by the source node, thus the source can immediately hand over the UE to one of the target, which speeds up the handover procedure. The below figure depicts the possible handover procedure after introducing [T1, T2] for legacy NG HO.

1.	Source gNB to AMF: Source gNB sends Handover Required message including two time-related IEs which are contained in Source NG-RAN Node to Target NG-RAN Node Transparent Container IE to request the preparation of resource at the candidate target gNB(s) via 5GC.

Note: there is nothing precluding parallel transaction for the UE handover to the target gNB.

2.	AMF to candidate target gNB(s): AMF sends Handover Request message including two time-related IEs to the candidate target gNB(s) to reserve resources at the target node(s) for the handover of the UE.

3.	The candidate target gNB(s) to AMF: The candidate target gNB(s) use the time-related information to allocate necessary resource for incoming HO. Furthermore, after receiving the time-related IEs, the candidate target node(s) will be aware of that UE will appear in the target cell within the period of T1 and T2 and the candidate nodes only reserve the resource dedicated for the period T1 and T2. The candidate gNB(s) sends HANDOVER REQUEST ACKNOWLEDGE message to AMF including a transparent container to be sent to the UE as an RRC message to perform the handover of candidate cell(s).

Note: the targets reserve resources well before T1 as soon as they receive the handover request, but the resources reserved are used for [T1, T2].

4.	The AMF to source gNB: AMF sends the HANDOVER COMMAND messages including a transparent container of the configurations of candidate cell(s) to the source gNB.

5.	The source gNB evaluates whether the UE meets handover requirements, e.g., evaluate by RRM measurement report or by location report or other condition. The source gNB makes the handover decision by choosing one of the candidate cell(s) as target cell and the handover execution should occur during the time of [T1, T2].

6.	The source node to UE: the source gNB triggers the Uu handover by sending an RRCReconfiguration message to the UE, containing the configuration of the target cell which is received in step 4. Data forwarding procedure is same as legacy NG HO. There is no need to enhance the data forwarding procedure.

In current spec, as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

In the time-based HO procedure, it’s better to initiate the data forwarding as soon as the transmission of the RRCReconfiguration message containing reconfigurationWithSync, so data forwarding can be re-used directly without any enhancement.

Note: We do understand that the time based information for legacy NG HO does not constraint the target node to maintain the full resources during this timeslot. The resource management e.g. buffer size, timer etc is let to the implementation of the target node. If this is not clear we suggest to make it clear in the specifications …

7.	The source node to candidate node(s): the source node may initiate Handover Cancel procedure to cancel the resource reservation towards the candidate nodes which prepared the resources before but not choosing by the source node for UE to access. But this step is optional as if UE doesn’t appear in the target cell before the time duration [T1, T2], the target shall release any resources previously reserved for the candidate cells directly.

With regards to the UE configuration, at end of the procedure. We do understand that the time-related information is on top of legacy HO over NG (not CHO) then the UE should be configured as usual legacy procedure.  There is no need to involve other groups like RAN2 and SA2.

To conclude, after detailed analysis, to avoid big impacts to NG and other WGs, RAN3 should consider using the time-related IE for pre-configuration

Proposal 3:  RAN3 to also consider use the time-related IEs for pre-configuration.

Summary

Based on the discussion, we have following observations and proposals:

Observation1: If more than one candidate cell is supported in the time-based NG HO, enhancement to parallel transaction should be considered, e.g., include the target CGI IE in the whole HO preparation procedures.

Observation2: Consider reusing HANDOVER SUCCESS message introduced for NG DAPS by adding the CGI of the successfully attached target cell.

Observation 3: There is no need to introduce HANDOVER CANCEL message over NG interface.

Observation 4: RAN3 considers to send LS to SA2 to evaluate whether multiple forwarding tunnels can be established with multiple gNBs for one PDU session or for one DRB.

Proposal 1: The existence of Xn interface depends on real deployment of different operators, which should not be precluded in Rel-18.

Proposal 2: RAN3 should discuss if any enhancement is needed for the time-based Handover.

Proposal 3:  RAN3 to also consider use the time-related IEs for pre-configuration.