3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231792

Online, April 17–26, 2023

Agenda Item:	10.2.1

Source:	CMCC

Title:	SON enhancement for SPR

Document for:	Discussion and Decision

# 1 Introduction

At last RAN3 meeting, some agreements on Successful PSCell Change Report were achieved:

SPR:

For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis.

For intra-SN classic PSCell change/CPC, the source SN decides SPR triggers of T310 and T312 and performs root cause analysis.

If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations.

But there is still discussion on which node decides SPR triggers of T310 and T312 and which node performs root cause for MN-initiated classic PScell change /CPC and no conclusion achieved.

In this contribution, we would further discuss on MN-initiated classic PScell change /CPC and provide some proposals on this open issue for SPR.

# 2 Discussion

Based on the discussion in last meeting, the following have been captured by Chair.

FFS Which Node decides the triggers and which node performs root cause?

Option 1: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis

Option 2: For MN-initiated classic PScell change /CPC, the source SN node decides the T310/T312 triggers and performs root cause analysis

Option 3: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis

For SON enhancements such as inter-RAT SHR and intra-SN classic PSCell change/CPC, normally, the node that initiates the procedure should decide the triggers and perform root cause analysis for the related parameters and configuration optimization. For MN-initiated classic PScell change /CPC, the PScell change procedure is initiated by MN and if the trigger is T312/310, the optimization of the related parameters and configuration in MN should be considered.

Observation 1: For MN-initiated classic PScell change /CPC, the PScell change procedure is initiated by MN and if the trigger is T312/310, the optimization of the related parameters and configuration in MN should be considered.

In this case, although Option2 (the source SN node decides the T310/T312 triggers) can simplify the generation of T310/T312 triggers condition, other issues arising from Option2 need to be taken into account. For example, how the source SN node decides the proper T310/T312 trigger threshold for MN-initiated classic PScell change /CPC. If the source SN node simply configure the same value of T310/T312 trigger threshold for MN-initiated classic PScell change /CPC as the value of SN-initiated classic PScell change /CPC and the source SN node also performs root cause analysis in the procedure of MN-initiated classic PScell change /CPC, it makes these two cases (MN-initiated classic PScell change /CPC and SN-initiated classic PScell change /CPC) look the same. If the source SN node attempts to configure a specific value of T310/T312 trigger threshold for MN-initiated classic PScell change /CPC, whether the coordination between MN and the source SN is needed.

Moreover, RAN2#120 meeting has agreed that only MN can retrieve the SPR from the UE. That means for Option 2 MN should send the SPR to the source SN node for root cause analysis and then the source SN node send back the results of analysis to MN for the related parameters and configuration optimization in MN. And RAN3 needs to further discuss and specify the content of analysis results from the source SN node to MN over Xn.

Agreements:

3	Only MN can retrieve the SPR from the UE.

4	For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.:

For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN.

T304 trigger needs to be configured by the target SN node.

Observation 2: Although Option2 (the source SN node decides the T310/T312 triggers and performs root cause analysis) can simplify the generation of T310/T312 triggers condition for MN-initiated classic PScell change /CPC, other issues arising from Option2 need to be taken into account. Some issues are listed below:

- How the source SN node decides the proper T310/T312 trigger threshold for MN-initiated classic PScell change /CPC.

- MN should send the SPR to the source SN node for root cause analysis and then the source SN node send back the results of analysis to MN for optimization. The content of analysis results from the source SN node to MN over Xn need to be specified.

Based on the above discussion on Option 2, we think it is better that the node which initiates the procedure should decide the T310/T312 triggers and perform root cause analysis for the related parameters and configuration optimization for MN-initiated classic PScell change /CPC.

Proposal 1: For MN-initiated classic PScell change /CPC, the MN node which initiates the procedure should decide the T310/T312 triggers and perform root cause analysis for the related parameters and configuration optimization.

For this case, whether MN decides the T310/T312 triggers based on source SN inputs of T310/T312 value, it can be further discussed according to RAN2’s progress on the thresholds configuration for SPR. If percentage is used for T312/T310 thresholds configuration for SPR, the MN node decides the T310/T312 triggers threshold without source SN inputs and performs root cause analysis. If an exact value is used for T312/T310 thresholds configuration for SPR, the MN node should decide the T310/T312 triggers threshold based on source SN inputs of T310/T312 value and performs root cause analysis.

From our point of view, following the existing thresholds configuration of Successful HO, if percentage is used for T312/T310 thresholds configuration for SPR, Option 1 is preferred, since it has no Xn impact for coordination between the MN and the source SN.

Proposal 2: If percentage is used for T312/T310 thresholds configuration for SPR, Option 1(the MN node decides the T310/T312 triggers and performs root cause analysis) is used for MN-initiated classic PScell change /CPC.

# 3 Conclusion

In this paper, we discuss on the open issue of MN-initiated classic PScell change /CPC, and we have the following observations and proposals:

Observation 1: For MN-initiated classic PScell change /CPC, the PScell change procedure is initiated by MN and if the trigger is T312/310, the optimization of the related parameters and configuration in MN should be considered.

Observation 2: Although Option2 (the source SN node decides the T310/T312 triggers and performs root cause analysis) can simplify the generation of T310/T312 triggers condition for MN-initiated classic PScell change /CPC, other issues arising from Option2 need to be taken into account. Some issues are listed below:

- How the source SN node decides the proper T310/T312 trigger threshold for MN-initiated classic PScell change /CPC.

- MN should send the SPR to the source SN node for root cause analysis and then the source SN node send back the results of analysis to MN for optimization. The content of analysis results from the source SN node to MN over Xn need to be specified.

Proposal 1: For MN-initiated classic PScell change /CPC, the MN node which initiates the procedure should decide the T310/T312 triggers and perform root cause analysis for the related parameters and configuration optimization.

Proposal 2: If percentage is used for T312/T310 thresholds configuration for SPR, Option 1(the MN node decides the T310/T312 triggers and performs root cause analysis) is used for MN-initiated classic PScell change /CPC.

# Reference

- R3-230846, Summary of Offline Discussion on CB: # 12_SONMDT1_SHRandSPR, March 2023.