3GPP TSG-RAN WG3 #119bis-e	                                                                             R3-231799

17th – 26th April 2023, Online

Agenda Item:	12.2.2.2

Source:	CMCC

Title:	On Predicted UE Trajectory Information

Document for:	Discussion and Decision

1	Introduction

In last RAN3#119 meeting, the detailed information of predicted UE Trajectory was discussed, and it was agreed to not to include predicted RRC state and beam index information in the predicted UE Trajectory in this release [1]:

There is no need to include predicted RRC state in the cells in the predicted UE Trajectory in this release.

There is no need to include beam index information in the predicted UE Trajectory in this release.

The presence of the predicted time of stay of a UE in a cell is FFS.

FFS whether predicted UE Trajectory spans across multiple NG-RAN nodes or it is limited within a single target NG-RAN node.

In this contribution, we provide considerations on above remaining issues on predicted UE Trajectory information.

2	Discussion

2.1	Predicted time of stay in a cell

During the offline discussion in last RAN3#119 meeting [2], all companies support to have the information of predicted time of UE stay on a cell. The benefits lie in that with the information of both predicted cell ID and time stay in the cell, the target node could better prepare resources for the upcoming UE.

As for whether the predicted time of stay in a cell is provided mandatory or optional, company views are split.

Some companies think the information is optional since it dependent on network deployment. If no input data is available to predict time stayed in the cell, then the node cannot provide this information. Another issue is that it depends on the model capability since the model maybe cannot generate the staying time.

While other companies deem that without the time of stay, the value to inform target NG-RAN node about the predicted cell list only with cell ID is quite limited. Since the target node does not have accurate information of the UE´s mobility pattern, and anyway needs to process further on when/how to perform handover towards the cell in the list. Furthermore, if a NG-RAN node could make the prediction on the subsequent cells, it could definitely predict the time of stay.

Observation1: Without the information of predicted time of stay in a cell, the value of predicted UE trajectory information is limited.

So, it is proposed that:

Proposal 1: Predicted time of stay in a cell is a mandatory field of predicted UE Trajectory information.

2.2	UE Trajectory spans one-hop or multi-hop

During the offline discussion in last RAN3#119 meeting [2], whether the actual UE Trajectory over a future period of time shall be provided to the source node (either as part of feedback or as part of input information) was discussed. Majority companies deemed the feedback of actual UE Trajectory is useful to test and update a model. But some companies concerned about the feasibility since all nodes over trajectory path need to maintain context to remember it is for this UE and keep forwarding this info. Another issue is how long is the future period of time. Time of stay in the next cell and the next cell should report feedback after this time of stay. Supporting companies think that a minimum set of UE context, e.g, UE XnAP ID, could be kept for the UE. Furthermore, if only one-hop is supported, then it is feasible technically.

Proposal 2: UE Trajectory spans one-hop is supported in this release.

3	Summary

In this contribution, we provide considerations on remaining issues on the detailed information of predicted UE Trajectory. Following observation and proposals are made:

Observation1: Without the information of predicted time of stay in a cell, the value of predicted UE trajectory information is limited.

Proposal 1: Predicted time of stay in a cell is a mandatory field of predicted UE Trajectory information.

Proposal 2: UE Trajectory spans one-hop is supported in this release.

4	Reference

[1] 	RAN3_119_agenda_20230303_EOM2

[2]	R3-230856, Summary of Offline Discussion 18_AIRAN2_ME