3GPP TSG-RAN WG3 Meeting #119bis-e	R3-232018

E-meeting, 17 April – 26 April, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1162 | rev | 1 | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Extended Packet Delay Budget |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CATT,China Telecom, Nokia , Nokia Shanghai Bell, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to support service that can be used over satellite access type, SA2 already agreed to extend PDB value for 5QI 10 to 1100ms in Rel-17 [S2-2106653]. Therefore, the maximum PDB value in RAN side should be extended to 1100ms for Rel-17. Also, it is good to support this extension while keeping the 0.01 ms granularity to be future-proof. In order to align the PDB value in all RAN interfaces, it is proposed to extend the PDB value in F1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce a new range value 65536.. 109999 for Extended Packet Delay Budget IE with granularity of 0.01ms. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has limited impact on the protocol and the impact can be considered isolated. This CR has no functionality impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between NGAP/XnAP for Value range of Extended Packet Delay Budget IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.145, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0987 TS 38.423 CR 1048 TS 37.483 CR 0062 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1: update from R3-231706 with following changes: 1: correction on Extended Packet Delay Budget IE instead of Packet Delay Budget IE, 2: Adding more co-signers |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

<Unchanged Text Omitted>

9.3.1.145	Extended Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Packet Delay Budget | M |  | INTEGER (0..65535, …, 65536..109999) | Upper bound value for the delay that a packet may experience expressed in unit of 0.01ms. |

<Unchanged Text Omitted>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Unchanged Text Omitted>

-- E

ExtendedPacketDelayBudget ::= INTEGER (1..65535, ..., 65536..109999)

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>