3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231859

Online, 17th – 26th Apr, 2023

Agenda Item:	8.3

Source:	Lenovo (moderator)

Title:	Summary of Offline Discussion on RAN Exposure for XRM

Document for:	Approval

Introduction

This contribution provides the summary of offline discussion on:

CB: # 3_RANexposure_XRM

- Whether to support UP based QoS Notification Control?

- Whether to support data rate information on a per QoS Flow exposure from RAN?

- Capture agreements and provide LS to SA2

(moderator - Lenovo)

Summary of offline disc R3-231859

Phase I: Please provide your inputs before UTC time 13:00 Wednesday 19th April.

Phase II: Finalization of the response LS before UTC time 13:00 Monday 24th April.

For the Chairman’s Notes

Agree with draft response LS in R3-232129 (Ericsson) to capture the following proposals:

Revised Proposal 1: Responds SA2 as:

RAN3 discussed the UP based QoS Notification Control and reached no consensus on the benefits of this feature. 
During RAN3´s discussions the following possible technical issues were raised by some companies that may need to be further clarified by SA2:

-	UP based QNC may not be able to support reporting alternative QoS parameters.

-	Even in UP based QNC, QNC reporting delay is also possible, e.g. due to the hysteresis before determining the QNC result, and the delay depends on RAN’s implementation.

-	UP based QNC is on top of GTP-U/UDP which may cause out of sequence delivery.

Revised Proposal 3: Responds SA2 as:

Concerning the data rate information, and assuming that “data rate information” is the measured value of the current bit rate, RAN3 thinks that :

-	The per QoS flow data rate measured by RAN cannot reflect the real bandwidth that can be provided by the network, since it is effected by many factors such as the traffic rate, the air-interface condition and the scheduling strategies

-	It is not feasible to assume that data rates measured per DRB by the RAN can be converted in data rates per QoS Flow, to be signalled to the CN.

-	The per QoS flow data rate measured by RAN before PDCP is exactly the same as the data rate that the UPF could measure, hence there is no need for RAN to report it.

RAN3 therefore cannot reach consensus on the benefits of a solution based on data rate information on a per QoS Flow basis measured and exposed by NG-RAN via NGAP to SMF or via GTP-U to UPF

2nd Round discussion

Proposal 1: Responds SA2 as:

RAN3 discussed the UP based QNC and identified the following possible technical issues that may need to be further investigated by SA2:

- UP based QNC may not be able to support reporting alternative QoS parameters.

- Even in UP based QNC, QNC reporting delay is also possible, and the delay depends on RAN’s implementation.

- UP based QNC is on top of GTP-U/UDP which may cause out of sequence delivery.

Proposal 2: Responds SA2 as:

There are two different understandings on data rate information:

-	Understanding 1: data rate information is the available mean bit rate for the non-GBR flow that can be provided based on the available scheduling radio resource to the non-GBR QoS Flow.

-	Understanding 2: data rate information is the measured value of the current bitrate.

RAN3 kindly askes SA2 that which understanding is correct.

Proposal 3: Responds SA2 as:

For the understanding 1, RAN3 thinks that :

-	The per QoS flow data rate measured by RAN cannot reflect the real bandwidth can be provided, since it is effected by many factors such as the traffic rate, the air-interface condition and the scheduling strategies

-	It is not feasible to assume that data rates measured per DRB by the RAN can be converted in data rates per QoS Flow, to be signalled to the CN.

-	The per QoS flow data rate measured by RAN before PDCP is exactly the same as the data rate that the UPF could measure, hence there is no need for RAN to report it.

Q: Companies provide their views on the proposals, if any?

| Company | Comments |
| --- | --- |
| Huawei | We disagree with Proposal 1 since this is not RAN3’s common understanding but only a few companies. We suggest to remove P1, or rephrase the proposal below. Apart from that, we intend to provide our further views why these claimed issues actually are not real issues. - Apparently, the Alternative QoS mechanism is not required for the UP based QNC per SA2’s LS. - We think the delay D composes two parts: D1(processing delay due to the measurement hysteresis) +D2(transmission delay from RAN to AF). No matter using CP or UP based QNC, the D1 will be same. But D2 of the two ways are different. UP based QNC will provides shorter routing path, especially for MEC related traffics (the APP server is deployed near/co-located with the UPF), the D2-UP<<D2-CP. So, the UP based solution can deliver the results to the APP server as soon as possible once obtained by RAN, but CP based solution will cause more transmission delay, we think the reason of QNC is no real time information to against UP based solution does not make sense to us. We also provide a figure to show the comparison of CP and UP based QNC. - As we already stated in 1st round, the out of sequence is not a problem for QNC, because QNC result will not be sent very frequently. Such issue can be perfectly avoided by proper network implementation. No issue is foreseen. Revised Proposal 1: Responds SA2 as: RAN3 discussed the UP based QNC and cannot reach consensus, because some companies identified the following possible technical issues that may need to be further investigated by SA2: - Is there any need for the UP based QNC to report the alternative QoS parameters given that the CP based QNC could support it? - Both CP and UP based QNC will require some hysteresis, which is implementation dependent, then report to CN. Does the short report delay provided by UP based QNC prevail over the CP based QNC? - Is there any issue for the UP based QNC, considering that the NG-U data is on top of GTP-U/UDP which may cause out of sequence delivery? For Proposal 3, suggest a few rewordings on the 1st bullet: - The per QoS flow data rate measured by RAN cannot reflect the real bandwidth which can be provided by the network, since it is effected by many factors such as the traffic rate, the air-interface condition and the scheduling strategies. |
| CATT | Proposal 1: the only “benefit” can be identified is that UP based solution may faster than CP based solution. But the longer path/more entities does not mean more delay, it is up to implementation. Even if UP based solution faster than CP based solution, we do not see the necessary to support UP based solution which will introduce significant specification changes only for more “fast”. Proposal 2: ok Proposal 3: I think it should for understanding 2 not understanding 1. In fact, we better to discuss the benefit/drawback after we have a conclusion about proposal 2 based on SA2 reply. No hurry to make some analyzations in this meeting. Prefer to remove it. |
| Qualcomm | We are fine with moderator summary for all proposals. We are also fine with Huawei proposed changed to P3. |
| Ericsson | Regarding P1, we agree with the moderator´s proposal and with CATT´s comments. The overall delay of conveying the QoS notification to the AF is totally implementation dependent. This is even more obvious if we consider that the 5GC architecture is service based, namely all functions involved in the QCN signalling are very likely implemented on the same cloud platform, hence delays differences between a UP and CP path are up to how the service based architecture is implemented. We suggest to add to the answer from the moderator the following: “RAN3 therefore cannot reach consensus on the benefits of a solution based on QCN signalling over the UP” We are ok with Proposal 2 We are fine with Proposal 3 and with the changes from Huawei. We suggest to add the following ad t the end of Proposal 3: “RAN3 therefore cannot reach consensus on the benefits of a solution based on data rate information on a per QoS Flow basis measured and exposed by NG-RAN via NGAP to SMF or via GTP-U to UPF” |
| Verizon | The following was excerpt from the SA2 LS: “SA2 is discussing the following new functionalities exposing NG-RAN information in the specifications under SA2 control: - For QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4 of TS 23.501, upon SMF request, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF - Based on SMF request over NGAP, data rate information on a per QoS Flow basis may be measured and exposed via NGAP to SMF or via GTP-U to UPF. SA2 asks RAN3 to provide feedbacks on the above functionalities.” -------------------------------------------------------------------------- VZ views: Proposal-1 on QNC for GFBR via GTP-U to UPF: - RAN3 discussed this function and believed that this is feasible technically. But not all the companies are seeing the benefit of the new function. RAN3 would like to ask SA2 to decide if this new function is beneficially for XR type of services per use case needs. In addition, there are some technical issues raised by some companies, we’d like SA2 to further clarify. (include the questions raised by HW as follows.) Proposal-2 on RAN data rate exposure: - RAN3 discussed this and had two understanding on the definition of the data rate in the context, and would like to ask SA2 to further clarify its definition. Proposal-3 is not necessary for now as it is based upon understanding-1 of data rate. No need to rush to conclusion on this. |
| Deutsche Telekom | For P1, we support the revised text including 3 questions to be raised as proposed by Huawei. P2: We are fine with moderator’s text. P3: We are fine with moderator’s text and the proposed rewording by Huawei. The information related to understanding 1 could be interesting for SA2 discussion even if finally SA2’s understanding in data rate is 2. |
| Samsung | P1：we agree with the view of CATT that the longer path/more entities does not mean more delay. We are also fine with Huawei’s rewording. P2: OK P3: OK |
| CMCC | For P1, we share the same view with HW, we think it is benefit to support UP based QNC. From the illustration provided by HW, it is clearly shown that the transmission delay from RAN to AF from UP based(RAN->UPF-> local NEF->AF) can be shortened comparing to CP based solution(RAN->AMF->SMF->PCF->central NEF->AF). Also, it is real corner case that the out of sequence delivery over GTP-U. This issue can be solved by NG-RAN implementation. Therefore, we are slight prefer the response reworded by HW. For P2, after check with our SA2 colleagues and TR, the common understanding of data rate information from SA2 is the measured value of the current bitrate. In SA2 TR, the specification also specifies that: Data rate, delay difference and round trip delay of QoS flow may be exposed to AF: - Data rate may be measured and exposed by PSA UPF. Exposure path defined in clause 6.4 of TS 23.548 [61] is reused to expose the above information. Exposure path of UPF reporting via SMF/PCF/NEF is also supported. It is obvious that the understanding from SA2 is the second interpretation. We think that P2 should be removed. Given that the SA2 XR will be closed after the meeting in May, we hope that we can focus on some major issues in our reply LS and try to keep this WI closed on time in SA2. |
| ZTE | P1: We are fine with Huawei’s rewording. P2: OK P3: We agree with CATT’s view. There’s no need to rush into the detailed analysis. We can wait for SA2’s response regarding proposal 2. |
| Lenovo | P1: OK. We are also fine with Huawei’s rewording. P2: OK. P3: OK with Huawei’s rewording. It should be ‘understanding 2’. A typo is corrected. |

Summary:

P1:

Moderator thinks Huawei’s proposal also make sense that are supported by operators (CMCC, DT and Verizon) and also some other companies. From this sense, Moderator would like to use Huawei’s version for Response:

Revised Proposal 1: Responds SA2 as:

RAN3 discussed the UP based QNC and cannot reach consensus, because some companies identified the following possible technical issues that may need to be further investigated by SA2:

- Is there any need for the UP based QNC to report the alternative QoS parameters given that the CP based QNC could support it?

- Both CP and UP based QNC will require some hysteresis, which is implementation dependent, then report to CN. Does the short report delay provided by UP based QNC prevail over the CP based QNC?

- Is there any issue for the UP based QNC, considering that the NG-U data is on top of GTP-U/UDP which may cause out of sequence delivery?

P2:

One company does not want to ask clarifications to SA2 since the WI is to be closed in SA2, while other companies are fine with P2.

P3:

Two companies think P3 is not needed before we have clear views on P2. But the other companies are fine with P3 with Huawei’s rewording.

Revised Proposal 3: Responds SA2 as:

For the understanding 2, RAN3 thinks that :

-	The per QoS flow data rate measured by RAN cannot reflect the real bandwidth which can be provided by the network, since it is effected by many factors such as the traffic rate, the air-interface condition and the scheduling strategies

-	It is not feasible to assume that data rates measured per DRB by the RAN can be converted in data rates per QoS Flow, to be signalled to the CN.

-	The per QoS flow data rate measured by RAN before PDCP is exactly the same as the data rate that the UPF could measure, hence there is no need for RAN to report it.

Discussion

QoS Notification Control

The new functionality is described as in the incoming LS

-	For QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4 of TS 23.501, upon SMF request, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF

Proponents think that:

-	The UP based notification will be used for information exposure via UPF-AF, which is much faster than way relies on the forwarding among multiple functions (SMF-PCF-NEF-AF) if using CP based notification. [1]

-	The potential specification impacts of the UP based QoS Notification Control functionality is limited. For example, the already supported CP notification request over NGAP for a GBR QoS flow can be easily extended to carry the UP notification request. [1]

-	Currently, the NG-RAN node has been able to determine ‘GFBR can be no longer guaranteed’ and ‘GFBR can be guaranteed again’ with/without alternative QoS profiles. [15]

-	Currently, the NG-RAN node has been able to provide some QoS monitoring information such as DL delay result to the UPF directly in the "PDU Session Container" GTP-U Extension Header. [15]

-	RAN3 does not see any technical issue that the NG-RAN indicates that "GFBR can no longer (or can again) be guaranteed" via GTP-U (e.g., in the "PDU Session Container" GTP-U Extension Header) to UPF. [15]

-	It is feasible for RAN to support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF upon SMF request. [9] [11].

Opponents thank that:

-	Supporting signalling of QoS Notification Control for GBR QoS Flow via GTP-U to UPF has a potential impact on all the RAN nodes and interfaces. Signalling QoS Notification Control over the UP may deny the possibility of reporting alternative QoS parameters. [5]

-	QoS Notification Control for GBR QoS Flow is deliberately delayed with respect to the event that triggered it, so to create an hysteresis and avoid “ping pong” effects, hence it should not be assumed that this information is sent in real time and that it can be used for real time application rate control. [5]

-	Lack of in order delivery over GTP-U may cause out of order information exposure at the UPF/AF and wrong rate adaptation decisions at the AF. [5]

-	QoS Notification Control for GBR QoS Flow has been support in CP based. Not sure the additional benefit to support UP based solution. [7]

Moderator would like to concentrate on the technical issues in the first phase of the email discussion. As pointed in the [5], the potential technical issues are:

1a. Signalling QoS Notification Control over the UP may deny the possibility of reporting alternative QoS parameters.

1b. QoS Notification Control for GBR QoS Flow is deliberately delayed with respect to the event that triggered it, so that it is not a real time information.

1c. Lack of in order delivery over GTP-U may cause out of order information exposure at the UPF/AF.

Q1: Companies provide their views of the technical issues 1a, 1b and 1c, e.g., whether the issues are valid, and if valid, whether we need to capture them in the response LS?

| Company | Comments |
| --- | --- |
| Huawei | 1a: Invalid. UP based QNC is used for network exposure, for which SA2 think the “fulfill/not fullfill” indication would be efficient for the AF/APP server without the alternative QoS. And it should be under SA2 scope to determine whether to link the UP based QNC with alternative QoS in their work. So, in our view, issue 1a is invalid. 1b: Invalid. The measurement for QNC may apply some hysteresis, but the core idea of the UP QNC is to enable the server to know the situation as soon as possible, after RAN derive the result. A quick notification towards the server will certainly be beneficial if the app server can perform some adjustment regarding to the results. 1c: Invalid. The UPF can perform reordering based on the SN per QoS flow in the GTP-U header, if included. Even if the SN is not included in the NG-U header, there is no issue. As quoted by [5], RAN may apply some hysteresis when measure the result for QNC, so the result will not change very quickly and frequently. Then, it is real corner case that the sequence of received QNC results is different from that of sending from RAN. Furthermore, such corner case is easy to be avoided by simple network implementation, e.g. not to trigger two different QNC results in a short period. |
| Ericsson | 1a: Valid. The purpose of exposing QCN to the AF is to enable rate control at the AF. It is of little benefit to expose QCN if the AF is not aware of the alternative bit rates that can be achieved, namely alternative QoS. Only exposing the QCN indication would result in the AF attempting to adjust the data rate and likely still failing to reach a data rate the RAN can come with. One reason that proves that this issue is valid is that the CR attached by SA2 to the LS in R3-230791 mentions: “The AF may provide the Alternative QoS parameter set requirements and Averaging Window to the NEF/PCF for the GBR QoS Flow as specified in clause 4.15.6.6 of TS 23.502 [3].” Hence the AF needs to know which alternative QoS is fulfilled to determine how to adapt QoS Flow QoS. 1b. Valid. QCN is not providing real time information about whether the QoS Flow rate is above or below the GBR rate, which is ultimately what the AF would need to throttle data rates. For this reason there is technically no difference in having QCN information signaled via the SMF because they cannot considered for real time rate adaptation. An example of how things could go wrong if CAN is used for real time rate adaptation is that an averaged measure (subject to hysteresis) of data rates triggers QCN = GBR not fulfilled. However, data rates are increasing at RAN. AF immediately reduces data rates. Hence, while the RAN can provide higher rates, AF reduces rates, leading to failure to exploit the available resources at RAN, which could serve a higher data rate. 1c. Valid. Due to packet loss a solution would need to include the QCN on multiple GTP-U PDUs. Hence if “QCN = GBR not fulfilled” occurs, the information will have to be signalled multiple times. An out of order delivery implies that the AF may receive a sequence of GBR Fulfilled/GBR Not Fulfilled/GBR Fulfilled, instead of a sequence of GBR Fulfilled/GBR Not Fulfilled/GBR Fulfilled. The AF then may reduce and increase data rates in a way that is counter productive and against the resource status of the NG-RAN We would also like to highlight that this solution leads to a total duplication of functionality at RAN level. Namely the gNB-CU-UP, the E1, F1-U and NG-U will need to be impacted for a functionality that already exists and that leads to no benefit. |
| Verizon | 1a. Invalid. reporting alternative QoS parameters is not mandatory. 1b. Invalid. The QNC for GFBR is deliberately delayed. It can be a real time information. Application server could apply a hysteresis to prevent ping-pang effect, and would appreciate fast real time feedback to do rate adaptation. 1c. Invalid. Agree HW’s suggestion, e.g., not to trigger two different QNC results in a short period of time. |
| Qualcomm | 1a: Valid : using UP based method, RAN may not be able to report what are alternative QoS that can be met. But AS may have to select some alternative QoS, it is not clear how AS can determine without any assistance from RAN ? 1b: Valid : QNC reporting delay is possible and it depends on how RAN does averaging in window time and delay depends on implementation as well. 1c: Invalid: this can be avoided based on proper implementation. Apart from above things, it seems redundant and will add impacts to all RAN interfaces in addition to NG-U, NG-AP and still benefits are not obvious. |
| CATT | 1a. valid/invalid. We agree that UP based solution cannot provide alternative QoS that can be met but SA2 only consider using UP based solution to support "GFBR can no longer (or can again) be guaranteed". It seems more than SA2 intended. 1b. valid. QNC reporting is not real-time. We cannot assume that UP based solution will faster than CP based solution. 1c: Invalid. It is not the main reason to not support UP based solution. UPF can consider the received indication in GTP-U header together based on implementation. |
| ZTE | 1a. Invalid. Agree with CATT. This reason seems to be out of the scope of SA2’s question. 1b. Valid. As specified in TS 23.501, some hysteresis will be applied by RAN implementation. 1c. Valid. This issue exists but can be easily solved. |
| CMCC | 1a. Invalid. From our view, there are no correlation between reporting alternative QoS parameters and signalling QoS Notification Control over the UP. Why we must to put them together? Alternative QoS parameters are used by NG-RAN to check whether the flow can fulfill before it sends the notification. 2b. Invalid. It depends. There is a high possibility sending a fast feedback to APP server. Take a step back, although it is not a real time information, it may reflect the status of a QoS flow within a time duration. It also could be used by AF for flow rate adaptation. 3c. Invalid. The UPF can identify the information based on the QFI in the GTP-U header. Also, we share the same understanding with HW and verizon, it is real corner case that the sequence of received QNC results is different from that of sending from RAN. |
| Deutsche Telekom | 1a: Partially valid. Applying UP QNC does not allow AQP handling, but it is up to SA2’s expertise if they think a notification without data rate information is sufficient. 1b: Partially valid. The delay for QNC is dependent on measurement window applied in RAN. 1c: Valid, but the issue can be solved according to the appropriate implementation. |
| Nokia | All are valid. Please remember SN is never used in NG-U. GTP-U uses UDP/IP which is an Unreliable data protocol and does not provide in-sequence delivery. In addition, the CP signaling has higher priority than UP traffic in the transport network. So it is not a valid argument that UP based solution will be faster than CP based solution. The main question is why introducing the additional UP based solution without a clear benefit! |
| Samsung | 1a: Invalid. Share the similar view with CMCC. SA2 should consider whether the UP based QNC is reported with alternative QoS. 1b: valid/invalid. Agree with DT. 1c: Invalid, this can be avoided by implementation. |
| Lenovo | 1a: Invalid. Firstly, we think reporting alternative QoS parameters is optional in NG interface. Secondly, it is SA2’s scope to consider whether alternative QoS is workable for UP based QNC. From our view, it is workable anyway with some spec update. 1b: valid/invalid. It depends on network implementation. 1c: valid/invalid. Out of ordering could happen, but it can be solved by implementation and should be corner case taking the reliability of transport network into account. |

Moderator tends to not discuss standard impact at this stage.

Summary:

1a: Companies tend to agree that UP based QNC may not be able to support of reporting alternative QoS parameters (at least some standard effort is needed to support report alternative QoS parameters by UP). And it is also fair enough that whether alternative QoS parameters are mandatorily needed by AF is in the scope of SA2.

1b: Even in UP based QNC, QNC reporting delay is also possible but it depends on RAN implementation. But whether it has sufficient impact is in the scope of SA2.

1c: UP based QNC is on top of GTP-U/UDP which may cause out of sequence delivery. But whether it has sufficient impact is in the scope of SA2.

Moderator suggest RAN3 only feedback the possible issues to SA2, and the impact on SA2 should be decided by SA2.

Proposal 1: Responds SA2 as:

RAN3 discussed the UP based QNC and identified the following possible technical issues that may need to be further investigated by SA2:

- UP based QNC may not be able to support reporting alternative QoS parameters.

- Even in UP based QNC, QNC reporting delay is also possible, and the delay depends on RAN’s implementation.

- UP based QNC is on top of GTP-U/UDP which may cause out of sequence delivery.

Data rate measurement

Firstly, companies have different understandings on the on the data rate information:

-	[7] [15] thinks that the data rate information is the available mean bit rate for the non-GBR flow as described in solution 6 of TR 23.700-060. The available mean bitrate is the bitrate that can be provided by the non-GBR QoS Flow based on the available scheduling radio resource to the non-GBR QoS Flow. The available mean bitrate can be treated as the near-future bitrate and is not the measured value of the current bitrate in the radio interface for the non-GBR QoS Flow.

-	[1] [5] thanks the data rates per QoS Flow measured at the UPF are sufficient, in which it is assumed that the data rate information is the measured value of the current bitrate.

Moderator would like to clarify the understanding of data rate information first

-	Understanding 1: data rate information is the available mean bit rate for the non-GBR flow that can be provided based on the available scheduling radio resource to the non-GBR QoS Flow.

-	Understanding 2: data rate information is the measured value of the current bitrate.

Q2: Companies provide their views on the understanding of the data rate information, e.g. understanding 1 v.s. understanding 2?

| Company | Comments |
| --- | --- |
| Huawei | Understanding 1 |
| Ericsson | Understanding 2 What SA2 has included in the CR attached to their LS (S2-23203880) is the following: - Data rate information may be measured and exposed to the AF based on SMF request as one of the following: - measured and reported by PSA UPF via Nupf_EventExposure service or via SMF/PCF/NEF as described in 5.8.2.X. - measured by RAN and reported via SMF/PCF/NEF Hence, SA2 never discussed about “available mean bit rate for the non-GBR flow that can be provided based on the available scheduling radio resource to the non-GBR QoS Flow”, and above all, SA2 has only discuss the possibility of exposure of measured data rates, which coincides with Understanding 2. |
| Verizon | Understanding 1. Leaning toward that the data rate to be exposed by RAN is the available mean bit rate for the non-GBR flow as described in solution 6 of TR 23.700-060. This data rate that RAN exposes to CN will be a near future, not measured, but a leading indicator, which has added value to enhance XR performance, vs the data rate measured by UPF, which is a lagging indicator. However, I think we need to ask SA2 to clarify the data rate definition here. |
| Qualcomm | Understanding 2 But we are fine to ask clarification from SA2 if needed. |
| CATT | Understanding 1 It has been captured in 23.700-60 that data rate information is the available mean bit rate for the non-GBR flow. But we are ok to provide both understandings to SA2 combined with the conclusion of Q3 |
| ZTE | Understanding 1 as captured in 23.700-60. It’s fine to confirm it with SA2. |
| CMCC | Understanding 1, we think this interpretation is in SA2 scope. No need to put too much time on discussing the interpretation of this issue. We can ask the clarification from SA2. |
| Deutsche Telekom | Understanding 2. But as the meaning seems to be ambiguous, we propose to ask SA2 for clarification. |
| Nokia | Understanding 2. But ok to further check SA2. |
| Samsung | Understanding 1, suggest to confirm it with SA2. |
| Lenovo | Understanding 1 as discussed in our contribution. |

Summary:

7 companies – Understanding 1, 4 companies – Understanding 2.

Proposal 2: Responds SA2 as:

There are two different understandings on data rate information:

-	Understanding 1: data rate information is the available mean bit rate for the non-GBR flow that can be provided based on the available scheduling radio resource to the non-GBR QoS Flow.

-	Understanding 2: data rate information is the measured value of the current bitrate.

RAN3 kindly askes SA2 that which understanding is correct.

Opponents thank that:

-	The per QoS flow data rate measured by RAN cannot reflect the real bandwidth can be provided, since it is effected by many factors such as the traffic rate, the air-interface condition and the scheduling strategies [1]

-	It is not feasible to assume that data rates measured per DRB by the RAN can be converted in data rates per QoS Flow, to be signalled to the CN. [5]

-	Such measurements are already available at the UPF and there is no motivation to introduce RAN based data rate measurement and exposure. [1] [5]

Proponents think that:

-	The NG-RAN can inform the CN about the currently fulfilled QoS parameters and the AF may do some changes accordingly. But there is no such mechanism for non-GBR flow. [7]

-	There is no explicit bit rate for the non-GBR Flow but the available mean bitrate can be treated as the near-future bitrate for non-GBR QoS flow. [7]

-	Dynamic adjust the data rate by AF based on available mean bit rate reported by NG-RAN would be useful for XR service. [7]

-	it is feasible to report the available bit rate for a QoS flow [7] [9] [11][15]and prefers a similar mechanism as QNC for reporting the available bit rate for a QoS flow [7] [15].

Moderator would like to concentrate on the technical issues in the first phase of the email discussion. The potential technical issues are:

2a: The per QoS flow data rate measured by RAN cannot reflect the real bandwidth can be provided, since it is effected by many factors such as the traffic rate, the air-interface condition and the scheduling strategies [1].

2b. It is not feasible to assume that data rates measured per DRB by the RAN can be converted in data rates per QoS Flow, to be signalled to the CN. [5]

2c. The per QoS flow data rate measured by RAN before PDCP is exactly the same as the data rate that the UPF could measure, hence there is no need for RAN to report it

Q3: Companies provide their views of the technical issues 2a and 2b, e.g., whether the issues are valid, and if valid, whether we need to capture them in the response LS?

| Company | Comments |
| --- | --- |
| Huawei | 2a. valid 2b. valid Ok to capture both of them in the response LS. |
| Ericsson | 2a. valid 2b. valid 2c. Valid. |
| Verizon | 2ª. Valid 2b. Valid Ask SA2 to clarify the definition of the data rate. |
| Qualcomm | 2a: Valid 2b: Valid 2c: Valid |
| CATT | 2a: Invalid for underdtanding 1. If we go for understanding 1, there is no real data rate is needed. It is a “capability” of NG-RAN about what data rate it can be supported in near-further considering the many factors such as the traffic rate, the air-interface condition and the scheduling strategies. It is similar as GFBR in alternative QoS parameters Set List and Alternative QoS Parameters Set Index. valid for underdtanding 2. 2b: Invalid for underdtanding 1. NG-RAN has the capability to feedback what the GFBR it can be support. GFBR is a per QoS parameter. The NG-RAN can identify the mean bit rate for non-GBR flow per QoS rather than per DRB. valid for underdtanding 2. 2c: Invalid for underdtanding 1. This is not a real time measurement about bit rate, it about NG-RAN “capability”. valid for underdtanding 2. |
| ZTE | 2a. valid 2b. valid 2c. Valid. |
| CMCC | 2a: Valid 2b: Valid |
| Deutsche Telekom | 2a: Valid 2b: Valid 2c: Valid. |
| Nokia | 2a: Valid 2b: Valid 2c: Valid |
| Samsung | 2a: Valid 2b: Valid 2c: Valid |
| Lenovo | All are valid for understanding 2, but invalid for undstanding 1. |

At this moment, moderator tends to not discuss UP or CP based solution and standard impact.

Summary:

For understanding 1, companies tend to agree the technical issues.

Proposal 3: Responds SA2 as:

For the understanding 1, RAN3 thinks that :

-	The per QoS flow data rate measured by RAN cannot reflect the real bandwidth can be provided, since it is effected by many factors such as the traffic rate, the air-interface condition and the scheduling strategies

-	It is not feasible to assume that data rates measured per DRB by the RAN can be converted in data rates per QoS Flow, to be signalled to the CN.

-	The per QoS flow data rate measured by RAN before PDCP is exactly the same as the data rate that the UPF could measure, hence there is no need for RAN to report it.

References

R3-231498 Discussion on the RAN information exposure for XRM (Huawei)

R3-231499 CR on support UP based QoS notification control for XR (Huawei)

R3-231500 CR on support UP based QoS notification control for XR (Huawei)

R3-231501 [draft] Reply LS on RAN information exposure for XRM (Huawei)

R3-231550 Discussion on SA2 LS on RAN information exposure for XRM (Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell)

R3-231551 Reply LS on RAN information exposure for XRM (Ericsson)

R3-231560 Discussion on RAN information exposure for XRM (CATT)

R3-231561 Reply LS on RAN information exposure for XRM (CATT)

R3-231802 Discussion on exposing NG-RAN information (CMCC)

R3-231803 Draft reply LS on RAN information exposure for XRM (CMCC)

R3-231851 Discussion on RAN information exposure for XRM (ZTE)

R3-231852 [DRAFT] Reply LS on RAN information exposure for XRM (ZTE)

R3-231853 TP to TS 38.413 for RAN information exposure for XRM (ZTE)

R3-231854 TP to TS 38.415 for RAN information exposure for XRM (ZTE)

R3-231452 Discussion on RAN information exposure for XRM (Lenovo)