3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231878

Online, 17th – 26th April 2023

Agenda Item:	12.2.1

Source:	CMCC (moderator)

Title:	                   Summary of CB: # AIRAN1_Stage2

Document for:	Discussion and Decision

1	Introduction

CB: # AIRAN1_Stage2

- Turn WA to agreement?

- Capture agreements to TS38.300?

(moderator - CMCC)

Summary of offline disc

This email discussion will comprise two phases:

- Phase 1 Deadline: Thursday April 20th, 10pm UTC

- Phase 2 Deadline: Monday April 24th, 8am UTC

In the second phase, we will try to capture agreements to TS38.300.

2	For the Chairman’s Notes

For agreement:

Proposal 1: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated.

Proposal 2: The requested prediction time and validity time are configured in the AI/ML INFORMATION REQUEST (FFS on the name), for one-time reporting.

Requested prediction time: time duration of the requesting prediction information in the AI/ML INFORMATION REQUEST (FFS on the name), e.g. start time plus end time.

Validity time: time period within which the requested prediction information in the AI/ML INFORMATION UPDATE (FFS on the name) is considered valid, hence the AI/ML prediction can be used by the requesting NG-RAN node.

2nd round:

- For periodic reporting, discuss if requested prediction time and validity time are explicitly or implicitly signalled (e.g. by means of the reporting period).

- Discuss on the TP for TS38.300

To be continued:

Whether accuracy/confidence of an AI/ML prediction is sent over Xn along with the prediction.

3	Discussion

As per the guidance from the chair, the first round of the CB will be structed as follows:

- Discuss on whether to turn following WA in last RAN3#119 meeting to be agreement:

WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.

- Discuss on further clarification on the WA

- Other open issues

3.1 Turn WA to agreement

In last RAN3 meeting, following WA is achieved:

WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.

Some companies think the current WA is not clear enough to reflect the actual intention. In [1], it is proposed to reformulate the WA to state that:

Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” which means that the intended use of the requested data shall not be indicated in the procedure.

In [2], similar clarification on the WA is provided as follows:

Proposal 1: The procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” in the sense that the purpose for which data is requested (e.g., input, output, feedback) shall not be indicated.

Q1: Companies are invited to provide views on whether to agree on above clarification on the WA (i.e., the intended use of the requested data is not indicated in the procedure)?

| Company | Yes/No | Reasons/Comments/Suggestions |
| --- | --- | --- |
| Ericsson | Yes | A combination of the two could be as follows: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” which means that the intended use of the requested data (e.g., input, output, feedback) at the requesting node shall not be indicated |
| Lenovo | Yes, with comment | We suppose it applies to not only “for which data is requested”, but also to “for which data is sent”, given the procedures includes request/response/update. We can simply say, Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” which means that the intended use of the requested data shall not be indicated in the procedure. |
| Intel | Yes | Lenovo’s update looks good to us. |
| Nokia | Yes | We are ok with Lenovo’s update with a further small change: “Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated.” |
| CATT | Yes |  |
| China Telecom | Yes | We also agree with Nokia’s view. |
| Samsung | Yes | Fine for Nokia’s wording. It is better to indicate what is the data type (input, output, feedback) here. |
| Huawei | Yes | We prefer Lenovo’s wording, i.e., not to mention the “(e.g., input, output, feedback)”. But if the majority is fine with Nokia’s version we are also OK |
| Qualcomm | No strong view | The intent of the WA is understood. If majority of companies prefer to clarify the WA, we are fine with it. |
| Deutsche Telekom | Yes | Fine with Nokia’s version as the terms in the parenthesis explain what data are meant in the WA. |
| ZTE | See comments | According to the WA, the intended use of the data should not be indicated in the procedure, which is acceptable for us. However, we have one concern that needs clarification. If in the future, there is a type of information that can either be collected immediately or collected after a certain action, how can the requested NG-RAN node know the requirement from the requesting NG-RAN node? In order to resolve this issue, an indication may be needed. Therefore, if the current WA does not limit this particular case, then it is acceptable for us. |
| InterDigital | Yes | Ok with Nokia or Lenovo |
| CMCC | Yes | Ok with Nokia. |
| LGE | Yes | We prefer Lenovo’s wording considering ZTE’s concern. |
| Orange | Yes | OK with Nokia’s version |

Summary of Q1:

It seems that almost all companies are fine with Nokia’s version. Therefore, the WA is clarified as:

Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated.

Q2: Companies are invited to provide views on whether to agree to turn the WA to agreement? If yes, whether to capture the clarification in Q1 into agreement?

| Company | Yes/No | Reasons/Comments/Suggestions |
| --- | --- | --- |
| Ericsson | Yes |  |
| Lenovo | Yes |  |
| Intel | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |
| China Telecom | Yes |  |
| Samsung | Yes |  |
| Huawei | Yes | It should also be captured in the 38.300 BLCR |
| Qualcomm | No strong view |  |
| Deutsche Telekom | Yes |  |
| ZTE | Same comments in Q1 |  |
| InterDigital | Yes |  |
| CMCC | Yes |  |
| LGE | Yes |  |
| Orange | Yes |  |

Summary of Q2:

Almost all companies agree to turn the rewording WA into agreement. It is proposed that:

Proposal 1: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated.

3.2 Open issues

In [5], other open issues such as the accuracy/confidence of an AI/ML prediction, requested time and validity time of AI/ML information, are discussed and following proposals are provided:

Proposal 1: Accuracy/confidence of an AI/ML prediction is not sent over Xn along with the prediction itself.

Proposal 2: The requested time and validity time should be configured in the AI/ML INFORMATION REQUEST (FFS on the name) in the case of one-time reporting.

Q3: Companies are invited to provide views on whether to agree on above proposals?

| Company | Yes/No |  | Reasons/Comments/Suggestions |  |
| --- | --- | --- | --- | --- |
| Ericsson | See comments |  | We agree on Proposal 1. Accuracy can only be achieved at model testing as an average for the model testing inputs. A model cannot derive the actual accuracy of an inferred output. Such accuracy would be in itself a prediction. Hence, there is no value in sending the “accuracy”. There would actually be drawbacks in sending “accuracy” with the prediction. For example, the model accuracy could be good, but if the model receives inputs different from those used at model testing, the accuracy could be totally different from the one calculated at testing. We agree on Proposal 2, but we believe that only the prediction time needs to be signalled. A prediction is carried out for a specific time into the future, hence the essential information to know is what such time is. For simplicity, we propose that such time is configured, e.g. it is a fixed time. |  |
| Lenovo | 1) No 2) Partially |  | 1) For accuracy/confidence information about the prediction, we give our analysis in paper R3-231433. In short, we believe it is useful for the prediction receiver to understand the likelihood a prediction will be true. In a reasonable RAN implementation, It will impact how a RAN node weights the prediction result when use it for a RAN decision. 2) The definition of validity time and requested time given in [5] is as following: Validity time: time period within which the requested prediction information in the AI/ML INFORMATION UPDATE (FFS on the name) is considered valid, hence the AI/ML prediction can be used by the requesting NG-RAN node. - Can I understand when it says a prediction “is considered valid” means the accuracy/confidence of the prediction is good enough? Isn’t it implying some accuracy/confidence information? [Huawei] it means that the provided information can be used by gNB for its own evaluations; our assumption is that if a prediction is provided then there is inherently the confidence that such information can be used, otherwise we prefer not to provide the prediction at all if the node sending the prediction already knows that such info has low accuracy Requested time: time duration of the requesting prediction information in the AI/ML INFORMATION REQUEST (FFS on the name), e.g. start time plus end time. - Maybe first RAN3 needs to agree what is a validity time, in our paper R3-231433, we consider Validity Time as how it is defined in [5] for Requested Time. And we believe, it is applicable to not only one-time reporting but also periodic reporting. Note that the validity time of a prediction is different than the period configured for the periodic reporting. |  |
| Intel | 1) No, see comment 2) see comment |  | For 1), we also believe accuracy level/confidence level is beneficial. For the predicted data received from a neighbouring NG-RAN node, it is important for the receiving NG-RAN node to know whether the predicted data can be trusted or not, i.e. the confidence level. If the confidence level of the predicted data is low, the NG-RAN node may be better not to use such information for model inference or model training. Alternatively, the NG-RAN node may also take the confidence level as the weight of using the predicted data for the corresponding AI/ML algorithm. For 2), to avoid ambiguity, since here we are discussing validity of the data, “validity duration” and “validity time” could be used, where the “validity duration” means the time period within which the data is valid as soon as the data is arrived, the validity time refers to the accurate time of when the data will be valid by knowing the start time and end time. Except including this information in Request message, we think the response message could also include the validity duration and validity time, in case the requested node may provide a subset of the requested validity duration based on its model implementation and accuracy. |  |
| Nokia | Yes but |  | In our view, a node requesting a prediction needs to know some information about the prediction that can be provided by a neighbour. If the prediction is needed as an input to another ML Model and it is poor, it can negatively impact the model’s performance. Namely if a prediction is requested at a certain time, a requesting node needs to know a level of confidence of this information. Indeed, it is very hard to monitor model accuracy but some information on the quality of the prediction can be provided through a confidence level. This can be provided as an error margin e.g., x% of the average (predicted value). But we do not think that the responding node needs to report accuracy or confidence information together with the prediction as long as it meets a minimum confidence (error margin) that the requesting node needs. The reporting node could internally use different techniques to maintain (or even exceed) the requested confidence but these would be up to the node’s implementation and the requesting node doesn’t need to know the actual values as long as they meet a minimum quality. On proposal 2, we think that the requested time and validity time need to be provided not only for one-time reporting, but also for periodic reporting. For periodic reporting, the subsequent requested times could be determined through the period T (i.e., t+ T, t+ 2T,…). If no validity time is indicated, it would mean that a prediction is only valid for the prediction time. Validity time gives a different angle to a prediction in that the requested information is meaningful not only for a specific time but over an interval of validity. We see value in that in the sense that a node receiving the prediction may utilize it over longer periods of time. We illustrate our understanding through an example below: |  |
| CATT | 1) No 2) Partly |  | For P2, We understand the intention of Nokia, however, from our point of view, it seems there is no need for a separate “requested time”. The t in Nokia’s figure is also when the request message is sent. Then, for simplicity, we think the validity time and prediction period could be the same at least in this release. Otherwise, there maybe some time period during which there is no prediction data available as showed in Nokia’s example. |  |
| China Telecom | 1) Agree 2) See comments |  | We also submitted a todc R3-231515 to discuss the validity time and prediction accuracy related issues. For P1, we think the accuracy/confidence of the AI model can be calculated on the testing date set but cannot reflect the effectiveness of the AI model in actual deployed scenario, because the actual data is not the same as the data set. If the network has deployed an AI model, then it can be considered that the accuracy of the AI model is sufficient. For P2, we think the requested time should be included in the AI/ML INFORMATION REQUEST (FFS on the name) both for one-time and periodic reporting, and the validity time of the prediction information can be deduced from the requested time information provided in the request message, there is no need to explicitly indicate the validity time towards the requesting node. |  |
| Samsung | 1) No 2) Yes, or it can be discussed case by case |  | Accuracy can give the reference for receivers about how to use the prediction information. For example, the receiver can use the high-accuracy information to make the decision directly, while for low-accuracy information, the receiver can just take it as the reference. The accuracy can be obtained in testing stage. And it can be regarded as the feature of the this model. Due to the model may change, e.g. fine-tuning to adjust model, the accuracy is not a static value, so the current accuracy of the model can be sent to the requesting node along with the prediction information. For the validity time, it is useful for the prediction information. The node needs to know when the prediction information is valid, such as 3min later. It can not be derived from the reporting periodicity. The reporting periodicity is x seconds. So the prediction information is sent at time 0, 0+x, 0+2x, 0+3x,… And the validity time is y seconds to z seconds after sending. Thus, the validity time of prediction info sending at time 0 is y to z; validity time of prediction info sending at time 0+x is x+y to x+z. There is no direct relation of reporting periodicity and validity time. Hence, the node needs to send the prediction information with validity time in both periodical and one-time reporting. Or it can be discussed case by case. |  |
| Huawei |  | Yes, for both the proposals |  | We – as proponents of the proposals under discussion under Q3 – believe that it is not useful to provide the prediction’s accuracy/confidence (along with the prediction) since accuracy is not a deterministic information by default but a prediction itself. Moreover, it is not clear to us which could be the behavior of the receiving NG-RAN node when it is provided with such kind of information, will it trust the prediction or not? Eventually, the prediction accuracy can only be calculated by comparing the prediction with the ground truth in the prediction time. However, the prediction information needs to be transferred over Xn before the requested node obtains the ground truth. But there could also be other means to evaluate an AI/ML prediction’s accuracy, e.g. via the UE performance feedback or via other use case specific means. For P2, because the requested node reports only once, the requesting node needs to explicitly indicate the requested time of the valid prediction information, that is, it may be considered that the requested time has the same meaning with the validity time. We should also discuss the case of periodic reporting: how to indicate the requested time and how to how to solve the problem that the reporting periodicity is different (i.e. shorter or longer) from the validity time of the prediction information. |
| Qualcomm |  | 1) No 2) Yes |  | We think it is very hard for the predicting node to achieve high accuracy due to feedback data unavailability, or model change or input parameter change etc. If the predicting node knows that accuracy of the predicted data is not upto the mark, then predicting node shall decide not to send the predicted data and send the predicted data only when the prediction accuracy is high. On P2, the validity time can be different for periodic reporting or one shot reporting. In case of periodic reporting the validity of the predicted data can be applicable until the next update or upto a certain validity time as per Nokia’s comments. In case of one shot reporting, the prediction validity is a must. Irrespective of whether we introduce Ies to provide validity time or preconfigure the validity time, can we at least have an agreement in this meeting that validity time is needed for both periodic and one shot reporting? We can discuss the stage 3 aspects once we have this agreement. |
| Deutsche Telekom |  | 1) Depends 2) Yes |  | On P1: Having accuracy/confidence information about prediction data would definitely help the receiving RAN node in its decision process, but as stated by some companies, it is hard to get actual accuracy/confidence values if input data sets for predicting node are different to training data. But do we assume homogeneous model deployment resulting in same accuracy/confidence levels across neighbouring nodes? If not, we should think about how such info can be provided: via Xn or e.g. via OAM taking care of training results? Please consider that models may be exchanged and/or updated during online training. On P2: We think to have a common approach for one shot and periodic reporting both time info should be included. |
| ZTE |  | Agree 1) Agree 2) but … |  | For P1, accuracy is an important piece of information that is highly relevant to AI/ML models. However, different models may have different evaluation methods to calculate accuracy, and it is not always represented as a value from 0 to 100. Additionally, the predicted information is generated by the NG-RAN node that is being requested, and it is up to that node to decide whether the predicted information is good enough to be sent back to the requesting NG-RAN node. Given these factors, we don't believe it is necessary to transfer accuracy information along with the predicted information. Moving on to P2, we believe that the requested time and validity time can be configured for predicted information as well as other measurements. For predicted information, the requesting NG-RAN node can inform the requested NG-RAN node of the required validity time for the prediction information. For other types of measurements, such as feedback information, the requesting NG-RAN node also needs to know the time window for collecting the measurement. Furthermore, we think that timing information can be used not only for one-time reporting but also for periodic reporting. In the case of periodic reporting, the timing information should be configured to include the periodicity of the report. |
| InterDigital |  | Agree 1, See comments for 2 |  | For 2 we agree with Ericsson |
| CMCC |  | 1) Depends 2) Yes |  | For P1, we see some benefits to provide the accuracy information to the requesting node, but as some companies commented, it is hard to get actual accuracy/confidence values if input data sets for predicting node are different to training data. So, it depends on whether practical to define a unified evaluation method to calculate accuracy. For P2, the requested time and validity time could be configured for both one-time reporting and periodical reporting. |
| LGE |  | 1) Yes |  | For P1, we have a similar view as Nokia. |
| Orange |  | For P1: No See comments for P2 |  | for P1, we believe that accuracy/confidence information about prediction data would be beneficial for the receiving RAN node to take a decision Proposal 2 should be modified to allow the requested time and validity time to be configured in the AI/ML INFORMATION REQUEST (FFS on the name) for both periodic reporting and in the case of one-time reporting. |

Summary of Q3:

For P1, company views are split half by half.

P2 is supported by majority companies, where some companies think the requested time is clarified as requested prediction time. Furthermore, some companies deem that the requested time and validity time could be configured for both one-time reporting and periodical reporting.

Proposal 2: The requested prediction time and validity time are configured in the AI/ML INFORMATION REQUEST (FFS on the name), for one-time reporting.

For periodical reporting, it needs to be further discussed if prediction time and validity time are explicitly or implicitly signalled (e.g. by means of the reporting period).

Requested prediction time: time duration of the requesting prediction information in the AI/ML INFORMATION REQUEST (FFS on the name), e.g. start time plus end time.

Validity time: time period within which the requested prediction information in the AI/ML INFORMATION UPDATE (FFS on the name) is considered valid, hence the AI/ML prediction can be used by the requesting NG-RAN node.

FFS whether accuracy/confidence of an AI/ML prediction is sent over Xn along with the prediction.

3.3 LS to SA5

In [4], it is observed that for the scenario where AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB, performance feedback info should be provided to OAM, so that OAM could take further actions accordingly, thus it is proposed that an LS should be sent to SA5 from RAN3 to make them aware of the RAN3 agreements.

Proposal: Agree to send an LS to SA5 to make them aware of the RAN3 agreements on performance feedback info.

Q4: Companies are invited to provide views on whether to send an LS to SA5 to make them aware of the RAN3 agreements on performance feedback info?

| Company | Yes/No | Reasons/Comments/Suggestions |
| --- | --- | --- |
| Ericsson | No | The work on AI/ML is still ongoing and RAN3 has not converged on all the information involved to support the use cases. We believe that an LS may be needed when the work is near completion. |
| Lenovo | Later | Agree with Ericsson. |
| Intel | No | We could send LS to SA5 when all the related input/output/performance feedback stage-3 becomes clearer. |
| Nokia | No | We think that we need to make some further progress in RAN3 before we send an LS to SA5. |
| CATT | No |  |
| China Telecom | No | No need to send LS to SA5 at this stage. |
| Samsung | Not now | The LS can be sent later when enough progress is achieved. |
| Huawei | Yes | We think it is beneficial for make SA5 aware of our agreements so far, so that we would be able to have some kind of guidance or suggestions for those training-related aspects that may impact OAM (there is a draft in the Annex-1 of our paper in [5]) |
| Qualcomm | No | RAN3 can wait for more progress in the WI. |
| Deutsche Telekom | Perhaps | In principle, we share Huawei’s view, but there is a need for further progress in RAN3. A LS to SA5 should be sent at least at the next meeting. |
| ZTE | Not now | Current progress in RAN3 is premature. It is necessary to send LS to SA5 when we are finishing the WI. |
| InterDigital | Not now |  |
| CMCC | Maybe later |  |
| LGE | No |  |
| Orange | Not yet | Further progress is needed in RAN3 before asking precise guidance from SA5 |

Summary of Q4:

Majority companies deem that it is a bit early to send LS to SA5. The LS can be sent later when enough progress is achieved in RAN3.

Proposal 3: Do not send LS to SA5 at this stage.

4	Conclusion, Recommendations

To be edited, if needed.

5	Reference

- R3-231615 (TP for AI/ML BLCR to TS38.300) Characteristics of the procedures for exchanging AI/ML-related information (Ericsson, InterDigital)

- R3-231655 (TP for TS 38.300) AI/ML General Aspects (Nokia, Nokia Shanghai Bell, Orange)

- R3-231797 (TP for AIML BLCR to TS38.300) Stage 2 updates on the new procedures of AIML for RAN (CMCC, CATT)

- R3-231822 (TP for AI&ML BLCR for TS 38.300) Further discussions on common issues and Stage 2 updates on the introduction of RAN AI/ML (Huawei)

- R3-231823 Further discussions on remaining common open isses for the introduction of RAN AIML (Huawei)

- R3-231841 (TP to BLCR for 38.300) Further discussion on AIML RAN function (ZTE)