3GPP TSG-RAN WG3 #119bis-e	R3-231883

17th – 26th April 2023

Item:	14.2

Source:	CATT (Moderator)

Title:	CB: # MobilityEnh1_RAN1 LS: SoD Reply LS to RAN1 LS in R3-231107

Document for:	Discussion

Introduction

This paper captures the following CB discussion:

CB: # MobilityEnh1_RAN1LS - Check the feasibility and potential impact on specs of RAN 3 of two options, i.e. with RAR and without RAR, - Working on drafting LS to RAN1, if needed. (moderator - CATT) Summary of offline disc R3-231883

Actions requested by RAN1 in LS R3-231107:

ACTION: RAN 1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN 3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in the agreement described in section B. Also, RAN1 respectfully asks RAN2 and RAN3 to take the RAN1 agreements into consideration for their work.

In first round of this CB, we will discuss the feasibility and the potential impact on specs of RAN3, including the a) RACH resource for TA acquisition alignment between source DU and candidate DU and b) impacts on” with RAR” and “without RAR” solutions.

And in second round, we will check the draft reply LS based on the agreements.

Deadline of the CB wendnesday, April 19, 08:00 UTC.

The deadline of the second round discussion is  12pm UTC Monday, April 24th

For the Chairman’s Notes

Agree the LS R3-232089

Second round discussion

Feasibility of all the options

We suggest to put this sentence in the reply LS and list the relative part as below, please provide your comments on the wording or other issues.

RAN3 thanks RAN1 for their LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM. RAN3 would like to provide the following feedback for RAN1’s question: RAN 1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN 3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in the agreement described in section B. - From RAN3 perspective, all options are feasible.

Potential impact of the options

“with RAR” and the RAR is received from serving cell

For the RACH solution, “with RAR” and the RAR is received from serving cell case, i.e., case (a), RAN3 needs to specify how the RAR is transmitted from the candidate DU to the source DU through the CU.

We suggest to put this sentence in the reply LS and list the relative part as below, please provide your comments on the wording or other issues.

- The following aspects will be potential RAN3 specs impact. <For with RAR and the RAR is received from the serving cell> - For the RACH solution,RAN3 needs to specify how the RAR is transmitted from the candidate DU to the source DU through the CU

Any other comments for this case?

| Company | Comment |
| --- | --- |
| Samsung | Agree. The content of RAR depends on RAN2 discussion. |
| E/// | Yes. Some rewording is provided below. The potential impacts of the following options on specifications are being taken into consideration by RAN3. - with RAR and the RAR is received from the serving cell Consider specifying how the RAR is transmitted from the candidate DU to the source DU through the CU |
| Lenovo | Yes. Also agree with the comments from Samsung and E///. |
| Huawei | Agree with above comments. |
| Nokia | Agree with rewording by Ericsson. |
| Qualcomm | Same view as Samsung. |
| CATT | Agree with all the above wording updates |
| LGE | Agree with the above comments. |
| NTT DOCOMO | Agree with rewording by E///. |
| ZTE | Agree with the comments from E///. |

Moderator’s summary:

The potential impacts of the following options on specifications are being taken into consideration by RAN3.

- with RAR and the RAR is received from the serving cell

RAN3 considers specifying how the RAR is transmitted from the candidate DU to the source DU through the CU

RAR” and the RAR is received from candidate cell

2. RAN3 need to consider potential RAN3 spec impact for the following two cases

(c) with RAR and the RAR is received from candidate cell

(a) with RAR and the RAR is received from serving cell, and

(b) without RAR

Follow the online discussion, moderate think case (c) is also has RAN3 impact, please provide your comments about whether include case (c)

| Company | Yes/No | Comment |
| --- | --- | --- |
| Samsung | No for now | RAN2 agreed to deprioritize this case in Rel-18. Thus, RAN3 needs wait for progress from RAN1/2 first. |
| E/// | No | The status is updated based on the progress made in both RAN1 and RAN2. RAN2 made the following assumption. R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. And RAN1 agreed to support RAR from serving cell only. |
| Lenovo | No for now | After checking the RAN1/RAN2 progress, we agree to deprioritize the case. RAN2 made the assumption as E/// pointed out. RAN1 made the following agreements: Agreement When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case. Agreement When reception of RAR is configured, support RAR is received from serving cell in inter-DU case. - FFS: RA response window related issues |
| Huaei | No |  |
| Nokia | No |  |
| Qualcomm | No | Agree with Ericsson. |
| CATT | No |  |
| LGE | No |  |
| NTT DOCOMO | No |  |
| ZTE | No |  |

If the above answer is yes, moderator suggests to put this sentence in the reply LS and list the relative part as below, please provide your comments on the wording.

- The following aspects will be potential RAN3 specs impact. <For with RAR and the RAR is received from the candidate cell> - For the RACH solution, RAN3 needs to specify how the source DU knows when the RACH procedure is successfully completed or failed between the UE and the candidate DU.

Any other comments for this case?

| Company | Comment |
| --- | --- |
| Samsung | Wait for RAN1/RAN2 progress first |
| E/// | No need any more. |
| Lenovo | No need. |
| Huawei | Not needed. |
| Nokia | Not needed |
| Qualcomm | No need to consider this for now. |
| CATT | No |
| LGE | No need. |
| NTT DOCOMO | Not needed. |
| ZTE | Not needed |

- Q1: Discuss the impact about case(c) with RAR and the RAR is received from candidate DU in the reply LS

Already included in the second question.

Moderator’s summary:

After companies checked RAN1 latest agreement, there is no need to capture case (c) in the LS.

“without RAR”

Q2: For case (b), discuss whether include the option 2 in the reply LS.

-Option 2: After LTM is triggered, the serving DU requests the target DU to transmit the latest TA value which is maintained by itself before sending LTM command, e.g., the target DU shall transmit the requested TA value to the serving DU by F1 interface through the CU.

For the option 2, moderator wants to give a further clarification.

Option 2 means source DU request TA from the target cell when it needs to execute LTM, and after get the TA, source DU sending the LTM command to the UE include the TA. This is because in lasting meeting, we send an LS to RAN2 ask the feasible of two approaches as below In approach 1 – LTM triggering without target candidate DU involvement, In approach 2 – LTM triggering with target candidate DU involvement, the serving gNB-DU first requests the candidate/target gNB-DU, via the gNB-CU, that a L1/L2 triggered mobility serving cell change is required, and the candidate/target gNB-DU makes a decision about target cell/beam including the TCI state and/or SSB index. Only thereafter the serving gNB-DU creates and transmits the lower layer signal to the UE. As RAN2 still not reply the LS, the purpose for option 2 is not exclude the approach 2 in our previous discussion

Please provide your comments about whether included option 2 in the chart below.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Samsung | No | Let’s wait for RAN2 reply first. |
| E/// | Yes | It would be premature to exclude any option while waiting for RAN2’s reply on the approaches. |
| Lenovo | No | We should wait for the RAN2 decision. |
| Huawei | No | If as clarified by moderator, what is the difference with approach 1? |
| Nokia | No |  |
| Qualcomm | Yes | Agree with Ericsson. |
| CATT |  | Yes for including the two options, prefer a simply way. |
| LGE | Yes | Agree with Ericsson. |
| NTT DOCOMO | No | We should not exclude any options in RAN3 discussion, however, if we are not asking RAN1 or RAN2 about the feasibility of the options (we have already asked RAN2 about Approach 1/2), the options need not be enumerated in the LS. |
| ZTE | No | No need to go into such details. Maybe we can just say that the source DU get the TA value from the candidate DU since target DU is also one of candidate DU. |

If the above answer is yes, moderator suggests to put below sentence in the reply LS, please provide your comments on the wording.

<For Without RAR> - For the RACH solution,RAN3 needs to discuss and specify how and when the TA value(s) is(are) transmitted from the candidate DU to the source DU and how the TA value is maintained: - Option 1: The candidate DUs transmit the TA values of candidate cells to source DU before LTM triggering and the source DU maintains the TA values, e.g., the candidate DUs shall transmit the TA values to the source DU by F1 interface through the CU. - Option 2: Option 2: After LTM is triggered, the source DU requests the target DU to transmit the latest TA value which is maintained by itself before sending LTM command, e.g., the target DU shall transmit the requested TA value to the source DU by F1 interface through the CU.

If the above answer is no, moderator suggests to put it simply in the reply LS and list the possible description as below, please provide your comments on the wording.

<For Without RAR> - For the RACH solution,RAN3 needs to discuss and specify how and when the TA value(s) is(are) transmitted from the candidate DU to the source DU and how the TA value is maintained:

Any other comments for this case?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Samsung | Comments | Some rewording towards the reply based on the answer “no”. <For Without RAR> For the RACH solution,RAN3 needs to discuss and specify how and when the TA value(s) is(are) transmitted from the candidate DU to the source DU and how the TA value is maintained The reason to delete “when” is that the reply from RAN2 on approach 1 and 2 is not received yet. |
| E/// | Yes | First the answer is to include both options. The outcome is contingent upon the response from RAN2 regarding the approaches. One possible way forward is to include a description indicating that the options will be revisited based on the response from RAN2. |
| Lenovo | See comments | We think it is for the RACH-less solution, but not for the RACH solution. Then we can reword it as: - Without RAR Consider specifying how the TA value(s) is(are) transmitted from the candidate DU to the source DU and how the TA value is maintained. |
| Huawei |  | Lenovo and Samsung’s comments seems reasonable. |
| Nokia | Yes | Agree with Lenovo’s comment. However, wording could be aligned with the “style” from the other option as follows. - Without RAR Consider specifying how the TA value(s) is(are) transmitted from the candidate DU to the source DU through the CU, and how the TA value is maintained. |
| Qualcomm | Yes |  |
| CATT | Prefer NO and use the second one | Second one not exclude any options, it is a high level description included the common part of two options. We are agree with all the above wording updates. |
| LGE | Yes |  |
| NTT DOCOMO | Yes | Agree with the reworded version from Nokia. |
| ZTE |  | Agree with the rewording from Nokia |

Moderator’s summary:

Without RAR

RAN3 needs to discuss and specify how the TA value(s) is(are) transmitted from the candidate DU to the source DU through the CU and how the TA value is maintained:

First round discussion

- For the Chairman’s Notes

First round:

For agreement:

Proposal 1: The CU requests the candidate DU to provide RACH resource for TA acquisition in inter-DU case,

Proposal 2: For “with RAR” and the RAR is received from serving cell case, i.e., case (a), RAN3 needs to specify how the RAR is transmitted from the candidate DU to the serving DU, e.g., the candidate DUs shall transmit the RARs to the serving DU by F1 interface through the CU [9][10], candidate DU transmits RAR to CU, and then CU transmits it to serving DU.

Proposal 3: RAN3 need to consider potential RAN3 spec impact for the following two cases

(a) with RAR and the RAR is received from serving cell, and

(b) without RAR

Proposal 4: No need to include the RACH resource for TA acquisition alignment in the reply LS

For online discussion

Q1: Discuss whether need include the impact about case(c) with RAR and the RAR is received from candidate DU in the reply LS

Q2: For case (b), discuss whether include the option 2 in the reply LS.

-	Option 2: After LTM is triggered, the serving DU requests the target DU to transmit the latest TA value which is maintained by itself before sending LTM command, e.g., the target DU shall transmit the requested TA value to the serving DU by F1 interface through the CU.

Plan to next round:

Check wording for the draft LS.

For further study:

FFS on the content of RACH resource.

FFS on the message for CU requesting “RACH resource”(UE Context Setup procedure or UE Context Modification procedure or both)

For intra-DU LTM, as all companies clarified in their papers, no RAN3 impact is foreseen for either RA “with RAR” or “without RAR” and everything can be done inside the gNB-DU by gNB-DU implementation. So we only need pay attention to inter-DU case in below discussion.

RACH resource for TA acquisition alignment

From the papers [1-7], they all mentioned RACH resource for TA acquisition alignment between source DU and candidate DU is needed for both “withRAR” and “withoutRAR” solutions. In the [1][3][4][5][6], they also specify the node decides to trigger TA acquisition is CU. Moderator thinks it is easy to get the agreement in RAN3 and want to check companies’ views.

Proposal: The CU requests the candidate DU to provide RACH resource for TA acquisition in inter-DU case.

Q1: Companies are invited to express their view on the above proposal?

| Company | Yes/No | Comment |
| --- | --- | --- |
| E/// | Yes with comments | CU is the one who requests the candidate DU for TA acquisition. The “RACH resource” could include the related configurations, e.g., Random Access Preamble indices and RACH occasions with the associated SSB indices for each LTM candidate cell configured for TA establishment. Though the contents are not defined by RAN3. |
| Huawei | Yes |  |
| Nokia | Yes with comments | Agree that gNB-CU will be the node retrieving the TA from the target gNB-DU. As to what exact contents are included in the response from the gNB-DU, that is FFS. |
| Lenovo | Yes |  |
| ZTE | Yes |  |
| Fujitsu | Yes | In order to reuse existing F1AP, CU should request this to the candidate DU. |
| CMCC | Yes |  |
| China Telecom | Yes |  |
| NTT DOCOMO | Yes |  |
| LGE | Yes |  |
| CATT | yes | We agree to set the content as FFS and waiting for more information |
| Qualcomm | Yes | RAN3 needs to wait for RAN2 progress on the basic questions such as whether RACH-less LTM cell switch is feasible and desirable for inter-DU LTM, and whether CU or source DU should initiate the early TA acquisition procedure. |

Moderator’s summary:

All the companies say yes, and 1 company gives suggestion about the content of “RACH resource”; 2 companies agree to set the content as FFS for further study.

As this is the first time to start this discussion and this part seems no impact on the reply the LS, the moderator suggest to set the discussion about content of “RACH resource” as FFS and discuss it in next meeting.

For chairman notes:

The CU requests the candidate DU to provide RACH resource for TA acquisition in inter-DU case, the content of RACH resource is FFS.

If the answer of Q1 is yes, the next issues is when will the CU request RACH resource for TA acquisition and which procedure is used for it. There are mainly two options according to [1][3][5]:

- Option 1: when an LTM candidate is being configured, and use the UE Context Setup procedure

- Option 2: at a later moment, after an LTM candidate cell for which a TA value is unknown, and UE Context Modification procedure.

Q2: Companies are invited to express their view on the above two options?

| Company | Option 1/ option 2 | Comment |
| --- | --- | --- |
| E/// | Both | As described in our paper, there are two cases related to TA acquisition, which are i) when an LTM candidate is being configured or ii) at a later moment, after an LTM candidate cell for which a TA value is unknown. Thus both UE Context Setup and UE Context Modification procedures should be taken into account. |
| Huawei | Either way is feasible | As in the LS , RAN1 just checks the feasibility of the TA acquisition. It seems too early to make a downselection on the two options. We think that both option is feasible and possible. We may decide whether RAN3 needs to support both or one of them later on with more RAN1/RAN2 progress. |
| Nokia | Option 2 | In our view there are two aspects. Firstly, configuration of the TA provisioning method, as this needs to be known and configured at both Source and Target DU. With that into consideration, the UE Context Setup/Modification procedures could be used to “configure” the TA acquisition method. Then as a second step, e the UE Context Modification procedure would be the one used for retrieving the TA from the Target DU. However, this depends on whether there are multiple TA provisioning mechanisms supported or not, which is up to RAN2. Hence, at this point is fair to take and consider both procedures (Setup and Modification) into account. |
| Lenovo | Both are feasible | We only need to answer the feasibility. The detailed solution should be decided with more RAN1/RAN2 inputs. |
| ZTE | Both are feasible | Too early to make the solution selection. At this stage, we only need to respond with the feasibility. |
| Fujitsu | No preference | Both are feasible. |
| CMCC | Both are feasible | Share the same view with Lenovo and ZTE. |
| China Telecom | Both | No preference. |
| NTT DOCOMO | Both | From RAN3 perspective, both options are feasible, and as E/// mentioned, UE Context Setup/Modification procedure can be used. |
| LGE | Both |  |
| CATT | yes | We also agree this part is not need to reflect in the reply LS, and set the two messages as FFS. |
| Qualcomm | Option 2 | An issue with Option 1 could be that the TA value is not valid when LTM execution is triggered since there could be significant gap in time between LTM configuration and LTM execution. |

Moderator’s summary:

10 companies think both options are feasible, 2 companies support option 2, as it is still early to down selection,  the moderator suggest we just put the two options in FFS and discuss it after RAN1/ran2 reach some relative conclusion.

For chairman notes:

FFS on the message for CU requesting “RACH resource”(UE Context Setup procedure or UE Context Modification procedure or both)

“With RAR” and “without RAR” solutions

After configured the RACH resource for TA acquisition, the source DU can decide to trigger early TA acquisition by PDCCH-ordered RACH as per RAN1 agreement[1], and then candidate DU transmit the RAR/TA back to serving DU follow the “with RAR” or “without RAR” solutions.

In case of “with RAR and the RAR is received from candidate cell”, the candidate DUs does not need to transmit RAR to the serving DU. Therefore, RAN3 only needs to consider potential RAN3 spec impact for the two cases, (a) with RAR and the RAR is received from serving cell, and (b) without RAR.

Observation: RAN3 only need to consider potential RAN3 spec impact for the following two cases

(a) with RAR and the RAR is received from serving cell, and

(b) without RAR

Q3: Companies are invited to express their view on whether agree with observation as above?

| Company | Yes/No | Comment |
| --- | --- | --- |
| E/// | Yes and No | In case b, without RAR, candidate DU generates TA value for the UE, and then sends to the serving DU. So the serving DU has it ready upon the cell switch assuming approach 1 is adopted for the execution. Case a is actually very similar to be, for the case “RAR” or RAR-like message is from serving cell. The reason is that the UE needs to know as soon as possible that the preamble was successfully transmitted, i.e., the C-DU needs to indicate the TA value as soon as it calculates to the CU/S-DU. RAN3 considers seeking for the same procedure to cover both cases. One additional case with potential RAN3 impacts would be in which RAR is from candidate cell, the S-DU may need to know when the procedure is successfully completed or failed between the UE and the C-DU, so the S-DU may stop/start scheduling restrictions while the UE is transmitting to the C-DU. Thus, RAN3 impacts should not be limited to case a and b only. |
| Huawei | Yes | In general, the use cases of a) and b) are fine. The failure case mentioned by Ericsson may need to consider as well, but prefer RAN2 to discuss it firstly. |
| Nokia | Yes | In case of provision of TA via RAR from the cell at the Target gNB-DU, in principle TA does not need to be informed to the Source gNB-DU and there is no need for the Source and the Target gNB-DUs to interact. However, knowledge of whether PRACH preamble transmission was succeeded or failed can be useful for the Source-gNB-DU. |
| Lenovo | Yes | Case (a) and (b) are fine. |
| ZTE | Yes | Case (a) and (b) are fine. |
| Fujitsu | Yes | This is our observation in our contribution in [9] |
| CMCC | Yes | Case (a) and (b) are fine. |
| China Telecom | Yes | Case (a) and (b) are fine. |
| NTT DOCOMO | Yes | TA acquisition related signaling is needed only for (a) and (b), and in case of RAR received in candidate cell(s), TA value is not needed to be exchanged between DUs. However, as E/// mentioned, other RAN3 impact than exchanging TA value also would be expected. |
| LGE | Yes | For the failure case, as mentioned by Ericsson, we have a similar view to Huawei. |
| CATT | yes | We notice in RAN2 yesterday meeting, they didn’t want to support with RAR from candidate cell, whether we should still include this in RAN3? We are also fine to mention the with RAR from candidate cell in reply LS. R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. |
| Qualcomm | Yes | In case of (b), the candidate DU provides the TA value via the CU to the source DU which then sends it in the LTM command to the UE. In case of (a), the candidate DU provides the TA value via the CU to the source DU which can then send it in a MAC CE to the UE. Though the RAN3 impacts can be listed, some evaluation is needed, probably up to RAN1/RAN2, as to which option is adopted. |

Moderator’s summary:

12 companies agree included case (a) and (b); 5 company suggest to also include case(c) with RAR and the RAR is received from candidate DU as it will also have interface impact, e.g. the S-DU may need to know when the procedure is successfully completed or failed between the UE and the C-DU. What’s more, 2 companies support the case(c) but prefer RAN2 to discuss it firstly.

The moderator noticed in yesterday RAN2 discussion, they assumes case(c) is not needed, we can discuss this issues online.

For chairman notes:

RAN3 need to consider potential RAN3 spec impact for the following two cases

(a) with RAR and the RAR is received from serving cell, and

(b) without RAR

Discuss whether need include the impact about case(c) with RAR and the RAR is received from candidate DU in the reply LS.

For “with RAR” and the RAR is received from serving cell case, i.e., case (a), RAN3 needs to specify how the RAR is transmitted from the candidate DU to the serving DU, e.g., the candidate DUs shall transmit the RARs to the serving DU by F1 interface through the CU [9][10], candidate DU transmits RAR to CU, and then CU transmits it to serving DU.

Q4: Companies are invited to express their view on whether agree the above impact on case (a), and whether there exists any other impacts?

| Company | Yes/No | Comment |
| --- | --- | --- |
| E/// | Yes |  |
| Huawei | yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| ZTE | Yes |  |
| Fujitsu | Yes | This is our observation in our contribution in [9] |
| CMCC | Yes |  |
| China Telecom | Yes |  |
| NTT DOCOMO | Yes |  |
| LGE | Yes |  |
| CATT | Yes |  |
| Qualcomm | Yes | We agree with the above-mentioned impact. |

Moderator’s summary:

All the companies say yes, we can put this wording as baseline in the second round discussion about draft reply LS.

For chairman notes:

For “with RAR” and the RAR is received from serving cell case, i.e., case (a), RAN3 needs to specify how the RAR is transmitted from the candidate DU to the serving DU, e.g., the candidate DUs shall transmit the RARs to the serving DU by F1 interface through the CU [9][10], candidate DU transmits RAR to CU, and then CU transmits it to serving DU.

For “without RAR” case, i.e., case (b), two options are foreseen on how and when the TA value(s) is(are) transmitted from the candidate/target DU to the serving DU and how the TA value is maintained[9][10]:

-	Option 1: The candidate DUs transmit the TA values of candidate cells to serving DU before LTM triggering and the serving DU maintains the TA values, e.g., the candidate DUs shall transmit the TA values to the serving DU by F1 interface through the CU.

-	Option 2: After LTM is triggered, the serving DU requests the target DU to transmit the latest TA value which is maintained by itself before sending LTM command, e.g., the target DU shall transmit the requested TA value to the serving DU by F1 interface through the CU.

From the perspective of RAN3, the Option 1 and option 2 are both feasible in “ without RAR”case, a relative LS about feasibility checking for two options has already sent to RAN2, as reply LS is not received, so we can just put the two options in the reply LS.

Q5: Companies are invited to express their view on whether agree the above impact on case (b), and whether there exists any other impacts?

| Company | Yes/No | Comment |
| --- | --- | --- |
| E/// | Yes with comments | Both options are feasible, and the selection would be depending on the approaches for execution. Furthermore, Approach 2 indeed opens up for the possibility of Option 2, but it is also possible to assume option 1 for approach 2. |
| Huawei | Need clarification | For option 2, what is the purpose to obtain the target cell’s TA via network after LTM is triggered? If my understanding is correct, the TA should be sent to the UE inside the LTM command. |
| Nokia | Yes | Both options are feasible from RAN3 perspective. |
| Lenovo |  | Same view with Huawei. |
| ZTE |  | Same view with Huawei. |
| Fujitsu | Yes | This is our observation in our contribution in [9] |
| CMCC |  | Same view with Huawei. |
| China Telecom |  | Same view with Huawei. |
| NTT DOCOMO | Yes | Agree with E///. |
| LGE | Yes | Both options are feasible. |
| CATT |  | For the option 2, we want to clarify it means serving DU request TA from the target cell, and after get the TA, serving DU sending the LTM command to the UE include the TA. This is because in lasting meeting, we send an LS to RAN2 ask the feasible of two approaches as below In approach 1 – LTM triggering without target candidate DU involvement, In approach 2 – LTM triggering with target candidate DU involvement, the serving gNB-DU first requests the candidate/target gNB-DU, via the gNB-CU, that a L1/L2 triggered mobility serving cell change is required, and the candidate/target gNB-DU makes a decision about target cell/beam including the TCI state and/or SSB index. Only thereafter the serving gNB-DU creates and transmits the lower layer signal to the UE. As RAN2 still not reply the LS, the purpose for option 2 is not Exclude the approach 2 in our previous discussion. |
| Qualcomm | Yes, the above are the suitable options | Option 2 seems to introduce additional delay during LTM execution, which makes the RACH-less LTM option less attractive. Option 1 is our preference. Regarding which option is more suitable is a question for RAN2 to consider also. |

Moderator’s summary:

7 companies think both option are feasible from RAN3 perspective, 8 companies need further clarification for option 2.

The moderator wants to clarify the option2 means serving DU request TA from the target cell, and after get the TA, serving DU sending the LTM command to the UE include the TA. This is one possible approaches for execution we have discussed in last meeting and send LS to RAN2. As RAN2 still not reply the LS, the purpose for option 2 is not excluding the approach we agreed in the previous discussion.

we suggest to discuss it online and if we exclude the option 2, the wording for option 1 also need update.

For chairman notes:

For case (b), discuss whether include the option 2 in the reply LS.

-	Option 2: After LTM is triggered, the serving DU requests the target DU to transmit the latest TA value which is maintained by itself before sending LTM command, e.g., the target DU shall transmit the requested TA value to the serving DU by F1 interface through the CU.

According to the analysis from companies, we can see from RAN3 perspective, both options is feasible. What’s more, “with RAR” and “without RAR” solution have the same impact on RACH resource for TA acquisition alignment between source DU and candidate DU. The different impact is on when/how to transmit the TA/RAR from the candidate/target DU to the serving DU., As the first part have a common impact, there seems no need to inform it to other groups. We suggest to just include the different impacts between “with RAR” and “without RAR” solution in the reply LS.

Proposal: No need to include the RACH resource for TA acquisition alignment in the reply LS, only tell other group both options are feasible and give the different potential impacts for “with RAR” and “without RAR” solutions.

Q6: Companies are invited to express their view on the above proposal about above proposal?

| Company | Yes/No | Comment |
| --- | --- | --- |
| E/// | Yes with comments | As indicated in the reply for Q3, other potential RAN3 impacts are foreseen, which should be included in the reply as well. |
| Huawei | Yes | Agree with Ericsson. |
| Nokia | Yes | RAN3 should reply that from RAN3 point of view, all options (i.e., from the Source gNB-DU via MAC CE for cell change, via RAR from the cell at the Source gNB-DU, Via RAR from the cell at the Target gNB-DU) are feasible and incur RAN3 impacts. Likewise, that it is up to RAN1/RAN2 to decide which RAR configuration option(s) is supported. |
| Lenovo | Yes | Agree with the proposal from moderator, we only need to reply the feasibility and potential impacts. |
| ZTE | Yes |  |
| Fujitsu | Yes | Actually, RAN1 needs to know just feasibility of their agreement and both of option 1 and 2 are feasible. |
| CMCC | Yes | Agree with the proposal from the moderator. |
| China Telecom | Yes |  |
| NTT DOCOMO | Yes |  |
| LGE | Yes | Agree with Ericsson. |
| CATT | Yes | Same as Q3 |
| Qualcomm |  | RAN3 could summarize the RAN3 impacts for the with and without RAR options, and if the impacts are similar, could indicate that too in the reply LS for other groups (RAN1/RAN2) to consider. |

Moderator’s summary:

11 companies say yes, 4 companies suggest also include case(c), and discuss whether need include case(c) as mentioned in Q3.

For chairman notes:

No need to include the RACH resource for TA acquisition alignment in the reply LS

Others

Q7: Other issues, if any.

| Company | Comment |
| --- | --- |

Conclusion, Recommendations [if needed]

References

[1] R3-231183, Discussion on TA Acquisition for LTM  (Nokia, Nokia Shanghai Bell)

[2] R3-231315, Signalling Support for LTM (Qualcomm Incorporated)

[3] R3-231447, Discussion on L1L2 based inter-cell mobility (Lenovo)

[4] (TP for LTM BL CR to TS 38.401) Solutions for LTM (Ericsson)

[5] (TP for L1L2Mob BLCR for TS 38.401): L1/L2 Mobility procedure on F1 (Huawei)

[6] (TP to Mob_enh2 BL CR TS38.401) Discussion on L1/L2 based Inter-cell Mobility (Samsung Electronics France SA)

[7] R3-231813, Further discussion on LTM (NTT DOCOMO INC.)

[8] R3-231107 LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM (RAN1, Fujitsu, CATT)

[9] R3-231327 Discussion on replying to the RAN1 LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM (Fujitsu, CATT)

[10] R3-231326 [Draft] Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM (Fujitsu, CATT)