3GPP TSG-RAN WG3 #119bis-e                                                           	R3-231961

Online, 17th – 26th April, 2023

Title: 	(TP for MT-SDT BL CR to TS 38.300) Support of MT-SDT

Source: 	CATT, Ericsson, ZTE, China Mobile, China Telecom, Nokia, Nokia Shanghai Bell, Lenovo

Agenda item:	20.2

Document for:	Discussion and Decision

- Introduction

Following the discussion of MT-SDT in RAN3#119bis-e, this contribution provides a stage 2 TP for MT-SDT BL CR to TS 38.300, to make support of MT-SDT.

2. (TP for MT-SDT BL CR to TS 38.300) Support of MT-SDT

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

18.X	MT-SDT with/without UE context relocation

The overall procedure for MT-SDT procedure with/without UE context relocation is illustrated in the figure 18.X-1.

Figure 18.X-1. MT-SDT with/without UE context relocation

1.  DL user data and/or DL NAS signalling are received at the Last Serving gNB for the UE in RRC_INACTIVE state.

2.  The Last Serving gNB may send MT-SDT information to the neighbour gNBs within the RNA, via XnAP RAN PAGING message.

3.  The gNB that receives MT-SDT information within the RNA, i.e. the Receiving gNB, takes into account the MT-SDT information received in the XnAP RAN PAGING message to decide whether to trigger MT-SDT Paging.

4/5.  The UE may decide to initiate MT-SDT procedure and send an RRCResumeRequest message for MT-SDT to the Receiving gNB.

Editor note: The above step 3/4/5 can be revisited by RAN2 progress.

6. The Receiving gNB identifies the Last Serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure.

7. The following steps are the same as Figure 18.2-1/18.3-1 from step 3, except that the first SDT user data/NAS signalling is DL SDT data and/or DL SDT NAS signalling.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>